| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0587 |
| --- | --- |

| Title: | Non-CE4: Simplification of MVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC, MV scaling is required to derive the spatial MVP candidates to construct the MVP list for inter mode. Since MV scaling process is nontrivial in terms of its complexity, the proposed method aims to further reduce the possibility of spatial MV scaling in deriving MVP. In the proposed scheme, the inclusion of scaled spatial MVP candidates is delayed until the history-based MVP are checked. With the proposed modification, the possibility of incurring spatial MV scaling in deriving MVP could be reduced significantly. Simulation results show that there is -0.02% and -0.06% BD-rates savings for RA and LDB respectively over VTM-5.0. Moreover, removing the scaled spatial MVPs from the MVP list introduces 0.01% and -0.03% BD-rate changes for RA and LDB.

# Introduction

In VVC, the MVP candidate list is constructed by including the following candidates in order:

- Derive up to one non-scaled MVP from left spatial neighbour CUs (A0A1)

- If no non-scaled MVP from left is available, derive up to one scaled MVP from left spatial neighbour CUs(A0A1)

- Derive up to one non-scaled MVP from above spatial neighbour CUs(B0B1B2)

- If both neighbor block A0 and A1 are not available or coded in intra modes, derive up to one scaled MVP from above spatial neighbour CUs(B0B1B2)

- Conditional Pruning

- Derive up to one MVP from temporal collocated CUs (T0T1)

- Derive up to two history-based MVP from an FIFO table

- Derive up to two Zero MVs.

As illustrated above, the MV scaling is introduced in step 2 and 4 to derive scaled MVP for the MVP list. Since MV scaling process is nontrivial in terms of its complexity, the proposed method aims to further reduce the possibility of incurring spatial MV scaling in deriving MVP.

# Proposed method

In this proposal, to reduce the possibility of selecting/deriving the scaled spatial MVPs, it is proposed to put the scaled spatial MVP to later position when constructing the MVP candidate list. The conditions for deriving the scaled MVP from the spatial neighboring blocks are kept the same as that in the current VVC. The MVP candidate list is constructed according to the following order.

- Derive up to one non-scaled MVP from left spatial neighbour blocks (A0A1)

- Derive up to one non-scaled MVP from above spatial neighbour blocks (B0B1B2)

- Conditional Pruning

- Derive up to one MVP from temporal collocated blocks (T0T1)

- Derive up to two history-based MVP from a FIFO table

- If no non-scaled MVP from left neighbor blocks is available, derive up to one scaled MVP from left spatial neighbour blocks (A0A1)

- If none of the neighbor blocks A0 and A1 are available or they are coded in intra modes, derive up to one scaled MVP from above spatial neighbour blocks (B0B1B2)

- Derive up to two Zero MVs.

Moreover, the results of removing the scaled MVP from the spatial blocks (the steps 6 and 7) are also tested in this proposal.

# Results

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementation under CTC [2] against VTM-5.0 are shown below.

Table 1. Performance of moving scaled spatial MVP to later position

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | 0.03% | 100% | 100% |
| Class A2 | -0.03% | -0.02% | -0.01% | 99% | 100% |
| Class B | -0.01% | -0.03% | -0.07% | 100% | 100% |
| Class C | -0.04% | -0.05% | -0.06% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.07% | -0.04% | 100% | 100% |
| Class D | -0.01% | 0.02% | 0.01% | 100% | 100% |
| Class F | 0.02% | 0.06% | 0.00% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.12% | 0.41% | 99% | 99% |
| Class C | -0.03% | -0.03% | 0.06% | 100% | 101% |
| Class E | -0.08% | 0.24% | -0.53% | 101% | 99% |
| Overall | -0.06% | 0.10% | 0.06% | 100% | 100% |
| Class D | -0.03% | -0.54% | 0.19% | 101% | 99% |
| Class F | 0.00% | 0.02% | 0.38% | 100% | 102% |

The statistics data are also collected to check the likelihood of deriving scaled spatial MVP at decoder end, based on AMVP coded blocks in all the bitstreams generated under CTC. As shown in Table 2, the chance of deriving scaled spatial MVP at decoder side is 26% and 43% for VTM-5.0 under RA and LDB while the chance is reduced to 6% and 18% in the proposed method.

Table 2. The percentage that the MVP list contains the scaled spatial MVP

|  | VTM5 | Proposed |
| --- | --- | --- |
| RA | 26% | 6% |
| LDB | 43% | 18% |

The results of removing the scaled spatial MVP are shown in Table 3.

Table 3. Performance of removing scaled spatial MVPs

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.09% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Class B | 0.01% | 0.07% | -0.02% | 101% | 99% |
| Class C | 0.02% | -0.01% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | 0.18% | 0.15% | 100% | 99% |
| Class F | 0.04% | 0.05% | 0.08% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.15% | 0.12% | 100% | 101% |
| Class C | -0.04% | 0.02% | -0.17% | 101% | 100% |
| Class E | -0.03% | -0.08% | -0.40% | 101% | 100% |
| Overall | -0.03% | 0.05% | -0.11% | 101% | 100% |
| Class D | 0.02% | 0.65% | 0.77% | 100% | 101% |
| Class F | 0.09% | 0.35% | -0.52% | 100% | 100% |

# Conclusion

In this proposal, methods are proposed to reduce the possibility of incurring spatial MV scaling in deriving MVP under AMVP mode. The proposed modification which moves the scaled spatial MVPs to the later position not only reduces the possibility of spatial MVP scaling, but also has no undesirable coding efficiency impact, with BD-rate changes of {-0.02%, -0.07%, -0.04} and {-0.06%, 0.10%, 0.06%} for Y, Cb and Cr components under RA and LD, respectively. It is recommended to consider adoption of the proposed simplification into VVC for AMVP mode MVP list construction.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).