| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0588 |
| --- | --- |

| Title: | CE4-related: On SbTMVP motion shift derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Tel: Email: | +1-858-705-2600 yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, the motion shift for SbTMVP is derived by checking the list0 and list1 MVs of the left neighboring block. The checking order is adaptive (i.e. either list0-first or list1-first) depending on whether current picture has reference picture with POC larger than the current one and the value of collocated_from_l0_flag. However, simulation results show that such an adaptive checking order offers no coding efficiency benefit. In this proposal, it is proposed to always use a fixed order (either list0-first or list1-first) to derive the motion shift from the neighboring block. Simulation results show that the fixed list0-first checking order provides 0.00% and 0.01 BD-rate change under RA and LDB, while the fixed list1-first checking order provides 0.00% and 0.00% BD-rate change under RA and LDB.

# Introduction

In current VVC, motion shift is applied to each SbTMVP to identify the corresponding blocks in the collocated picture more accurately. The left spatial neighbor block is examined to derive the motion shift. If the left spatial neighbor block has a motion vector that uses the collocated picture as its reference picture, this motion vector is used to derive the motion shift to be applied. If no such motion is identified, then the motion shift is set to (0, 0). The process of deriving the motion shift from the neighboring block is illustrated in Table 1. As illustrated in Table 1, the motion shift for SbTMVP is derived by checking the list0 and list1 MVs of the left neighboring block. The checking order is either list0-first or list1-first depending on whether current picture has reference picture with POC larger than the current one and the collocated_from_l0_flag.

Table1 WD for determining the motion shift for the SbTMVP in VVC

- If all of the following conditions are true, tempMv is set equal to mvL1A1:

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0,

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- Otherwise if all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0.

# Proposed method

To remove the adaptive checking order depending on whether current picture has reference picture with POC larger than the current one and the collocated_from_l0_flag. In this proposal, the derivation of motion shift is modified to check the list0 and list1 MVs from the neighboring block using the fixed order: either list0list1 or list1list0.

In the first method, the motion shift is first derived from the list 0 mv of the neighboring block; if the list 0 mv is not available, the list 1 mv of the neighboring block is then used to derive the motion shift for SbTMVP. With the modification, the checking of POC of the reference pictures and the collocated_from_l0_flag can be removed. The modified algorithms are described as shown in Table 2.

Table2 Modified WD for determining the motion shift using the order list0list1

- If all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0.

- Otherwise if all of the following conditions are true, tempMv is set equal to mvL1A1:

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0,

  - slice_type is equal to B.

In the second method, the list1-first checking order is used and the modified algorithm is shown in Table 3.

Table2 Modified WD for determining the motion shift using the order list1list0

- If all of the following conditions are true, tempMv is set equal to mvL1A1:

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0,

  - slice_type is equal to B.

- Otherwise if all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0.

# Simulation results

The tables below show the BD-rate performance and processing time of the method proposed. The simulations are conducted following the common test conditions [1] on top of reference software VTM-5.0 [2].

Table 4. Performance of the first method (list0list1)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.01% | 101% | 100% |
| Class A2 | 0.00% | -0.04% | 0.01% | 100% | 101% |
| Class B | 0.02% | 0.07% | 0.01% | 101% | 99% |
| Class C | -0.01% | 0.01% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.01% | 0.08% | 101% | 100% |
| Class F | 0.00% | 0.01% | -0.01% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.05% | 99% | 100% |
| Class C | 0.05% | -0.28% | 0.04% | 100% | 101% |
| Class E | -0.01% | 0.06% | -0.81% | 99% | 100% |
| Overall | 0.01% | -0.05% | -0.17% | 99% | 100% |
| Class D | -0.04% | 0.11% | 0.62% | 101% | 101% |
| Class F | -0.09% | -0.23% | 0.00% | 101% | 101% |

Table 5. Performance of the second method (list1list0)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | -0.03% | -0.02% | 100% | 101% |
| Class B | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.04% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.02% | 100% | 100% |
| Class D | 0.01% | 0.02% | -0.02% | 100% | 100% |
| Class F | 0.00% | -0.01% | 0.01% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

# Conclusions

In this proposal, it is proposed to always derive the motion shift from the neighboring block either by the list0-first or list1-first. Both methods introduce negligible coding performance changes. With the modification, the checking of POC of the reference pictures and the collocated_from_l0_flag can be removed. It is recommended to consider adoption of the proposed simplification into VVC for SbTMVP.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).