| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0589 |
| --- | --- |

| Title: | Non-CE4: Simplification of HMVP in merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Tel: Email: | +1-858-705-2600 yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, the MV redundancy checking is applied to the first two HMVP candidates against the left and above spatial candidates (up to 4 MV redundancy checking) when inserting the HMVP candidate into the regular merge list. It is proposed to apply the MV redundancy check to only the first HMVP candidate. Moreover, the insertion order of HMVPs from the HMVP table is also changed to a reverse order (i.e. insertion from the first to latest entry HMVP) because the HMVPs which is located closer to the first entry has less possibility to be identical to the spatial neighboring MVs.

Besides, when inserting HMVP candidates into the regular merge list, there is one check to see whether the total number of available merge candidates reaches the maximally allowed merge candidates minus one. If the size reaches the maximum size minus one, the merge candidate list construction process from HMVP is terminated. The motivation of this is to keep at least one entry for the pair-wise candidates for better coding efficiency. This contribution presents a method to remove this check on HMVP candidates with minor impact to the coding efficiency.

The results reportedly show that the first method introduces x.xx% (RA), x.xx%(LDB) BD-rate changes while the second method introduces -0.03% (RA), 0.04%(LDB) BD-rate changes against VTM-5.0. The combination of the two methods introduces x.xx% (RA), x.xx%(LDB) BD-rate changes against VTM-5.0. With the proposed methods, the number of MV redundancy checking is reduce from 4 to 2 and the check on the number of available merge candidates when inserting HMVPs can be removed.

# Introduction

In current VVC, as shown in Figure 1, there are up to 4 MV redundancy checking processes applied when inserting the HMVP candidates into the regular merge list. MV redundancy checking is not preferred when inserting candidates into the MV candidate list due to its additional logics. In this proposal, it is proposed to reduce the MV redundancy checking process while maintaining the similar coding performance.

Moreover, when inserting HMVP candidates into the regular merge list, in the current HMVP design, there is one check to see whether the total number of available merge candidates reaches the maximally allowed merge candidates minus one. If the size reaches the maximum size minus one, the merge candidate list construction process from HMVP is terminated.

The motivation of this is to keep at least one entry for the pair-wise candidates which is inserted after the HMVP for better coding performance. However, it needs one additional check whenever inserting one HMVP candidate. Moreover, this check becomes obstacle of inserting HMVP candidates when the size of the merge candidate list is small. For example, when the merge list size is one, HMVP is not allowed to be inserted due to this condition check. When the merge list size is two, the pairwise candidate has no chance to be inserted and the condition check on HMVP becomes meaningless. In this proposal, one method is proposed to remove the condition check while maintaining similar coding performance.

Figure 1. HMVPs in regular merge list

# Proposed method

Two methods are proposed in this contribution. In the first method, when inserting the HMVP candidates into the merge list, only the first HMVP candidate is compared against the left and above spatial candidates. To compensate the coding loss caused by less MV redundancy checking, the insertion order is changed from 6(newest)1(oldest) to 1(oldest)6(newest) because the HMVPs which is located closer to the first entry has less possibility to be identical to the spatial neighboring MVs.

In the second method, it is proposed to move the insertion position of the HMVP candidates to the position after the pair-wise merge candidate. Since the pair-wise candidates is put before the HMVP, it is not necessary to keep the additional check to check whether the total number of available merge candidates reaches “the maximally allowed merge candidates minus 1”. With the proposed modification to move the pair-wise candidate before the HMVP, the additional check utilized when inserting the HMVP can be removed.

The combination of the proposed two methods are also tested. In the combined method, 2 MV redundancy checking are removed and one condition check on the number of available candidates is also removed.

# Simulation results

The table below shows the BD-rate performance and processing time of the combined method which include the proposed first and second method. The simulations are conducted following the common test conditions [1] on top of reference software VTM-5.0 [2].

Table 1. Performance of the first method

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.06% | 0.00% | -0.07% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.08% | 0.11% | 0.29% | 99% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | -0.26% | 0.31% | 99% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Performance of the second method

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.07% | 0.01% | 100% | 99% |
| Class A2 | -0.02% | 0.06% | -0.04% | 99% | 101% |
| Class B | 0.00% | 0.01% | -0.12% | 100% | 100% |
| Class C | -0.07% | 0.02% | -0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.01% | -0.08% | 100% | 100% |
| Class D | -0.08% | -0.01% | 0.03% | 101% | 101% |
| Class F | 0.01% | 0.10% | 0.05% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.41% | 0.17% | 100% | 100% |
| Class C | 0.00% | 0.04% | 0.15% | 100% | 100% |
| Class E | 0.08% | 0.37% | 0.18% | 101% | 99% |
| Overall | 0.04% | 0.27% | 0.16% | 100% | 100% |
| Class D | 0.01% | -0.05% | 0.62% | 99% | 101% |
| Class F | 0.02% | 0.07% | -0.74% | 100% | 101% |

Table 1. Performance of the combination of the first and second methods

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.03% | -0.10% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.14% | 0.24% | 98% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.54% | 0.87% | 97% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

In this proposal, two methods are proposed to reduce the MV redundancy checking and to remove the additional checking on the number of available merge candidates when inserting the HMVP candidates. The results show that the combination of the proposed method introduces only x.xx%, x.xx% BD-rate change against VTM-5.0 while the MV redundancy checking is reduce by 2 and the additional check could be removed when inserting the HMVPs into the regular merge list. It is recommended to consider adoption of the proposed simplification into VVC for HMVP.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).