| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0590 |
| --- | --- |

| Title: | Non-CE9: Improvement of DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, there is no control flag to control the enabling of the DMVR. However, it is not guaranteed that the DMVR refined MV is always better than the one before the refinement. In case DMVR refinement process produces a MV that is worse than the original one, coding efficiency may be scarified. To reduce the penalty of the uncertainty of DMVR refinement, it is proposed to favor the original MV during the DMVR process. In the proposed method, the SAD between the reference blocks referred by the initial MV is decreased by 1/4 of the SAD value. The modification is minimum which adding only one line to both the software and working draft. In terms of computational complexity, only one shift and add is added for each DMVR process unit. The simulation results reportedly show that the proposed method shows {-0.06%, -0.06%, -0.07%} BD-rates changes on Y, Cb and Cr components for RA over VTM-5.0.

# Introduction

Decoder-side Motion Vector Refinement (DMVR) utilizes the bilateral matching (BM) to derive motion information of the current CU by finding the closest match between two blocks along the motion trajectory of the current CU in two different reference pictures. The BM method calculates the distortion between the two candidate blocks in the reference picture list L0 and list L1. As illustrated in Figure 1, the SAD between the red blocks based on each MV candidate around the initial MV is calculated. The MV candidate with the lowest SAD becomes the refined MV and used to generate the bi-predicted signal. The cost function used in the matching process is row-subsampled SAD (sum of absolute difference).

Figure 1 – Decoding side motion vector refinement

# Proposed Methods

In this proposal, to reduce the penalty of the uncertainty of DMVR refinement, it is proposed to favor the original MV during the DMVR process. The SAD between the reference blocks referred by the initial (or called original) MV candidate is decreased by 1/4 of the SAD value.

# Results

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementation under CTC [2] are shown below against VTM-5.0.

Table 1. Performance of Test 1

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | 0.06% | 100% | 100% |
| Class A2 | -0.06% | -0.02% | -0.04% | 100% | 100% |
| Class B | -0.09% | -0.03% | -0.09% | 100% | 100% |
| Class C | -0.08% | -0.09% | -0.17% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.06% | -0.07% | 100% | 100% |
| Class D | -0.08% | -0.18% | 0.01% | 99% | 101% |
| Class F | -0.03% | 0.00% | 0.00% | 100% | 100% |

# Conclusion

In this proposal, the SAD value between the reference blocks referred by the initial MV candidate is decreased by 1/N of the SAD. Therefore, the initial MV is favored relative to other candidate MVs since its SAD value is decreased. The modification provides {-0.06%, -0.06%, -0.07} BD-rate increase for Y, Cb and Cr components under RA. It is recommended to consider adoption of the proposed modification into VVC for DMVR improvement.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).