| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0591 |
| --- | --- |

| Title: | Modifications to support the lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Xianglin Wang | Email: | tsung-chuanma@kwai.com yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, several changes are proposed to VVC to support lossless coding. First, several software fixes are made to VTM-5.0 to enable lossless encoding and decoding. Second, the newly developed transform tools (i.e. MTS, SBT, LFNST) and in-loop filters (i.e. ALF) are exclusively disabled for a lossless coded CU. The enabling flags of these transform and in-loop filter tools are not signaled for a lossless CU. Third, the coding tools which utilize decoder side derivation (i.e. BDOF and DMVR) introduce additional decoder complexity but they do not introduce noticeable BD-rate improvement to the lossless coding. It is proposed to disable the BDOF and DMVR for a lossless coded CU.

When compared with HEVC RExt, the BD rate reduction of VTM with the proposed changes are -5.24% (AI), -3.12% (RA) and -2.60%(LB) for CTC sequences, respectively. VTM with the proposed changes introduce -16.77% (AI), -14.19% (RA) and -10.72%(LB) for Class F and -48.71% (AI), -30.07% (RA) and -26.87% (LB) for Class TGM, respectively.

# Introduction

The lossless coding mode is achieved by simply bypassing transform, quantization, and in-loop filters (de-blocking filter, sample adaptive offset, and adaptive loop filter). In VVC, several new tools are developed for transform, quantization and in-loop filter but some of them does not support lossless coding.

## Transform tools

The transform coding tools and the lossless coding mode shall be exclusively enabled. However, in VVC, the transform coding tools include MTS, subblock transform (SBT) and LFNST still can be signaled as enabled for a lossless coded CU.

## In-loop filters

In current VVC, de-blocking filter and SAO are not applied to a CU which coded as lossless mode while ALF still applies.

# Proposed Methods

In this proposal, it is aimed to enable the lossless coding with minimum changes required to the regular VVC encoder and decoder implementation for mainstream applications. The proposed modifications are elaborated in the following sections.

## Transform tools

The transform coding tools and the lossless coding mode are exclusively enabled. The transform coding tools include MTS, subblock transform (SBT) and LFNST can not be turned on for a CU coded as lossless mode (cu_transquant_bypass_flag =1). One example of the signaling of SBT flag is illustrated as below. The detail modification can be seen in the attached WD.

| if(!cu_transquant_bypass_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |

## In-loop filters

In current VVC, de-blocking filter and SAO are not applied to a CU which coded as lossless mode while ALF still applies. It is proposed to prohibit the in-loop filtering of a lossless coded block. Specifically, when ALF is enabled for a CTB, the ALF filtering is not applied to the CBs which are coded as lossless mode and are located within this CTB. The modification of the decoding process of ALF is illustrated as followed.

- The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows: - If one or more of the following conditions are true, alfPictureL[ xCtb + x ][ yCtb + y ] is set equal to recPictureL[ hx, vy ], - pcm_loop_filter_disabled_flag and pcm_flag[ xCtb+ x ][ yCtb + y ] are both equal to 1 - cu_transquant_bypass_flag[ xCtb+ x ][ yCtb + y ] is equal to 1 - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum ) (8-1208)

## Disallowing BDOF and DMVR

It is observed that BDOF and DMVR do not introduce coding performance improvement for lossless coding. In this proposal, it is proposed to disabled BDOF and DMVR for a lossless coded CU.

## Software fixes

In VTM, some issues related to software implementation are fixed:

- the residual buffer for lossless CU is enlarged to 128x128

- fix the issue when calculating the RD cost of CIIP

- encoding fast algorithm “REUSE_CU_RESULTS” is not supported for lossless encoding

- the bits calculation for the merge modes RD “JVET_N0327_MERGE_BIT_CALC_FIX” is not supported for lossless coding

Moreover, for LMCS and Joint Chroma Residual Coding, due to these two coding tools are lossy operations, it is proposed to bypass both tools at encoder side to achieve CU level lossless coding when pps_transquant_bypass_enabled_flag is set equal to 1.

# Results

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementation are shown in Table 1 below against the HM-16.19 under CTC [2] with the additional configurations “--TransquantBypassEnable=1 --CUTransquantBypassFlagForce=1 --CostMode=lossless”. Results of TGM classes are also included. For Table 2, the result snows the comparison between HM-16.19 and proposed implementation with enabling BDOF and DMVR.

Table 1. Performance comparison between HM-16.19 and proposed modifications on VTM-5.0 (Disable BDOF and DMVR)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | HM | VTM |  | HM | VTM |  | HM | VTM |  |
| Class A1 | 2.1 | 2.2 | -6.11% | 2.2 | 2.3 | -3.97% | 0.0 | 0.0 | 0.00% |
| Class A2 | 1.7 | 1.7 | -3.57% | 1.8 | 1.9 | -2.61% | 0.0 | 0.0 | 0.00% |
| Class B | 2.0 | 2.1 | -4.26% | 2.2 | 2.3 | -2.72% | 2.2 | 2.3 | -2.47% |
| Class C | 2.0 | 2.1 | -5.34% | 2.5 | 2.6 | -3.18% | 2.5 | 2.6 | -2.78% |
| Class D | 1.8 | 1.9 | -5.37% | 2.6 | 2.8 | -3.57% | 2.7 | 2.7 | -3.13% |
| Class E | 2.7 | 2.9 | -6.94% | 0.0 | 0.0 | 0.00% | 3.2 | 3.3 | -2.56% |
| Class F | 4.5 | 5.5 | -16.77% | 30.3 | 40.3 | -14.19% | 47.9 | 61.4 | -10.72% |
| Overall | 2.1 | 2.2 | -5.24% | 2.2 | 2.2 | -3.12% | 2.6 | 2.7 | -2.60% |
| TGM | 6.2 | 13.1 | -48.71% | 81.3 | 113.8 | -30.07% | 108.0 | 142.8 | -26.87% |
| Enc Time[%] | 3174% |  |  | 1260% |  |  | 917% |  |  |
| Dec Time[%] | 183% |  |  | 186% |  |  | 140% |  |  |

Table 2. Performance comparison between HM-16.19 and proposed modifications on VTM-5.0 (Enable BDOF and DMVR)

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  | Low delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | compression ratio |  |  |  | Bit-rate saving |  | compression ratio |  |  |  | Bit-rate saving |  | compression ratio |  |  |  | Bit-rate saving |
|  |  | HM |  | VTM |  |  |  | HM |  | VTM |  |  |  | HM |  | VTM |  |  |
| Class A1 |  | 2.1 |  | 2.2 |  | -6.11% |  | 2.2 |  | 2.3 |  | -3.95% |  | 0.0 |  | 0.0 |  | 0.00% |
| Class A2 |  | 1.7 |  | 1.7 |  | -3.57% |  | 1.8 |  | 1.9 |  | -2.59% |  | 0.0 |  | 0.0 |  | 0.00% |
| Class B |  | 2.0 |  | 2.1 |  | -4.26% |  | 2.2 |  | 2.3 |  | -2.67% |  | 2.2 |  | 2.3 |  | -2.47% |
| Class C |  | 2.0 |  | 2.1 |  | -5.34% |  | 2.5 |  | 2.6 |  | -3.23% |  | 2.5 |  | 2.6 |  | -2.78% |
| Class D |  | 1.8 |  | 1.9 |  | -5.37% |  | 2.6 |  | 2.8 |  | -3.73% |  | 2.7 |  | 2.7 |  | -3.13% |
| Class E |  | 2.7 |  | 2.9 |  | -6.94% |  | 0.0 |  | 0.0 |  | 0.00% |  | 3.2 |  | 3.3 |  | -2.56% |
| Class F |  | 4.5 |  | 5.5 |  | -16.77% |  | 30.3 |  | 40.3 |  | -14.18% |  | 47.9 |  | 61.4 |  | -12.29% |
| Overall |  | 2.1 |  | 2.2 |  | -5.24% |  | 2.2 |  | 2.2 |  | -3.11% |  | 2.6 |  | 2.7 |  | -2.60% |
| TGM |  | 6.2 |  | 13.1 |  | -48.71% |  | 81.3 |  | 114.0 |  | -30.09% |  | 108.0 |  | 142.8 |  | -26.87% |
| Enc Time[%] | 3274% |  |  |  |  | 1324% |  |  |  |  | 946% |  |  |  |  |
| Dec Time[%] | 187% |  |  |  |  | 195% |  |  |  |  | 143% |  |  |  |  |

Moreover, the VTM with the proposed modifications also supports the mixed of lossy and lossless CUs coding (i.e. the middle area of each picture is forced to be coded in lossless mode while the other area are lossy coded) with the configurations “--TransquantBypassEnable=1 --CUTransquantBypassFlagForce=0 --CostMode=mixed_lossless_lossy”. Some examples of the reconstructed pictures are shown in Figure1. In these examples, a rectangle located in the middle is the region of interest and is coded as lossless while other area are lossy coded using the regular configurations. Figure 1 is an example for mixed_lossless_lossy coding with proposed modification in VTM while Figure 2 is the original source frame for comparison.

Sequence: BlowingBubble, Qp=32, CostMode=mixed_lossless_lossy, Frame # 389, Low delay B Picture

Figure 1. reconstructed pictures with the mixed coding of lossy and lossless CUs.

Sequence: BlowingBubble, Frame # 389

Figure 2. original pictures of Figure 1.

# Conclusion

In this proposal, we propose several modifications to VVC to support lossless coding. First, several issues with software implementation are fixed. Second, the newly developed transform tools (i.e. MTS, SBT, LFNST) and in-loop filters (i.e. ALF) are exclusively disabled for a lossless coded CU. Third, the coding tools which utilize decoder side derivation (i.e. BDOF and DMVR) are disabled for lossless CUs. Compared to HM16.19, VTM with the proposed changes introduce -5.24% (AI), -3.12% (RA) and -2.60%(LB) for CTC sequences, respectively. For class F, the result is -16.77% (AI), -14.19 (RA) and -10.72%(LB). For class TGM, the result is -48.71% (AI), -30.07% (RA) and -26.87%(LB). It is recommended to adopt the proposed modifications into VVC.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).