| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0592 |
| --- | --- |

| Title: | CE4-related: Motion estimation improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, two encoder improvements are proposed to enhance the motion estimation (ME) efficiency of the VTM-5.0 encoder, including 1) adding the motion vector (MV) candidates from previously inter coding units (CUs) for the starting point selection of integer-pel ME; 2) local refinement of the control-point motion vectors (CPMVs) for affine ME. Compared to the VTM-5.0 anchors, the proposed encoder improvements reportedly provides average {Y, U, V} BD-rate savings of {0.31%, 0.52%, 0.53%} and {0.16%, 0.16%, 0.10%} for RA and LD configurations, respectively, with 1% encoding time increase.

# Introduction

Like the HEVC test model (HM), block-matching based motion estimation (ME) is used at the VTM-5.0 encoder [1] to derive the optimal motion vectors (MVs) for regular inter coding units (CUs). Instead of searching all the MV candidates at fractional positions, one test zone (TZ) algorithm is firstly applied to find the best MV at integer-pel accuracy followed by half-pel and quarter-pel motion refinements around the best integer MV when the adaptive motion vector resolution (AMVR) is disabled for the current CU. Figure 1 illustrates the flowchart of the TZ algorithm in the VTM-5.0, which mainly contains the following steps:

- The MVP candidate obtained from the AMVP process is tested.

- The zero MV is tested.

- Hash-based ME can be optionally enabled. When the hash-based ME succeeds, the corresponding best MV will be selected and the whole ME process is early terminated.

- When no hash match is identified, one local motion search is further carried out by checking the neighboring integer MVs around the starting integer MV (as determined from step #1 and #2) in diamond search pattern. After that, one additional raster search may be conducted when the difference between the MVs before and after the diamond search is larger than one pre-defined threshold.

Figure 1 Flowchart of the TZ algorithm in the VTM-5.0

For the affine ME in the VTM-5.0 [1], after the optical-flow-based estimation of the control-point motion vectors (CPMVs), one local refinement is applied to improve the precision of each CPMV. Specifically, denote one affine CPMV as (MVx, MVy), its eight nearest neighboring MVs centered at (MVx, MVy) with a distance offset (which is determined based on the applied AMVR precision), i.e., (MVx – offset, MVy – offset), (MVx – offset, MVy + offset), (MVx + offset, MVy + offset), (MVx + offset, MVy – offset), (MVx – offset, MVy), (MVx, MVy – offset), (MVx, MVy + offset) and (MVx + offset, MVy) are further tested. The process is repeated by updating the search center until the best CPMV is not updated or the maximum iteration is reached. In the VTM-5.0, such local CPMV refinement process is only enabled when the affine AMVR is enabled, i.e., 1/16-pel and 1-pel MV precision cases.

# Proposal

To further improve the inter coding efficiency, the following non-normative changes are proposed to enhance the ME algorithms used for the regular inter mode and the affine mode in the VTM-5.0:

- Firstly, one MV candidate list is maintained at encoder which contains at most the MV candidates of twenty previous CUs according to the coding order. The list is emptied at the beginning of one slice and updated after the ME of one CU is done by replacing partial old MV candidates in the list with the optimal MVs of the current CU. The MV candidates in the list will be rounded to the integer precision and used as the additional candidates to select the optimal starting MV for the integer motion search and the initial MV candidates for the symmetrical motion vector difference (SMVD) mode.

- Secondly, it is proposed to enable the CPMV local refinement process to the 1/4-pel CPMV precision. Additionally, in order to control encoding complexity, the eight neighboring MVs are divided into two groups. The group #1 contains the MVs of four nearest neighbors in a cross shape, i.e., (MVx – offset, MVy), (MVx, MVy – offset), (MVx, MVy + offset) and (MVx + offset, MVy) while the group #2 contains the rest neighboring MVs, i.e., (MVx – offset, MVy – offset), (MVx – offset, MVy + offset), (MVx + offset, MVy + offset), (MVx + offset, MVy – offset). The neighboring MVs in the group #1 are firstly tested and the corresponding search is updated. The neighboring MVs in the group #2 will be entirely skipped if the group #1 does not update the search center.

# Simulation results

The proposed methods were implemented based on the VTM-5.0 reference software [2] and tested using the common test condition (CTC) as specified in [3]. Table 1 shows the overall BD-rate performance of the proposed encoder improvements for RA and LD configurations, respectively, compared to VTM-5.0 anchors.

Table 1 BD-rate performance of the proposed encoder improvements compared to VTM-5.0 anchors

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.42% | -0.31% | 102% | 101% |
| Class A2 | -0.40% | -0.54% | -0.54% | 101% | 99% |
| Class B | -0.25% | -0.51% | -0.47% | 101% | 100% |
| Class C | -0.42% | -0.61% | -0.78% | 100% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.52% | -0.53% | 101% | 99% |
| Class D | -0.33% | -0.49% | 0.01% | 100% | 101% |
| Class F | -0.37% | -0.42% | -0.45% | 102% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.28% | -0.01% | 100% | 100% |
| Class C | -0.22% | -0.34% | -0.28% | 103% | 99% |
| Class E | -0.06% | 0.29% | 0.00% | 100% | 100% |
| Overall | -0.16% | -0.16% | -0.10% | 101% | 100% |
| Class D | -0.21% | 0.07% | 0.12% | 102% | 95% |
| Class F | -1.20% | -1.36% | -1.40% | 100% | 102% |

# References

J. Chen, Y. Ye and S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).