| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0593-r1 |
| --- | --- |

| Title: | CE4-related: Harmonization of BDOF and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# In this update, complete simulation results are provided.

# Abstract

Prediction refinement with optical flow (PROF) was proposed at the 14th JVET meeting and is now studied in core experiment (CE) 4 test 2.1. Similar to bidirectional optical flow (BDOF), the PROF provides sample-wise refinement on top of subblock based affine motion compensation based on optical flow model. In this contribution, two modifications are proposed to the existing PROF design in order to harmonize with the BDOF. In the first modification, the precisions of sample gradients and motion refinements used in the PROF are modified to be the same as that of the BDOF. In the second modification, it is proposed to modify the bi-prediction process of the PROF such that the PROF refinements are derived and applied during the combination of L0 and L1 prediction signals. It is reported that the performance impact of the proposed modifications is minor. Compared to the CE4-2.1 results, the overall {Y, U, V} BD-rate variations are {0.01%, -0.02%, -0.02%} and {0.02%, -0.03%, 0.34%} for the RA and LD configurations, respectively.

# Introduction

In the VVC working draft (WD) 5 [1], bidirectional optical flow (BDOF) is applied to enable the optical flow based motion refinement for bi-predicted coding units (CUs) that has both forward and backward predictions. In [2], prediction refinement with optical flow (PROF) is proposed to refine the subblock based affine motion compensation using the optical flow model. The method was included in core experiment (CE) 4 test 2.1 for further evaluation [3].

# Problem statement

Given that both the BDOF and the PROF are built upon the optical flow concept, it is highly desirable to harmonize the BDOF and PROF designs as much as possible that the PROF can maximally leverage the existing logics of the BDOF to facilitate hardware implementations. Based on such consideration, the following two discrepancies are identified between the current BDOF and PROF designs.

## Different precisions of sample gradients and motion refinements for the BDOF and the PROF

Firstly, the precisions of the applied sample gradients and motion refinements are not fully aligned when calculating per-sample refinements for the BDOF and the PROF. Specifically, in the current PROF, the motion refinement Δv at sample position x,y is derived as

| Δvx=clip3-2max⁡6, BD-6, 2max⁡6, BD-6-1,2L∙c∙Δx+d∙Δy≫7Δvy=clip3-2max⁡6, BD-6, 2max⁡6, BD-6-1,2L∙e∙Δx+f∙Δy≫7 | (1) |
| --- | --- |

where Δx and Δy be the horizontal and vertical offset represented at 1/4-pel accuracy from the position x,y to the center of the sub-block that the sample belongs to; L is the bit-depth increase to ensure the accuracy of the derived motion refinements, which is set to 7 in [2]; BD is the internal coding bit-depth; c, d, e and f are the affine parameters which are derived based on the affine control-point motion vectors (CPMVs); thPROF is the motion refinement threshold to prevent irregular motion which is equal to 2max⁡6, BD-6. Therefore, as shown in (1), the derived PROF motion refinement magnitude is in the precision of max⁡6, BD-6 bit.

On the other hand, in the VVC WD 5 [1], the BDOF motion refinements at sample position x,y are derived as

| Δvx=clip3-2max⁡5, BD-7, 2max⁡5, BD-7-1,sGxdI≪3≫log2sGx2Δvy=clip3-2max⁡5, BD-7, 2max⁡5, BD-7-1,Δvx∙sGxGym≪12+Δvx∙sGxGys≫1≫log2sGx2 |  |
| --- | --- |

where sGxdI, sGxGym, sGxGys and sGx2 are the BDOF correlation parameters [1]; thBDOF is the motion refinement threshold of the BDOF, which is equal to 2max⁡5, BD-7. Thus, the corresponding precision of the derived BDOF motion refinement magnitude is max5, BD-7 bit. Based on the comparison of (1) and (2), different precisions are used to represent the motion refinements for the BDOF and the PROF. Specifically, the motion refinement precision applied by the PROF is 1-bit higher than that used by the BDOF.

Secondly, it is reported that there is one mismatch between the proposed text in [2] and the CE4-2.1 software on the precision of the sample gradients that are used to calculate PROF sample refinements. Specifically, according to the proposed text in [2], the horizontal and vertical gradients of one sample at position x,y are calculated from its two neighboring samples, i.e.,

| ghx,y=Ix+1, y-Ix-1, y≫max⁡(6, BD-6) gvx, y=Iix,y+1-Ix,y-1≫max⁡(6, BD-6) | (3) |
| --- | --- |

where Ix,y is sample value at position x,y. Note that equation (3) is also the gradient derivation method used by the BDOF. But in the CE4-2.1 software, the horizontal and vertical gradients are calculated as

| ghx,y=Ix+1, y-Ix-1, y≫6 gvx, y=Iix,y+1-Ix,y-1≫6 | (4) |
| --- | --- |

## Different bi-prediction workflow of the BDOF and the PROF

In CE4-2.1, when one affine coding unit (CU) is bi-predicted, the PROF based sample refinement is applied in a unilateral manner by separately refining the prediction samples in list L0 and L1. After that, the refined L0 and L1 prediction samples are combined based on bi-prediction with CU-level weight (BCW). More specifically, the bi-prediction samples of one affine CU with the PROF are calculated by the following steps:

- For each sample at position (x,y), calculate the L0 prediction refinement ΔI0(x,y) based on the PROF and add the refinement to the original L0 prediction sample I0x,y, i.e.,

| ΔI0(x,y)= gh0(x,y)∙Δvx0(x,y)+gv0(x,y)∙Δvy0(x,y) I0'x,y=I0x,y+clip3-2dI,2dI-1, ΔI0x,y+1≫1 | (5) |
| --- | --- |

where I0'x,y is the refined sample and dI is sample refinement threshold which is set to 14; gh0(x,y) and gv0(x,y) and Δvx0(x,y) and Δvy0(x,y) are the L0 horizontal/vertical gradients and the L0 horizontal/vertical motion refinements at position (x,y).

- For each sample at position (x,y), calculate the L1 prediction refinement ΔI1(x,y) based on the PROF and add the refinement to the original L1 prediction samples I1x,y, i.e.,

| ΔI1x,y= gh1x,y∙Δvx1x,y+gv1x,y∙Δvy1x,y I1'x,y=I1x,y+clip3-2dI,2dI-1, ΔI1x,y+1≫1 | (6) |
| --- | --- |

where I1'x,y is the refined sample; gh1(x,y) and gv1(x,y) and Δvx1(x,y) and Δvy1(x,y) are the L1 horizontal/vertical gradients and the L1 horizontal/vertical motion refinements at position (x,y).

- Combine the refined L0 and L1 prediction samples, i.e.,

| Ibix,y=8-w∙I0'x,y+w∙I1'x,y+οoffset≫shift | (7) |
| --- | --- |

where is the BCW weight; shift and οoffset are the offset and right shift that are applied to the weighted average of the L0 and L1 prediction signals for bi-prediction, which are equal to 14-BD+3 and 1≪14-BD+2+8∙1≪13, respectively.

The above process is different from the BDOF where the sample refinements are derived and applied when combining the L0 and L1 prediction samples, i.e.,

| Ibix,y=I0x,y+I1x,y+bBDOF+ο'offset≫shift' | (8) |
| --- | --- |

where is the bBDOF is the sample refinement derived by the BDOF; shift' and ο'offset are the offset and right shift that are applied to the average of the L0 and L1 prediction signals for bi-prediction. Because the BCW is disabled for the BDOF, the values of shift' and ο'offset are equal to 15-BD and 1≪14-BD+2∙1≪13, respectively.

It is asserted that such difference between the bi-prediction workflow of the BDOF and the PROF could potentially cause the following problems for practical hardware design:

- Because the BDOF is applied to non-affine CUs and the PROF is applied to affine CUs, the two coding tools cannot be invoked simultaneously. Therefore, for practical hardware implementation, it is desirable to carry out the BDOF and the PROF within the same pipeline stage. However, due to the different bi-prediction workflow, it is hard to assign the same pipeline process for both the BDOF and the PROF.

- Like the HEVC, additional memory is needed by the VVC to store the intermediate L0 and L1 prediction samples that are used to generate the final prediction samples of one bi-predicted CU. Because the dynamic range of intermediate samples is 16-bit, the storage requirement for the intermediate sample buffer is 16-bit per sample in the current VVC WD. However, because the bi-prediction samples of the PROF are generated by averaging the refined L0 and L1 prediction samples (which are formed by adding PROF sample refinements to intermediate L0 and L1 prediction samples), the size of the required intermediate sample storage will be increased.

# Proposal

To achieve one more uniform design, two changes are proposed to modify the PROF to be aligned with the existing design of the BDOF.

## Unified precisions of sample gradients and motion refinements for the BDOF and the PROF

In the first modification, it is proposed to unify the precisions of the sample gradients and motion refinements used by the BDOF and the PROF. Specifically, the sample gradient and motion refinement precisions that are used in the current BDOF design are reused for the PROF. By such modification, the values of the right shift and the motion refinement threshold thPROF in equation (1) are changed to 13-max⁡5, BD-7 and 2max⁡5, BD-7 (i.e., the value of thBDOF) while the values of the other variables in equation (1) are kept unchanged. Correspondingly, with the first modification, the motion refinement Δv at sample position x,y is derived as

| Δvx=clip3-thBDOF, thBDOF-1,2L∙c∙Δx+d∙Δy≫13-max5, BD-7Δvy=clip3-thBDOF, thBDOF-1,2L∙e∙Δx+f∙Δy≫(13-max⁡5, BD-7) | (9) |
| --- | --- |

## Additionally, to fix the mismatch between the proposed text and the software of CE4-2.1, it is proposed to change the sample gradient calculation in the software to be the same as the method used in the text, i.e., equation (3) .

## Harmonized bi-prediction workflow for the BDOF and the PROF

In the second modification, it is proposed to modify the bi-prediction process of the PROF to be fully aligned with that of the BDOF. Specifically, instead of separately applying the refinement for each prediction direction, the PROF sample refinements are derived and applied during the combination of the L0 and L1 prediction samples in the proposed method. Based on the derived L0 and L1 prediction refinements ΔI0(x,y) and ΔI1(x,y) in equations (5) and (6), the final prediction samples of one bi-predicted CU are derived by

| Ibix,y=8-w∙I0x,y+w∙I1x,y+b+οoffset≫shift b=8-w∙ΔI0(x,y)+w∙ΔI1(x,y) | (10) |
| --- | --- |

Note that in equation (10) the unnecessary clipping operations in the existing PROF (as shown in (5) and (6)) are removed. Figure 2 shows one example to illustrate the possible pipeline stage when the proposed method is applied. As shown in Figure 2, after the bi-prediction workflow for the BDOF and PROF is harmonized, the same pipeline stage can be shared by the two coding tools. Additionally, because in the proposed method the input the PROF process is the initial L0 and L1 intermediate prediction samples (i.e., 16-bit, it does not need to increase the storage requirement for intermediate sample buffer.

Figure 1 Example pipeline stages with the BDOF and the PROF

# Simulation results

The proposed method was implemented based on the CE4-2.1a reference software [2] and tested following the common test conditions as specified in [4]. In the following tables, the anchors are the results of CE4-2.1 and the tests are the results of the anchors plus the proposed methods being applied.

Table 1 BD-rate performance of the proposed modifications compared to CE4-2.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | -0.01% | 98% | 97% |
| Class A2 | 0.01% | 0.04% | -0.08% | 96% | 96% |
| Class B | 0.03% | 0.03% | -0.04% | 100% | 99% |
| Class C | -0.01% | -0.02% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.02% | 99% | 98% |
| Class D | -0.04% | -0.20% | -0.12% | 100% | 100% |
| Class F | 0.01% | -0.01% | 0.05% | 99% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.33% | 100% | 100% |
| Class C | 0.00% | -0.30% | -0.09% | 95% | 93% |
| Class E | 0.05% | 0.18% | 0.93% | 101% | 100% |
| Overall | 0.02% | -0.03% | 0.34% | 99% | 97% |
| Class D | -0.04% | 0.64% | -1.09% | 98% | 97% |
| Class F | 0.03% | 0.71% | -0.57% | 100% | 97% |

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

J. Luo, Y. He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, July 2019, Gothenburg, Sweden.

H. Yang, C.-C. Chen, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-N1024, March 2019, Geneva, Switzerland.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).