| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0595 |
| --- | --- |

| Title: | CE4-related: Modified adaptive motion vector resolution (AMVR) for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the VVC specification draft 5, adaptive motion vector resolution (AMVR) is applied to affine mode to reduce the overhead of signaling the motion vector differences (MVDs). In the current affine AMVR design, the MVD resolution of one affine block can be adaptively selected among 1/4-pel, 1/16-pel and 1-pel. In this contribution, one modified MVD resolution set {1/4-pel, 1/8-pel and 1-pel} is proposed for the affine AMVR mode. Simulation results show that compared to the VTM-5.0 anchors, the proposed modification reportedly provides {Y, U, V} BD-rate savings of {0.07%, 0.10%, 0.15%} for RA configuration without increasing the encoding and decoding time. Additionally, when the affine AMVR is enabled for LD configuration, the corresponding BD-rate savings of the proposed modification are {0.05%, 0.05%, 0.40%}.

# Introduction

To reduce the overhead of signaling motion vector differences (MVDs), adaptive motion vector resolution (AMVR) was extended to affine mode at Marrakech meeting [1], wherein multiple MVD resolutions, including 1/4-pel, 1/16-pel and 1-pel, can be adaptively selected for affine coding unit (CU).

# Proposal

In this contribution, one modified AMVR resolution set {1/4-pel, 1/8-pel and 1-pel} is proposed for the affine AMVR. Specifically, for one affine CU that has at least one non-zero MVD, one first flag amvr_flag is signaled to indicate whether the AMVR is applied to the current CU. When the flag is equal to 0, no further syntax is signaled and 1/4-pel precision is used to signal the MVDs of the CU. Otherwise (i.e., amvr_flag is equal to 1), one second flag amvr_precision_flag is signaled to indicate whether 1/8-pel or 1-pel MVD resolution is selected.

Except the modified MVD resolutions, the existing affine AMVR design, e.g., syntax elements, binarization, context modeling, and the encoder logics that are applied in this proposal are kept the same as that used in the VVC specification draft 5 [2] and the VTM-5.0 [3].

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [3] and tested following the common test conditions (CTC) as specified in [4].

Table 1 BD-rate performance of the proposed modification compared to VTM-5.0 anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.18% | -0.02% | 100% | 101% |
| Class A2 | -0.11% | -0.08% | -0.10% | 100% | 103% |
| Class B | -0.07% | -0.08% | -0.19% | 98% | 98% |
| Class C | -0.06% | -0.08% | -0.24% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.10% | -0.15% | 100% | 100% |
| Class D | -0.01% | 0.04% | 0.16% | 100% | 95% |
| Class F | -0.07% | -0.02% | -0.03% | 100% | 101% |

In the CTC, the affine AMVR mode is not applied for the LD configuration. Therefore, additional tests are conducted to check the performance of the proposed modification for the LD. In Table 2, the anchors are the LD results of the VTM-5.0 with the affine AMVR being enabled (by setting the parameters AffineAmvr and AffineAmvrEncOpt to 1 in the encoder cfg file) and the tests are the results of the anchors plus the proposed modification.

Table 2 BD-rate performance of the proposed modification compared to VTM-5.0 anchor with enabling the affine AMVR mode for the LD

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with affine AMVR enabled for LD |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.04% | -0.15% | 99% | 101% |
| Class C | 0.00% | -0.11% | -0.18% | 99% | 101% |
| Class E | -0.12% | 0.42% | -1.10% | 102% | 100% |
| Overall | -0.05% | 0.05% | -0.40% | 100% | 101% |
| Class D | -0.09% | -0.50% | -0.62% | 102% | 98% |
| Class F | -0.03% | 0.42% | 0.40% | 102% | 104% |

# Proposed text changes

#### 7.4.8.5 Coding unit semantics

amvr_precision_flag[ x0 ][ y0 ] equal to 0 specifies that the resolution of the motion vector difference is one integer luma sample if inter_affine_flag[ x0 ][ y0 ] is equal to 0, and 1/168 of a luma sample otherwise. amvr_precision_flag[ x0 ][ y0 ] equal to 1 specifies that the resolution of the motion vector difference is four luma samples if inter_affine_flag[ x0 ][ y0 ] is equal to 0, and one integer luma sample otherwise. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When amvr_precision_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The motion vector differences are modified as follows:

- If inter_affine_flag[ x0 ][ y0 ] is equal to 0, the variable MvShift is derived and the variables MvdL0[ x0 ][ y0 ][ 0 ], MvdL0[ x0 ][ y0 ][ 1 ], MvdL1[ x0 ][ y0 ][ 0 ], MvdL1[ x0 ][ y0 ][ 1 ] are modified as follows:

MvShift = ( amvr_flag[ x0 ][ y0 ] + amvr_precision_flag[ x0 ][ y0 ] ) << 1	(7-113)

MvdL0[ x0 ][ y0 ][ 0 ] = MvdL0[ x0 ][ y0 ][ 0 ]  << ( MvShift + 2 )	(7-114)

MvdL0[ x0 ][ y0 ][ 1 ] = MvdL0[ x0 ][ y0 ][ 1 ]  <<  ( MvShift + 2 )	(7-115)

MvdL1[ x0 ][ y0 ][ 0 ] = MvdL1[ x0 ][ y0 ][ 0 ]<<( MvShift + 2 )	(7-116)

MvdL1[ x0 ][ y0 ][ 1 ] = MvdL1[ x0 ][ y0 ][ 1 ]  << ( MvShift + 2 )	(7-117)

- Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variable MvShift is derived and the variables MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 0 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] and MvdCpL0[ x0 ][ y0 ][ 2 ][ 1 ] are modified as follows:

MvShift = amvr_precision_flag[ x0 ][ y0 ]  ? 
				 ( amvr_precision_flag[ x0 ][ y0 ]  <<  1 )  :  ( − (amvr_flag[ x0 ][ y0 ]  <<  1) )-amvr_flag[ x0 ][ y0 ] )	(7-118)

MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ]  << ( MvShift + 2 )	(7-119)

MvdCpL1[ x0 ][ y0 ] [ 0 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ]  <<  ( MvShift + 2 )	(7-120)

MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ]  << ( MvShift + 2 )	(7-121)

MvdCpL1[ x0 ][ y0 ] [ 1 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ]  <<  ( MvShift + 2 )	(7-122)

MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ]  << ( MvShift + 2 )	(7-123)

MvdCpL1[ x0 ][ y0 ] [ 2 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ]  <<  ( MvShift + 2 )	(7-124)

# References

H. Liu, K, Zhang, L. Zhang, J. Xu, J. Luo, Y. He, X. Xiu, “CE2-related: Joint Test of AMVR for Affine Inter Mode (Test 2.1.1 and Test 2.1.2)”, JVET-M0247, Jan. 2019, Marrakech, Morocco.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).