| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0596 |
| --- | --- |

| Title: | CE7-related: On chroma CBF signaling of subblock transform mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Subblock transform (SBT) mode is included in the VVC working draft 5 to enhance the transform coding efficiency of inter coding units (CUs). It is reported that there is a mismatch between the VVC working draft 5 and the test model software VTM-5.0 on signaling the coded block flags (CBFs) of chroma components for the SBT mode. Fixes are provided in this contribution to align the test model software with the text specification. Simulation results reportedly show the BD-rate impact of the proposed fixes is negligible.

# Introduction

Subblock transform (SBT) mode was adopted into the VVC at Marrakech meeting to improve the transform coding efficiency of inter coding units (CUs) [1]. When the SBT is applied, the residuals of one inter CU is implicitly partitioned into two transform units (TUs). The residuals in the first TU is coded with certain pre-defined transforms (e.g., DST-7 and DCT-8) and transmitted from encoder to decoder while the residuals in the second TU is always inferred to be zeros without signaling. In this contribution, it is reported that there is one mismatch between the VVC working draft 5 [2] and VTM-5.0 reference software [3] on chroma coded block flag (CBF).

Firstly, as per the VVC working draft 5 [2], two chroma CBFs tu_cbf_cb and tu_cbf_cr are signaled for each SBT TU separately and the presence of the chroma CBFs of one SBT TU is conditioned on the position information of the SBT partition. Specifically, chroma CBFs are only signaled for the SBT TU that contains non-zero coefficients but omitted for the other TU where the residuals are zero out, as illustrated by

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

Additionally, the TU depth trDepth = 1, which specifies the level of one TU partition inside the corresponding CU, is used to determine the context model (i.e., ctxInc = 1) to code the syntax tu_cbf_cb.

On the other hand, in the VTM-5.0 reference software [3], the two chroma CBFs are signaled at the CU level for the two chroma components, respectively. When the CBF of one chroma component is equal to 0, all the residuals of the chroma component are set to zero without signaling. Otherwise (i.e., the CBF is equal to 1), only the residuals of the non-zero SBT partition are signaled in bitstream. Correspondingly, because the chroma CBFs are signaled at CU level, the TU depth trDepth = 0 is used to select the context mode (i.e., ctxInc = 0) to code the syntax tu_cbf_cb.

# Proposal

To solve the mismatch, this contribution proposes to change the VTM-5.0 software to be aligned with the specification text. Additionally, after the proposed fix is applied, the chroma CBF signaling methods of all the TU partition scenarios in the VVC, i.e., the SBT mode, the intra sub-partition (ISP) mode and maximum-transform-size based TU partition, are well aligned by always signaling the two chroma CBFs at TU level.

# Simulation results

The proposed changes were implemented based on the VTM-5.0 reference software [3] and tested by following the common test conditions (CTCs) as specified in [4].

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.16% | 0.03% | 99% | 102% |
| Class A2 | 0.03% | 0.02% | 0.00% | 103% | 97% |
| Class B | 0.01% | 0.11% | -0.05% | 99% | 99% |
| Class C | -0.01% | -0.07% | -0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.04% | 100% | 100% |
| Class D | 0.01% | 0.04% | 0.11% | 104% | 97% |
| Class F | 0.01% | 0.06% | 0.04% | 100% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.66% | 0.06% | 100% | 99% |
| Class C | 0.01% | 0.26% | -0.31% | 100% | 102% |
| Class E | 0.07% | 0.27% | -0.70% | 100% | 99% |
| Overall | 0.01% | 0.43% | -0.25% | 100% | 100% |
| Class D | 0.01% | 0.13% | 0.33% | 99% | 100% |
| Class F | -0.05% | 0.10% | -0.43% | 100% | 100% |

# References

Y. Zhao, H. Gao, H. Yang, J. Chen, “CE6: Sub-block transform for inter blocks (Test 6.4.1)”, JVET-M0140, Jan. 2019, Marrakech, Morocco.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March. 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).