| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0598 |
| --- | --- |

| Title: | CE3-related: Simplification on MIP Look-up Table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Hong-Jheng Jhu Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com jhuhong-jheng@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove three tables in MIP module: g_intraMode65to33AngMapping, g_mapMipToAngular65 and g_mapAngular33ToMip. They are used to perform the intra mode conversion between MIP and conventional intra mode. According to this proposal, instead of performing mode conversion using those tables, a MIP mode is simply considered as “planar” where a conventional intra mode is needed, while a conventional intra mode is considered as “Not Available” where a MIP mode is needed. The proposed method is compared with both VTM5 JVET-O0084, with simulation results as follows:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are -0.03%, 0.01%, and 0.03% for AI, RA and LD, respectively.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are 0.02%, 0.02%, and 0.01% for AI, RA and LD, respectively.

# Introduction

In VTM-5.0 [1], the three look up tables (LUTs) g_intraMode65to33AngMapping, g_mapMipToAngular65 and g_mapAngular33ToMip are used for Matrix Intra Prediction (MIP) mode [2, 3] to perform the intra mode conversion between MIP and conventional intra mode. Specifically, these tables are used in MPM list construction and chroma mode derivation.

For example, when constructing the MIP MPM list, it is needed to check the coding mode of the above and left neighboring blocks. If the coding mode of a neighboring block is a conventional intra mode (i.e. not MIP), it has to be converted into a MIP mode through LUT. On the other hand, conversion from a MIP mode into a conventional intra mode is also needed in constructing conventional intra MPM list, where similar LUT operations are needed.

# Proposed Implementation

In this proposal, a method is proposed to remove these three LUTs, g_intraMode65to33AngMapping, g_mapMipToAngular65, and g_mapAngular33ToMip. Instead of performing mode conversion through LUT operations, a neighboring block with MIP mode is simply considered as Planar mode when constructing conventional intra MPM list, and a neighboring block with a conventional intra mode is considered as “Not Available” when constructing MIP MPM list. In deriving an intra chroma mode where the corresponding luma intra mode is needed, the corresponding luma intra mode is also considered as Planar if it is a MIP mode. The memory reduction of these three LUTs are 240 Bytes if one entry is 1 Byte.

# Results

The proposed modification is integrated into VTM-5.0 [1] and its results against VTM-5.0 under CTC [4] are shown in Table 1. The proposed method is also tested on top of JVET-O0084, with its BD-rate results shown in Table 2. For more details in each test, please refer to the attached spreadsheets.

Table 1. Results of proposed Test 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.06% | 0.09% | 101% | 100% |
| Class A2 | 0.00% | -0.01% | -0.04% | 101% | 100% |
| Class B | -0.02% | -0.01% | 0.01% | 101% | 100% |
| Class C | -0.03% | -0.02% | 0.09% | 102% | 99% |
| Class E | -0.04% | -0.04% | 0.01% | 102% | 99% |
| Overall | -0.03% | -0.01% | 0.03% | 101% | 100% |
| Class D | -0.04% | 0.11% | 0.04% | 101% | 101% |
| Class F | -0.03% | 0.00% | -0.05% | 103% | 100% |

Table 2. Results of proposed Test 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.16% | 101% | 100% |
| Class A2 | 0.03% | 0.04% | 0.02% | 102% | 100% |
| Class B | 0.01% | 0.04% | 0.03% | 101% | 99% |
| Class C | 0.03% | -0.01% | -0.01% | 102% | 99% |
| Class E | 0.02% | -0.02% | 0.06% | 101% | 101% |
| Overall | 0.02% | 0.02% | 0.05% | 101% | 100% |
| Class D | 0.02% | 0.13% | -0.12% | 100% | 100% |
| Class F | -0.02% | -0.02% | 0.04% | 102% | 100% |

# Conclusion

In this proposal, we proposed a mode-conversion method with no need of LUTs for intra mode conversion between MIP mode and conventional intra mode. Simulation results show no degradation on coding performance with the proposed method. We recommend to consider adoption of such simplifications into VVC.

# References

| [1] | "VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | J. Pfaff , B. Stallenberger, M. Schäfer, P. Merkle , P. Helle , T. Hinz, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)," Document JVET-N0217, Geneva, CH, 2019. |
| [3] | J. Pfaff , T. Hinz , P. Helle , P. Mekle , B. Stallenberger , M. Schäfer, H. Schwarz, D. Marpe and T. Wiegand, "8-bit implementation and simplification of MIP," Document JVET-O0084, Gothenburg, SE, 2019. |
| [4] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," Document JVET-N1010, Geneva, CH, March 2019. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).