### Derivation process for MIP mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the matrix-based intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb ) and ( xCb, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candMipModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate MIP mode candMipModeX is derived as follows:

- If one or more of the following conditions are true, candMipModeX is set equal to −1.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- CuPredMode[ xNbX ][ yNbX] is equal to MODE_INTRA and intra_mip_flag[ xNbX ][ yNbX ] is not equal to 1

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, the following applies:

- If intra_mip_flag[ xNbX ][ yNbX ] is equal to 1, the following applies:.

- If MipSizeId[ xCb ][ yCb ] is equal to MipSizeId[ xNbX ][ yNbX ], candMipModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- Otherwise, candMipModeX is set equal to −1.

- Otherwise, candMipModeX is -set equal to -1

- The candMipModeList[ x ] with x = 0..2 is derived as follows, using mipMpmCand[ sizeId ] as specified in Table 8-2 with sizeId set equal to MipSizeId[ xCb ][ yCb ]:

- If candMipModeA and candMipModeB are both equal to −1, the following applies:

candMipModeList[ 0 ] = mipMpmCand[ sizeId ][ 0 ]	(8-10)

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 1 ]	(8-11)

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-12)

- Otherwise, the following apllies:

- If candMipModeA is equal to candMipModeB or if either candMipModeA or candMipModeB is equal to −1, the following applies:

candMipModeList[ 0 ] = ( candMipModeA != −1 )  ?  candMipModeA  :  candMipModeB	(8-13)

- If candMipModeList[ 0 ] is equal to mipMpmCand[ sizeId ][ 0 ], the following applies:

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 1 ]	(8-14)

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-15)

- Otherwise, the following applies:

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 0 ]	(8-16)

candMipModeList[ 2 ] = ( candMipModeList[ 0 ] != mipMpmCand[ sizeId ][ 1 ] )  ?  
	mipMpmCand[ sizeId ][ 1 ]  :  mipMpmCand[ sizeId ][ 2 ]	(8-17)

- Otherwise, the following applies:

candMipModeList[ 0 ] = candMipModeA		(8-18)

candMipModeList[ 1 ] = candMipModeB		(8-19)

- If candMipModeA and candMipModeB are both not equal to mipMpmCand[ sizeId ][ 0 ], the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 0 ]	(8-20)

- Otherwise, the following applies:

- If candMipModeA and candMipModeB are both not equal to mipMpmCand[ sizeId ][ 1 ], the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 1 ]	(8-21)

- Otherwise, the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-22)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_mip_mpm_flag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ xCb ][ yCb ] is set equal to candMipModeList[ intra_mip_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candMipModeList[ i ] is greater than candMipModeList[ j ] for i = 0..1 and for each i, j = ( i + 1 )..2, both values are swapped as follows:

( candMipModeList[ i ], candMipModeList[ j ] ) = Swap( candMipModeList[ i ], 	candMipModeList[ j ] )	(8-23)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_mip_mpm_remainder[ xCb ][ yCb ].

  - For i ranging from 0 to 2, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candMipModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

-

Table 8-2 – Specification of MIP candidate modes mipMpmCand[ sizeId ][ x ]

| sizeId | candidate mode x |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 17 | 34 | 5 |
| 1 | 0 | 7 | 16 |
| 2 | 1 | 4 | 6 |

### Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-3 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1 or intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 0, IntraPredModeY[ xCb ][ yCb ] is set equal to INTRA_PLANAR.

- Otherwise (intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 1), the following ordered steps apply:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- CuPredMode[ xNbX ][ yNbX ] is equal to MODE_INTRA and intra_mip_flag[ xNbX ][ yNbX ] is equal to 1

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is derived as follows:

- -

- candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..4 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-24)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-25)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-26)

candModeList[ 3 ] = INTRA_DC		(8-27)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-28)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-29)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-30)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-31)

candModeList[ 1 ] = candIntraPredModeB	(8-32)

candModeList[ 2 ] = INTRA_DC		(8-33)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-34)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-35)

  - Otherwise, the following applies:

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-36)

candModeList[ 4 ] = 2 + ( ( maxAB ) % 64 )	(8-37)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = maxAB		(8-38)

candModeList[ 1 ] = INTRA_DC		(8-39)

candModeList[ 2 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-42)

- Otherwise, the following applies:

candModeList[ 0 ] = INTRA_DC			(8-43)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-44)

candModeList[ 2 ] = INTRA_ANGULAR18	(8-45)

candModeList[ 3 ] = INTRA_ANGULAR46	(8-46)

candModeList[ 4 ] = INTRA_ANGULAR54	(8-47)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..3 and for each i, j = ( i + 1 )..4, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-48)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - The value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

  - For i equal to 0 to 4, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

-

### Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is -set equal to INTRA_PLANAR

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6.

Table 8-5 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |