#### Derivation process for ctxTable, ctxIdx and bypassFlag

##### General

Input to this process is the position of the current bin within the bin string, binIdx.

Outputs of this process are ctxTable, ctxIdx and bypassFlag.

The values of ctxTable, ctxIdx and bypassFlag are derived as follows based on the entries for binIdx of the corresponding syntax element in Table 9-17:

- If the entry in Table 9-17 is not equal to "bypass", "terminate" or "na", the values of binIdx are decoded by invoking the DecodeDecision process as specified in clause 9.5.4.3.2 and the following applies:

- ctxTable is specified in Table 9-5

- The variable ctxInc is specified by the corresponding entry in Table 9-17 and when more than one value is listed in Table 9-17 for a binIdx, the assignment process for ctxInc for that binIdx is further specified in the clauses given in parenthesis.

- The variable ctxIdxOffset is specified in Table 9-5 depending on the current value of initType.

- ctxIdx is set equal to the sum of ctxInc and ctxIdxOffset.

- bypassFlag is set equal to 0.

- Otherwise, if the entry in Table 9-17 is equal to "bypass", the values of binIdx are decoded by invoking the DecodeBypass process as specified in clause 9.5.4.3.4 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 1.a

- Otherwise, if the entry in Table 9-17 is equal to "terminate", the values of binIdx are decoded by invoking the DecodeTerminate process as specified in clause 9.5.4.3.5 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 0.

- Otherwise (the entry in Table 9-17 is equal to "na"), the values of binIdx do not occur for the corresponding syntax element.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_brick_one_bit | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| alf_ctb_use_first_aps_flag | 0 | na | na | na | na | na |
| alf_use_aps_flag | 0 | na | na | na | na | na |
| alf_luma_fixed_filter_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| alf_luma_prev_filter_idx_minus1 | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | na | na | na | na |
| sao_eo_class_chroma | bypass | bypass | na | na | na | na |
| split_cu_flag | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| split_qt_flag | 0..5 (clause 9.5.4.2.2) | na | na | na | na | na |
| mtt_split_cu_vertical_flag | 0..4 (clause 9.5.4.2.3) | na | na | na | na | na |
| mtt_split_cu_binary_flag | ( 2 * mtt_split_cu_vertical_flag ) + ( mttDepth < = 1 ? 1 : 0 ) | na | na | na | na | na |
| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_flag | 0,1 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_ibc_flag | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_bdpcm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_bdpcm_dir_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_mip_flag[ ][ ] | (Abs( Log2(cbWidth) − Log2(cbHeight) ) > 1) ? 3 : ( 0,1,2 (clause 9.5.4.2.2) ) 0 | na | na | na | na | na |
| intra_mip_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_mip_mpm_idx[ ][ ] | bypass | bypass | na | na | na | na |
| intra_mip_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | na | na | na | na |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | na |
| intra_subpartitions_split_flag | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_not_planar_flag[ ][ ] | intra_subpartitions_mode_flag | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | bypass | bypass | bypass | bypass | na | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |
| general_merge_flag[ ][ ] | 0 | na | na | na | na | na |
| regular_merge_flag[ ][ ] | cu_skip_flag[ ][ ] ? 0 : 1 | na | na | na | na | na |
| mmvd_merge_flag[ ][ ] | 0 | na | na | na | na | na |
| mmvd_cand_flag[ ][ ] | 0 | na | na | na | na | na |
| mmvd_distance_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | bypass |
| mmvd_direction_idx[ ][ ] | bypass | bypass | na | na | na | na |
| merge_subblock_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| merge_subblock_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| ciip_flag[ ][ ] | 0 | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_split_dir[ ][ ] | bypass | na | na | na | na | na |
| merge_triangle_idx0[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx1[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) > 12 ? 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ) : 4 | 4 | na | na | na | na |
| inter_affine_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| cu_affine_type_flag[ ][ ] | 0 | na | na | na | na | na |
| sym_mvd_flag[ ][ ] | 0 | na | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| amvr_flag[ ][ ] | inter_affine_flag[ ][ ] ? 1 : 0 | na | na | na | na | na |
| amvr_precision_flag[ ][ ] | 0 | na | na | na | na | na |
| bcw_idx[ ][ ] NoBackwardPredFlag = = 0 | 0 | bypass | na | na | na | na |
| bcw_idx[ ][ ] NoBackwardPredFlag = = 1 | 0 | bypass | bypass | bypass | na | na |
| cu_cbf | 0 | na | na | na | na | na |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | na | na | na | na | na |
| cu_sbt_quad_flag | 0 | na | na | na | na | na |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | na | na | na | na | na |
| cu_sbt_pos_flag | 0 | na | na | na | na | na |
| lfnst_idx[ ][ ] | ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| tu_cbf_luma[ ][ ][ ] | intra_bdpcm_flag[ ][ ] ? 4 : 0,1,2,3 (clause 9.5.4.2.5) | na | na | na | na | na |
| tu_cbf_cb[ ][ ][ ] | trDepth = = 0 ? 0 : 1 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | tu_cbf_cb[ ][ ][ ] | na | na | na | na | na |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| transform_skip_flag[ ][ ] | 0 | na | na | na | na | na |
| tu_mts_idx[ ][ ] | cqtDepth | 6 | 7 | 8 | na | na |
| tu_joint_cbcr_residual[ ][ ] | 0 | na | na | na | na | na |
| last_sig_coeff_x_prefix | 0..22 (clause 9.5.4.2.4) |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..22 (clause 9.5.4.2.4) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | ( MaxCcbs > 0) ? ( 0..7 (clause 9.5.4.2.6) ) : bypass | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | ( MaxCcbs > 0) ? ( 0..93 (clause 9.5.4.2.8) ) : bypass | na | na | na | na | na |
| par_level_flag[ ] | ( MaxCcbs > 0) ? ( 0.332 (clause 9.5.4.2.9) ) : bypass | na | na | na | na | na |
| abs_level_gtx_flag[ ] | ( MaxCcbs > 0) ? ( 0..70 (clause 9.5.4.2.9) ) : bypass | na | na | na | na | na |
| abs_remainder[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| dec_abs_level[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] transform_skip_flag[ x0 ][ y0 ] = = 0 | bypass | na | na | na | na | na |
| coeff_sign_flag[ ] transform_skip_flag[ x0 ][ y0 ] = = 1 | ( MaxCcbs > 0) ? ( intra_bdpcm_flag ? 1 : 0 ) : bypass | na | na | na | na | na |

[Ed. (BB): In VTM, ctxInc of tu_cbf_cb is set to trDepth but trDepth can only takes values 0 and 1 because MaxTbSizeY is always equal to 64 and the maximum CTU size is 128. Thus, the depth resulting from implicit split cannot exceed 1.]

##### Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, the width and the height of the current coding block in luma samples cbWidth and cbHeight, and the variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, allowSplitTtHor, and allowSplitQt as derived in the coding tree semantics in clause 7.4.8.4.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the variable availableA specifying the availability of the coding block located directly above the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

The assignment of ctxInc is specified as follows with condL and condA specified in Table 9-18:

- For the syntax elements alf_ctb_flag[ x0 ][ y0 ][ cIdx ], split_qt_flag, split_cu_flag, cu_skip_flag[ x0 ][ y0 ], pred_mode_ibc_flag[ x0 ][ y0 ], inter_affine_flag[ x0 ][ y0 ] and merge_subblock_flag[ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3	(9-28)

- For the syntax element pred_mode_flag[ x0 ][ y0 ]:

ctxInc = ( condL  &&  availableL ) | | ( condA  &&  availableA )	(9-29)

Table 9-18 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] | cIdx |
| split_qt_flag | cqtDepth[ xNbL ][ yNbL ] > cqtDepth | cqtDepth[ xNbA ][ yNbA ] > cqtDepth | ( cqtDepth < 2) ? 0 : 1 |
| split_cu_flag | CbHeight[ xNbL ][ yNbL ] < cbHeight | CbWidth[ xNbA ][ yNbA ] < cbWidth | ( allowSplitBtVer + allowSplitBtHor + allowSplitTtVer + allowSplitTtHor + 2 * allowSplitQt − 1 ) / 3 |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | 0 |
| pred_mode_flag[ x0 ][ y0 ] | CuPredMode[ xNbL ][ yNbL ] = = MODE_INTRA | CuPredMode[ xNbA ][ yNbA ] = = MODE_INTRA | 0 |
| pred_mode_ibc_flag[ x0 ][ y0 ] | CuPredMode[ xNbL ][ yNbL ] = = MODE_IBC | CuPredMode[ xNbA ][ yNbA ] = = MODE_IBC | 0 |
| intra_mip_flag[ x0 ][ y0 ] | intra_mip_flag[ xNbL ][ yNbL ] | intra_mip_flag[ xNbA ][ yNbA ] | 0 |
| merge_subblock_flag[ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |
| inter_affine_flag [ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |