| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0599 |
| --- | --- |

| Title: | CE3-related: Simplification on MIP context model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Hong-Jheng Jhu Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com jhuhong-jheng@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce context model from 3 to 1 and remove selection logic when signaling 1-bit MIP flag. In this proposal, two tests and results are provided as following:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

# Introduction

In VTM-5.0 [1], Matrix-based Intra Prediction (MIP) is adopted in Geneva on March 2019 [2]. The context selecting index ctxInc of MIP flag is initialized as 0 and currently defined as following:

- If left cu exists and is MIP mode, ctxInc is set equal to 1,

- Otherwise, if above coding unit exists and is MIP mode, ctxInc is incremented by 1,

- Otherwise, if Abs( Log2(cbWidth) – Log2(cbHeight) ) > 1, ctxInc is set equal to 3

According to above condition, the procedure of selecting context model is dependent on the coding mode of neighboring block and the aspect ratio of current coding block as highlighted. The third condition of selecting context model is a by-product of encoder speed-up. However, as the test 1 and test 2 shown, this encoder speed-up is not significant comparing to coding efficiency and introduces additional context model. Therefore, it is proposed to reduce context model from 3 to 1 and remove the selection logic when signaling 1-bit MIP flag.

# Proposed Implementation

In this proposal, it is proposed to remove 2 context model and preserve only 1 for signaling MIP flag. More than that, since the trade-off of encoder speed-up is not significant, it is proposed to lessen the constraint in encoder as well as its corresponding selection logic when selecting context model of MIP flag. In encoder side, the modification is following:

From

testMip = mipAllowed && mipModesAvailable(pu.Y()) && !(fastMip && (cu.lwidth() > 2 * cu.lheight() || cu.lheight() > 2 * cu.lwidth()));

To

testMip = mipAllowed && mipModesAvailable(pu.Y());

The highlight part is encoder speed-up and introduces third context model.

# Results

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementations are shown below against the VTM5.0 under CTC [3]. Table 1 shows the BD-rate result when our implementation is applied on VTM5.0. Table 2 show the BD-rate result if our implementation is applied on JVET-O0084. For more detail in each setting, please refer to the attached spreadsheets.

Table 1. Results of proposed Test 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | -0.02% | 103% | 101% |
| Class A2 | -0.03% | -0.04% | -0.01% | 103% | 99% |
| Class B | -0.03% | -0.07% | 0.00% | 102% | 100% |
| Class C | -0.05% | -0.06% | 0.05% | 103% | 101% |
| Class E | -0.07% | 0.01% | -0.02% | 101% | 99% |
| Overall | -0.04% | -0.04% | 0.00% | 102% | 100% |
| Class D | -0.04% | 0.17% | 0.02% | 101% | 101% |
| Class F | -0.03% | 0.01% | -0.03% | 102% | 100% |

Table 2. Results of proposed Test 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over JVET-O0084 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | 0.06% | 103% | 100% |
| Class A2 | -0.04% | 0.01% | 0.04% | 102% | 100% |
| Class B | -0.03% | -0.03% | 0.02% | 102% | 99% |
| Class C | -0.04% | -0.01% | -0.01% | 102% | 99% |
| Class E | -0.06% | -0.02% | -0.14% | 101% | 101% |
| Overall | -0.04% | -0.02% | 0.00% | 102% | 100% |
| Class D | -0.03% | 0.06% | 0.00% | 103% | 102% |
| Class F | -0.03% | 0.02% | 0.14% | 102% | 100% |

# Conclusion

In this proposal, it is proposed to reduce context model from 3 to 1. As the results shown, the coding performance is improved without significant increasing of encoder run-time as well as removing 2 context model and additional selection logic. Therefore, it is recommended to adopt proposed modification of VVC.

# References

| [1] | "VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | J. Pfaff , B. Stallenberger, M. Schäfer, P. Merkle , P. Helle , T. Hinz, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)," Document JVET-N0217, Geneva, CH, 2019. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," Document JVET-N1010, Geneva, CH, March 2019. |
| [4] | J. Pfaff , T. Hinz , P. Helle , P. Mekle , B. Stallenberger , M. Schäfer, H. Schwarz, D. Marpe and T. Wiegand, "8-bit implementation and simplification of MIP," Document JVET-O0084, Gothenburg, SE, 2019. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).