| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0600 |
| --- | --- |

| Title: | CE8-related: Palette mode excluding small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Xiaoyu Xiu Yi-Wen Chen Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

It is observed that applying palette mode on small CUs introduces insignificant coding gain and brings extra complexity on the small blocks. This contribution proposes to disable palette mode for CU that are smaller or equal to 16 pixels. The proposed method reports negligible BD-rate changes compared to CE8-2.1 and reduces complexity.

Introduction

In CE8 palette mode base software [1], palette mode can apply for CU that are equal or smaller than 64x64 pixels. However, it is observed that applying palette mode on small CUs shows insignificant coding performance change but causes extra complexity on small CUs.

Proposed Implementation

In this contribution, it is proposed to disable palette mode for CU that are smaller or equal to 16 pixels. This modification can reduce the complexity for decoding a small CU.

Results

Test sequences 4:2:0

The software is implemented on top of CE8-2.1 [2] and its results against CE8-2.1 under CTC [4] are shown in Table 1.

Table 1. Performance of the proposed modification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE8-2.1, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.05% | 97% | 97% |
| Class A2 | 0.00% | 0.00% | -0.03% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.04% | 99% | 97% |
| Class C | -0.01% | -0.07% | 0.02% | 100% | 97% |
| Class E | -0.01% | -0.04% | 0.03% | 103% | 103% |
| Overall | 0.00% | -0.03% | 0.00% | 100% | 98% |
| Class D | -0.01% | -0.13% | -0.03% | 104% | 101% |
| Class F | -0.01% | -0.01% | 0.11% | 104% | 103% |
| Class TGM | -0.01% | 0.08% | 0.10% | 101% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE8-2.1, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 101% | 100% |
| Class A2 | -0.01% | 0.24% | -0.04% | 102% | 101% |
| Class B | -0.02% | -0.02% | 0.04% | 99% | 99% |
| Class C | -0.01% | 0.00% | -0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | -0.03% | 100% | 99% |
| Class D | -0.03% | 0.16% | -0.08% | 100% | 100% |
| Class F | -0.03% | -0.03% | 0.00% | 97% | 99% |
| Class TGM | 0.07% | 0.09% | 0.06% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE8-2.1, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.15% | -0.10% | 97% | 96% |
| Class C | -0.03% | -0.02% | -0.14% | 101% | 102% |
| Class E | -0.01% | -0.12% | -0.31% | 100% | 95% |
| Overall | 0.00% | -0.10% | -0.17% | 99% | 98% |
| Class D | 0.05% | -0.13% | 1.06% | 99% | 100% |
| Class F | 0.00% | 0.01% | -0.40% | 98% | 98% |
| Class TGM | 0.09% | 0.15% | 0.13% | 100% | 100% |

Test sequences 4:4:4

The software is implemented on top of CE8.2.1 [2] and its results against CE8.2.1 on YUV4:4:4 sequences are shown as following. Table 3 shows the results with DualITree turned off while Table 4 shows the results with DualITree turned on.

Table 3. Performance of the proposed modification

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE8-2.1, DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | 0.15% |  | 0.23% |  | 0.13% |  | 100% |  | 102% |
| TGM 720p | 0.12% |  | 0.00% |  | -0.03% |  | 100% |  | 104% |
| Animation | -0.01% |  | -0.01% |  | -0.03% |  | 98% |  | 99% |
| Mixed content | 0.01% |  | -0.14% |  | -0.05% |  | 101% |  | 102% |
| Camera-Captured content | 0.00% |  | 0.01% |  | -0.03% |  | 101% |  | 98% |
| Overall | 0.07% |  | 0.03% |  | 0.01% |  | 100% |  | 102% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over CE8-2.1, DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | -0.42% |  | 0.07% |  | -0.06% |  | 99% |  | 100% |
| TGM 720p | -0.04% |  | 0.09% |  | 0.07% |  | 103% |  | 103% |
| Animation | -0.02% |  | 0.01% |  | -0.04% |  | 103% |  | 100% |
| Mixed content | 0.09% |  | -0.06% |  | 0.23% |  | 101% |  | 101% |
| Camera-Captured content | -0.01% |  | -0.11% |  | -0.10% |  | 99% |  | 99% |
| Overall | -0.11% |  | 0.02% |  | 0.03% |  | 101% |  | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over CE8-2.1, DT0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | -0.22% | -0.12% |  | -0.20% |  | 98% |  | 98% |  |
| TGM 720p | -0.14% | -0.06% |  | 0.04% |  | 101% |  | 99% |  |
| Animation | 0.02% | 0.09% |  | -0.09% |  | 100% |  | 104% |  |
| Mixed content | 0.15% | -0.17% |  | -0.24% |  | 106% |  | 102% |  |
| Camera-Captured content | 0.00% | -0.10% |  | 0.07% |  | 101% |  | 99% |  |
| Overall | -0.07% | -0.08% |  | -0.09% |  | 101% |  | 100% |  |

Table 4. Performance of the proposed modification

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE8-2.1, DT1 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | 0.02% |  | 0.10% |  | 0.08% |  | 99% |  | 98% |
| TGM 720p | -0.03% |  | 0.03% |  | 0.02% |  | 105% |  | 104% |
| Animation | -0.01% |  | 0.03% |  | 0.04% |  | 102% |  | 102% |
| Mixed content | -0.01% |  | 0.00% |  | 0.00% |  | 99% |  | 100% |
| Camera-Captured content | -0.01% |  | 0.00% |  | -0.01% |  | 103% |  | 101% |
| Overall | -0.01% |  | 0.04% |  | 0.03% |  | 102% |  | 101% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over CE8-2.1, DT1 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | -0.35% |  | -0.01% |  | -0.16% |  | 101% |  | 102% |
| TGM 720p | -0.12% |  | -0.08% |  | -0.05% |  | 102% |  | 102% |
| Animation | -0.04% |  | -0.02% |  | 0.05% |  | 101% |  | 100% |
| Mixed content | 0.09% |  | 0.16% |  | 0.17% |  | 100% |  | 100% |
| Camera-Captured content | 0.00% |  | 0.05% |  | -0.01% |  | 99% |  | 97% |
| Overall | -0.11% |  | 0.01% |  | -0.02% |  | 101% |  | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over CE8-2.1, DT1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | -0.05% | 0.09% |  | 0.14% |  | 99% |  | 99% |  |
| TGM 720p | 0.08% | 0.12% |  | 0.05% |  | 104% |  | 104% |  |
| Animation | -0.01% | -0.17% |  | -0.03% |  | 96% |  | 95% |  |
| Mixed content | -0.20% | -0.39% |  | 0.21% |  | 99% |  | 99% |  |
| Camera-Captured content | -0.01% | -0.06% |  | -0.06% |  | 101% |  | 98% |  |
| Overall | -0.04% | -0.05% |  | 0.08% |  | 100% |  | 100% |  |

Conclusion

In this proposal, palette mode excluding small blocks are tested. The results show that the test improves the decoding throughput with negligible BD-rate changes compared to CE8-2.1. We recommend to consider adoption of such constraints into palette mode.

References

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, the 14th JVET meeting, Mar. 2019, Geneva, CH.

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang “ CE8-2.1: Palette mode in HEVC”, JVET-O0119, the 15th JVET meeting, July 2019, Gothenburg, SE.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Mar. 2019, Geneva, CH.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).