| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0601 |
| --- | --- |

| Title: | CE4-related: On motion vector population under triangle prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma | Tel: Email: | 1-858-353-9648 {xianglinwang, yiwenchen, xiaoyuxiu, tsung-chuanma}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the current VVC under triangle prediction mode, 4x4 blocks in each triangle partition are populated with the respective uni-prediction motion vector (MV) for that partition. For those blocks sitting on the triangle partition border, a potentially bi-prediction MV is derived from the two uni-prediction MVs and assigned to them. At decoder side, certain logic is needed to classify each individual 4x4 block into one of the three cases and assign a MV to the block accordingly. In this document, a method is proposed to simplify such motion vector population process, with each quarter area of a triangle mode CU filled with a same motion vector. Simulation results show that the proposed simplification method leads to {Y, U, V} BD-rate variations of {0.00%, -0.07%, 0.02%} and {0.02%, 0.23%, 0.05%} for RA and LD configurations, respectively.

# Introduction

According to the current VVC[1], for a CU coded in triangle prediction mode, 4x4 blocks in each triangle partition are populated with the uni-prediction motion vector used for that partition. For those blocks sitting on the triangle partition border, a potentially bi-prediction motion vector is from the two uni-prediction MVs and assigned to them. An example is shown in the following Figure 1. In this figure, those highlighted blocks are populated with the derived MV.

Figure 1 4x4 block motion vector population under triangle prediction mode

However, due to the slant triangle partition direction, at decoder side certain logic is needed to classify each individual 4x4 block into one of the three cases and assign a MV to the block accordingly, which seems unnecessarily complex.

# Proposal

To simplify the process illustrated above, we propose to populate each quarter area of a CU coded in triangle prediction mode with a same motion vector, as exemplified in the following Figure 2.

Figure 2 Proposed 4x4 block motion vector population method for triangle prediction mode

Specifically, for those two quarter areas that are not sitting on the triangle partition border, the corresponding uni-prediction motion vector of that triangle partition is used to populate the 4x4 blocks inside each area. For the two highlighted quarter areas sitting on the partition border, the derived motion vector is used. The derivation scheme of the potentially bi-prediction motion vector is unchanged from that in the current VVC. In special cases where the height or width of a triangle mode CU is 4-pixels, all 4x4 blocks of the CU are populated with the derived bi-prediction motion vector.

# Simulation results

The proposed method was implemented on VTM-5.0 [2] and tested following the common test conditions as specified in [3]. Simulation results are as follows.

Table 1 BD-rate performance of proposed method compared to VTM-5.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.24% | 0.06% | 101% | 101% |
| Class A2 | 0.03% | -0.01% | -0.04% | 100% | 101% |
| Class B | 0.00% | 0.03% | 0.09% | 100% | 101% |
| Class C | -0.01% | -0.11% | -0.07% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | 0.02% | 100% | 100% |
| Class D | -0.01% | -0.02% | 0.09% | 100% | 100% |
| Class F | 0.01% | 0.07% | -0.02% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.23% | 0.13% | 96% | 97% |
| Class C | 0.03% | -0.03% | -0.06% | 100% | 95% |
| Class E | 0.02% | 0.56% | 0.06% | 99% | 96% |
| Overall | 0.02% | 0.23% | 0.05% | 98% | 96% |
| Class D | 0.03% | -0.06% | 0.32% | 99% | 103% |
| Class F | -0.03% | 0.20% | -0.17% | 100% | 96% |

# Proposed text changes

The added texts and removed texts of the proposed methods are highlighted in yellow and strikethrough, respectively

#### 8.5.7.3. Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-814)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-815)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-816)

- The variable sType is derived as follows:

  - If minSb is equal to 1, sType = 2

  - Otherwise,

  - If triangleDir is equal to 0, the following applies:

sType = (xSbIdx >= numSbX/2 && ySbIdx < numSbY/2)? 0: (xSbIdx < numSbX/2 && ySbIdx >= numSbY/2)? 1 : 2

  - Otherwise, if triangleDir is equal to 0, the following applies:

sType = (xSbIdx < numSbX/2 && ySbIdx < numSbY/2)? 0: (xSbIdx >= numSbX/2 && ySbIdx >= numSbY/2)? 1 : 2

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-817)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-818)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-819)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-820)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-821)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-822)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-823)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-824)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-825)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-826)

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).