| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0602 |
| --- | --- |

| Title: | Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to reduces the line buffer of Matrix Intra Prediction (MIP)/Intra Block Copy (IBC) mode flag storage. For MIP mode flag storage, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. The proposed method is compared with both VTM5 JVET-O0084, with simulation results as follows:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are 0.00%, 0.02%, and 0.02% for AI, RA and LD, respectively, without increasing encoding and decoding time.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are 0.00%, 0.00%, and 0.00% for AI, RA and LD, respectively, without increasing encoding and decoding time.

For IBC mode flag storage, the context selection for the CABAC coding of IBC flag is not depending on the IBC flag of above CU when the above CU is not located in the same CTU as current CU. Furthermore, the above neighbor candidate for IBC AMVP/merge/skip mode prediction is not used when the above CU is not in the same CTU as current CU. Compared to CE8 anchors, for Class SCC sequences, The BD-rate results are 0.11%, 0.1%, and 0.05% for AI, RA and LD, respectively, without increasing encoding and decoding time.

Introduction

In VTM5.0 [1], Matrix-based Intra Prediction (MIP) is adopted in Geneva on March 2019 [2][3]. The MIP flag can be 0 and 1 to indicate if current coded block is MIP mode or not. In order to calculate the context index (ctxIdx) of MIP flag, the MIP flags of both the left and above neighboring blocks are used. As a result, the information of MIP flag needs to be stored in line buffer.

There is the same issue for IBC mode flag. The IBC mode flag can be 0 and 1 to indicate if current coded block is IBC mode or not. In order to calculate the context index (ctxIdx) of IBC mode flag, the IBC mode flag of both the left and above neighboring blocks are used. Furthermore, in order to generate IBC AMVP/skip/merge mode prediction, the block vector from above neighbor (if IBC coded) may be used as candidate. As a result, the information of IBC mode flag needs to be stored in line buffer.

Proposed Implementation

In this contribution, it is proposed to reduce the line buffer of MIP/IBC mode storage. For MIP mode, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. In this case, the value of MIP flag of the above CU can be assumed as 0. As a result, there is no need to store MBIP flag information in line buffer. The memory reduction of 4K sequences are 4K bits if one mode flag is 1 bit.

For IBC mode, the context selection for the CABAC coding of IBC mode flag is not depending on the IBC mode flag of above CU when the above CU is not in the same CTU as current CU. In this case, the value of IBC mode flag of the above CU can be assumed as 0. And the above neighbor candidate is not used for IBC skip/merge mode prediction when the above CU is not in the same CTU as current CU. Those modifications can reduce the line buffer, since there is no need to store IBC mode flag information in line buffer. The memory reduction of 4K sequences are 4K bits if one mode flag is 1 bit.

Results

Remove MIP mode storage from line buffer

The proposed modification is integrated into VTM-5.0 [1] and its results and its results against VTM-5.0 under CTC [4] are shown in Table 1. Table 1 shows the BD-rate result when our implementation is applied on VTM5.0. The proposed modification is integrated into JVET-O0084 and its results and its results against JVET-O0084 under CTC [4] are shown in Table 2.

Table 1. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 103% | 102% |
| Class A2 | 0.01% | -0.04% | -0.05% | 100% | 103% |
| Class B | 0.01% | -0.03% | 0.03% | 99% | 100% |
| Class C | 0.00% | -0.05% | 0.01% | 92% | 93% |
| Class E | 0.00% | -0.05% | -0.03% | 100% | 99% |
| Overall | 0.00% | -0.03% | 0.00% | 98% | 99% |
| Class D | 0.01% | 0.00% | -0.04% | 100% | 94% |
| Class F | -0.02% | -0.06% | 0.02% | 100% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.20% | 0.06% | 102% | 101% |
| Class A2 | 0.03% | 0.06% | 0.10% | 102% | 104% |
| Class B | 0.02% | -0.04% | -0.11% | 100% | 102% |
| Class C | -0.01% | -0.10% | -0.01% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.07% | -0.01% | 100% | 101% |
| Class D | 0.04% | 0.10% | 0.37% | 99% | 97% |
| Class F | 0.03% | 0.09% | 0.06% | 100% | 99% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.30% | 0.06% | 101% | 99% |
| Class C | 0.01% | 0.07% | -0.25% | 100% | 104% |
| Class E | 0.05% | 0.08% | -0.40% | 102% | 103% |
| Overall | 0.02% | 0.17% | -0.16% | 101% | 102% |
| Class D | -0.02% | 0.39% | 0.26% | 98% | 98% |
| Class F | 0.03% | -0.22% | -0.57% | 100% | 100% |

Table 2. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.10% | 103% | 101% |
| Class A2 | -0.01% | 0.02% | 0.01% | 102% | 102% |
| Class B | 0.01% | 0.02% | -0.01% | 101% | 104% |
| Class C | 0.00% | 0.01% | 0.01% | 101% | 103% |
| Class E | 0.00% | -0.05% | -0.01% | 97% | 98% |
| Overall | 0.00% | 0.01% | 0.02% | 101% | 102% |
| Class D | 0.01% | 0.01% | -0.02% | 97% | 100% |
| Class F | -0.01% | 0.02% | 0.00% | 102% | 101% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.07% | 102% | 103% |
| Class A2 | 0.02% | -0.07% | -0.04% | 102% | 102% |
| Class B | 0.00% | 0.03% | 0.04% | 101% | 103% |
| Class C | -0.01% | -0.03% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.04% | 101% | 102% |
| Class D | 0.01% | 0.07% | 0.16% | 97% | 97% |
| Class F | 0.03% | 0.02% | 0.04% | 99% | 101% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.11% | 0.09% | 101% | 100% |
| Class C | 0.00% | 0.26% | 0.14% | 98% | 101% |
| Class E | -0.01% | -0.34% | 0.38% | 101% | 103% |
| Overall | 0.00% | 0.05% | 0.18% | 100% | 101% |
| Class D | 0.10% | 0.25% | 0.06% | 101% | 104% |
| Class F | -0.05% | 0.12% | 0.63% | 104% | 104% |

Remove IBC mode storage from line buffer

The proposed methods were implemented based on the VTM-5.0 [1] reference software. The test conditions and evaluation criterion that are used in CE8 [5] are applied. Its results against IBC mode are shown in Table 3.

Table 3. Performance of the proposed modification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 97% | 101% |
| Class A2 | 0.00% | -0.05% | 0.01% | 100% | 98% |
| Class B | 0.01% | 0.05% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class E | 0.01% | -0.01% | 0.01% | 101% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.01% | 100% | 102% |
| Class F | 0.02% | -0.02% | 0.00% | 99% | 102% |
| Class TGM | 0.11% | 0.11% | 0.08% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.09% | 102% | 102% |
| Class A2 | 0.02% | 0.36% | -0.05% | 100% | 98% |
| Class B | -0.02% | 0.04% | -0.02% | 101% | 101% |
| Class C | 0.02% | -0.01% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.00% | 101% | 101% |
| Class D | 0.00% | -0.04% | -0.01% | 103% | 103% |
| Class F | -0.01% | -0.10% | 0.01% | 103% | 102% |
| Class TGM | 0.10% | 0.08% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.17% | 0.34% | 100% | 98% |
| Class C | 0.09% | 0.17% | 0.35% | 102% | 104% |
| Class E | 0.06% | 0.46% | 0.27% | 102% | 98% |
| Overall | 0.05% | 0.10% | 0.33% | 101% | 100% |
| Class D | 0.06% | -0.02% | 0.01% | 102% | 104% |
| Class F | -0.11% | 0.30% | -0.46% | 100% | 101% |
| Class TGM | 0.05% | 0.05% | 0.09% | 102% | 103% |

Conclusion

In this proposal, we propose to reduce the line buffer of MIP/IBC mode storage. The results show that the tests reduce line buffer without major penalty in terms of coding efficiency. We recommend to consider adoption of the proposed method into VVC.

References

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle , P. Helle , T. Hinz, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)," Document JVET-N0217, Geneva, CH, 2019.

- J. Pfaff, T. Hinz , P. Helle , P. Mekle , B. Stallenberger , M. Schäfer, H. Schwarz, D. Marpe and T. Wiegand, "8-bit implementation and simplification of MIP," Document JVET-O0084, Gothenburg, SE, 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Mar. 2019, Geneva, CH.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, the 14th JVET meeting, Mar. 2019, Geneva, CH.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).