| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0603 |
| --- | --- |

#### 8.5.6.6 Weighted sample prediction process for combined merge and intra prediction

The weight w is derived as follows:

- If isIntraCodedNeighbourA and isIntraCodedNeighbourB are both equal to TRUE, w is set equal to 3.

- Otherwise, if isIntraCodedNeighbourA and isIntraCodedNeighbourB are both equal to to FALSE, w is set equal to 1.

- Otherwise, w is set equal to 2.

When cIdx is equal to 0 and slice_lmcs_enabled_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ] >> Log2( OrgCW ) 
predSamplesInter [ x ][ y ] = Clip1Y( LmcsPivot[ idxY ] + 	(8-819)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) + 
	( 1 << 10 ) ) >> 11 )

The prediction samples predSamplesComb[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are derived as follows:

predSamplesComb[ x ][ y ] = ( w * predSamplesIntra[ x ][ y ] + 	(8-820)
 	( 4 − w ) * predSamplesInter[ x ][ y ] + 2) >> 2

#### 8.7.5.2 Picture reconstruction with mapping process for luma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the block width,

- a variable nCurrSh specifying the block height,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the luma predicted samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the luma residual samples of the current block.

Outputs of this process is:

- a modified array of luma predicted samples predSamples,

- a reconstructed luma picture sample array recSamples.

The (nCurrSw)x(nCurrSh) array of mapped predicted luma samples PredMapSamples, which is used in the picture reconstruction with luma dependent chroma residual scaling process for chroma samples as specified in clause 8.7.5.3, is derived as follows:

- The variables minPredSample and maxPredSample are derived as

minPredSample = OrgCW * lmcs_min_bin_idx

maxPredSample = OrgCW * (LmcsMaxBinIdx + 1)

- The luma predicted sample array predSamples[ i ][ j ] is modified for i = 0..nCurrSw-1, j=0..nCurrSh-1:

predSamples[ i ][ j ] = Clip3(minPredSample, maxPredSample, predSamples[ i ][ j ])

- PredMapSamples[ i ][ j ] is set equal to predSamples[ i ][ j ] for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one of the following conditions is true, PredMapSamples[ i ][ j ] is set equal to predSamples[ i ][ j ] for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- CuPredMode[ xCurr ][ yCurr ] is equal to MODE_INTRA.

- CuPredMode[ xCurr ][ yCurr ] is equal to MODE_IBC.

- CuPredMode[ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 1.

- Otherwise (CuPredMode[ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
PredMapSamples[ i ][ j ] =  LmcsPivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + ( 1 << 10 ) ) >> 11 	(8-1009)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

- The luma residual sample array resSamples[ i ][ j ] for i = 0..nCurrSw-1, j=0..nCurrSh-1:

- The variable varScale is dervied as follows

- The variable idxY is derived as predSamples[ i ][ j ]>>Log2(OrgCW).

- If resSamples[ i ][ j ] is no smaller than 0,

varScale = (invScaleCoeff[idxY] + invScaleCoeff[idxY+1] + 1)>>1

- Otherwise,

varScale = (invScaleCoeff[idxY] + invScaleCoeff[idxY-1] + 1)>>1

- The residual sample is modified as

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepthY ), ( 1 << BitDepthY ) − 1, resSamples[ i ][ j ] )

resSamples[ i ][ j ] = Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 )

The reconstructed luma picture sample recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1Y( PredMapSamples[ i ][ j ]+ resSamples[ i ][ j ] ] )	(8-1010)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

## 8.8 In-loop Filter Process

### 8.8.1 General

The picture inverse mapping process with luma samples and the three in-loop filters, namely deblocking filter, sample adaptive offset and adaptive loop filter, are applied as specified by the following ordered steps:

- When sps_lmcs_enabled_flag is equal to 1, the following applies:

- The picture inverse mapping process for luma samples as specified in clause 8.8.2.1 is invoked with the reconstructed luma sample array SL as inputs, and the modified reconstructed luma sample array S′L after picture inverse mapping process for luma samples as outputs.

- The array S′L is assigned to the array SL (which represent the decoded picture).

- For the deblocking filter, the following applies:

- The deblocking filter process as specified in clause 8.8.3.1 is invoked with the reconstructed picture sample array SL and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr as inputs, and the modified reconstructed picture sample array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr after deblocking as outputs.

- The array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr are assigned to the array SL and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr (which represent the decoded picture), respectively.

### 8.8.2 Picture inverse mapping process for luma samples

#### 8.8.2.1 General

Input to this process is a reconstructed picture luma sample array SL.

The output to this process is a modified reconstructed picture luma sample array SL.

The inverse mapping process for a luma sample SL[ x ][ y ] with x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1 is invoked as specified in clause 8.8.2.2 with the variable lumaSample set equal to SL[ x ][ y ] as the input and the output is assigned to the luma sample SL[ x ][ y ].

#### 8.8.2.2 Inverse mapping process for a luma sample

Input to this process is a luma sample lumaSample.

Output of this process is a modified luma sample invLumaSample .

The value of invLumaSample is derived as follows:

- If slice_lmcs_enabled_flag of the slice that contains the luma sample lumaSample is equal to 1, the following ordered steps apply:

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with lumaSample as the input and idxYInv as the output.

- The variable invSample is derived as follows:

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * 	(8-1016)
	( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

- The inverse mapped luma sample invLumaSample is derived as follows:

invLumaSample = Clip1Y( invSample )	(8-1017)

- Otherwise, invLumaSample is set equal to lumaSample.