| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0603-r1 |
| --- | --- |

| Title: | CE2-related: Prediction dependent luma residual scaling for adaptive in-loop re-shaper |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, S. Ye, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, shuimingye, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# In this update version, the LD simulation results and the proposed specification changes are provided.

# Abstract

The luma mapping with chroma scaling (LMCS) (previously referred to as adaptive in-loop re-shaper) was adopted into the VVC at Marrakech meeting. In the VVC working draft 5, the LMCS is applied to luma and chroma components in different ways: 1) the luma reshape is performed in sample domain using adaptive piecewise linear model; 2) chroma residuals are scaled based on the average of the corresponding luma prediction samples. To achieve one more unified design, this contribution proposes to extend the existing chroma residual scaling method to luma component in which the scaling factor of luma residuals are calculated based on the corresponding luma prediction samples. It is asserted that after applying the proposed method all the in-loop mapping of the predicted and reconstructed luma samples in the existing LMCS design can be removed. Compared to the VTM-5.0 anchors, the proposed method reportedly leads to BD-rate variations {0.04%, 0.06%, 0.12%} and {-0.05%, -0.12%, -0.13%} and {-0.06%, 0.01%, 0.02%} for AI ,RA and LD configurations, respectively.

# Introduction

Luma mapping with chroma scaling (LMCS) was adopted into the VVC as one new coding tool that is applied before loop filters [1]. The LMCS is composed of two main modules [2]: 1) in-loop mapping of the luma component based on adaptive piecewise linear models; 2) luma-dependent chroma residual scaling. Figure 1 illustrates the decoding architecture of luma component when the LMCS is applied. In Figure 1, the shaded blocks indicate the decoding modules that are carried out in the mapped domain; the unshaded blocks indicate the decoding modules that are performed in the original domain; the yellow blocks represent the new LMCS operational modules, including forward and inverse mapping of luma samples.

Figure 1 Decoder architecture with the LMCS

For the chroma residual scaling, the scaling factor of one coding unit (CU) is determined based on the average of the corresponding luma prediction samples. Denote avg’Y as the average of the luma prediction samples in the mapped domain. The scaling parameter CScaleInv is computed by:

- Find the index Y1dx of the piecewise linear model to which avg’Y belongs to in the mapped domain.

- CScaleInv = cScaleInv[Y1dx], where cScaleInv[i], i = 0 … 15, is a pre-computed 16-piece look-up table (LUT).

As shown in Figure 1, the prediction and reconstruction luma samples in both the original domain and the mapped domain are used at various decoding steps. For example, for inter CUs, because the luma reconstruction is done in the mapped domain, luma prediction samples that are generated in the original domain needs to be converted into the mapped domain before they are added together with the luma residuals. On the other hand, for both intra and inter CUs, the inverse mapping is needed to convert the reconstructed luma samples from the mapped domain to the original domain before being stored into decoded picture buffer (DPB). Such design not only increases computational complexity due to additional forward/inverse mapping operations but also requires more memory to store multiple versions of reconstructed luma samples. For instance, in the existing LMCS, intra prediction and deblocking filter use the reconstructed luma samples in the original domain the mapped domain, respectively.

# Proposal

In this contribution, one adaptive luma residual scaling method is proposed to reduce the complexity of the LMCS while achieving one more unified design for luma and chroma components. Specifically, similar to the chroma residual scaling, one luma residual scaling scheme is introduced in the proposed method to replace the luma sample reshaping in the existing LMCS design. Figure 2 depicts the modified decoder architecture for luma component when the proposed method is applied. As shown in Figure 2, except for the residual decoding modules (e.g., the inverse quantization and the inverse transform), all the other decoding modules (including intra/inter prediction, reconstruction and all in-loop filters) are operating in the original sample domain. Ash shown in Figure 2, the forward and inverse luma sample mapping operations can be removed in the proposed method. Correspondingly, the two luma mapping LUTs FwdMap[] and InvMap[] (which take up around 2,560 Bytes) in the VTM-5.0 [3] are not needed anymore. Further, compared to the existing LMCS design in Figure 1, the proposed method only needs to maintain the reconstructed luma samples in the original sample domain. This can efficiently reduce the storage size to keep the reconstructed samples for intra prediction and deblocking filters.

Figure 2 Decoder architecture with the proposed luma residual scaling

Similar to the chroma residual scaling, the scaling factor of luma residuals are calculated based on the the corresponding collocated luma prediction samples. To improve the accuracy of the residual scaling, instead of deriving the scaling factor merely from the segment of the piecewise linear model where one luma prediction sample belongs to, the scaling factor is derived as the average of two neighboring segments. Specifically, given one luma prediction sample PredY and its associated residual Yres', the scaling factor that is applied to scale Yres' is calculated as follows.

- Find the index IdxY of the piecewise linear model to which PredY belongs to in the original domain.

- When Yres' is no smaller than 0, the luma residual scaling factor is calculated as

YScale=scaleFwdIdxY+scaleFwdIdxY+1≫1

- Otherwise (i.e., Yres' is smaller than 0), the luma residual scaling factor is calculated as

YScale=scaleFwdIdxY+scaleFwdIdxY-1≫1

where scaleFwd[i], i = 0 … 15 is one pre-computed 16-piece LUT which can be directly derived from the existing chroma scaling factor LUT, i.e., cScaleInv[i]. Given YScale, the proposed luma residual scaling is applied as follows:

At encoder side: Yres'=YRes≪SCALE_FP_PREC/YScale

At decoder side: YRes=Yres'*YScale≫SCALE_FP_PREC

where SCALE_FP_PREC is the precision of scaling factor that is equal to 11. The LUT scaleFwd[] contains 16 entries and each entry is in the precision of 32-bit. Therefore, the total storage size of scaleFwd[] is 16 * 32 = 512 bits = 64 bytes. Additionally, in order to reduce the LUT storage size, scaleFwd[] is used for both the luma residual scaling and the chroma residual scaling.

Except the proposed luma residual scaling process, all the other LMCS normative designs (e.g., syntax elements and binarization of signaling LMCS piecewise linear model and slice-level enabling/disabling of the MCS) and the encoder non-normative designs (e.g., the LMCS parameter estimation and the rate-distortion optimization) are keep unchanged in this contribution.

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [3] and tested using the common test conditions (CTCs) as specified in [4].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.26% | 0.40% | 111% | 102% |
| Class A2 | 0.12% | 0.09% | 0.11% | 104% | 101% |
| Class B | 0.04% | 0.05% | 0.13% | 107% | 102% |
| Class C | -0.02% | -0.04% | -0.01% | 105% | 102% |
| Class E | 0.04% | -0.01% | 0.00% | 105% | 102% |
| Overall | 0.04% | 0.06% | 0.12% | 106% | 102% |
| Class D | -0.03% | 0.16% | -0.09% | 102% | 105% |
| Class F | 0.18% | 0.08% | 0.02% | 106% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.37% | -0.24% | 103% | 101% |
| Class A2 | -0.05% | 0.11% | 0.03% | 101% | 96% |
| Class B | 0.00% | -0.03% | -0.12% | 100% | 100% |
| Class C | -0.06% | -0.21% | -0.18% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.12% | -0.13% | 101% | 100% |
| Class D | -0.01% | 0.04% | 0.18% | 100% | 101% |
| Class F | 0.06% | 0.08% | 0.01% | 104% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.05% | -0.27% | 100% | 98% |
| Class C | -0.02% | -0.03% | -0.43% | 99% | 101% |
| Class E | -0.11% | -0.01% | 1.09% | 99% | 99% |
| Overall | -0.06% | 0.01% | 0.02% | 100% | 99% |
| Class D | -0.03% | 0.04% | 0.76% | 98% | 97% |
| Class F | -0.08% | -0.13% | -0.06% | 99% | 100% |

# References

T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team (JVET), JVET-M0427, Marrakech, MA, Jan 2019.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).