| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0604 |
| --- | --- |

| Title: | CE4-related: Simplification on block location validation in SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | +1-858-705-2600 yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, to make sure the temporal mv used by SbTMVP is derived from a valid area (i.e. one CTU plus one column on the right of the CTU in the collocated picture), for every 8x8 sub-CU a clipping operation is applied to clip the position of its corresponding block so that it is confined in the valid area. Since the sub-CU is 8x8, it requires 256 clipping operations for a 128x128 CU, which introduces non-trivial complexity. To alleviate such a burden, it is proposed to check only the corresponding blocks of the top-left and the bottom-right sub-CUs to see whether the two corresponding blocks are within the valid area. If either one is located outside the valid area, zero vector is used as motion shift; otherwise (both corresponding blocks are located within the valid area), the derived motion shift is used. With the proposed modifications, no position clipping operation is required while all the corresponding blocks of the sub-CUs are guaranteed to be located within the valid area. Simulation results show that the proposed modification incurs 0.00% and 0.01 BD-rate increase under RA and LDB.

# Introduction

In current VVC, only the motion field within a collocated CTU plus one column could be used for SbTMVP derivation. With this constraint, a clipping process is applied to clip the position of the corresponding 8x8 block to be located within the valid area (i.e. CTU plus one column). The position ( xColCb, yColCb ) of the corresponding 8x8 block is derived by clipping the original position of the corresponding 8x8 block to be located within the valid area using the below equation which is used in current VVC working draft.

xColCb = Clip3( xCtb,Min( CurPicWidthInSamplesY − 1, xCtb + CTUWidthInSamplesY + 7 ), ColCtrCb + MotionShiftX ) )

yColCb = Clip3( yCtb,Min( CurPicHeightInSamplesY − 1, yCtb + CTUHeightInSamplesY − 1 ), yColCtrCb + MotionShiftY )

There are 256 sub-CUs in a 128x128 CU and they require 256 clipping processes which are non-trivial. Therefore, in this proposal, a method is proposed to remove the number of clipping processes.

# Proposed method

It is proposed that whenever there is any corresponding block of a sub-CU located outside the valid area in the collocated picture, the zero vector is used as motion shift to derive the SbTMVP. By doing so, the corresponding blocks of all the sub-CUs of current CU are guaranteed to be located within the valid area. Therefore, no position clipping process is required for each sub-CU. Specifically, the corresponding block of the top-left 8x8 sub-CU and the corresponding block of the bottom-right 8x8 sub-CU are checked to see whether the two corresponding blocks are within the valid area. If either one is located outside the valid area, zero vector is used as motion shift; otherwise (both corresponding blocks are located within the valid area), the derived motion shift is used for SbTMVP. With the modification, the clipping processes applied for each sub-CU could be removed. For a 128x128 CU, 256 clipping processes are removed and the position compares of two sub-CU are used instead. And it is noted that the position compares of the two sub-CUs requires only up to 8 compares which is comparable to single clipping operations.

# Simulation results

The table below shows the BD-rate performance and processing time of the method proposed. The simulations are conducted following the common test conditions [1] on top of reference software VTM-5.0 [2].

Table 1. Performance of the second modification

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | 0.01% | 99% | 100% |
| Class A2 | -0.01% | 0.01% | 0.00% | 97% | 98% |
| Class B | 0.01% | 0.03% | -0.07% | 97% | 98% |
| Class C | -0.01% | 0.03% | 0.02% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 98% | 98% |
| Class D | 0.01% | 0.01% | 0.05% | 99% | 97% |
| Class F | -0.01% | -0.01% | -0.03% | 99% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.40% | 0.16% | 101% | 101% |
| Class C | 0.04% | -0.04% | -0.22% | 100% | 103% |
| Class E | -0.02% | 0.29% | -0.17% | 96% | 97% |
| Overall | 0.01% | 0.23% | -0.05% | 100% | 101% |
| Class D | 0.00% | -0.07% | 0.18% | 97% | 99% |
| Class F | 0.01% | 0.03% | -0.21% | 99% | 100% |

# Conclusions

In this proposal, one modification is proposed to remove the position clipping applied to each sub-CU. The proposed modification introduces negligible coding performance changes while up to 256 clipping processes are removed for a 128x128 CU. It is recommended to consider adoption of the proposed simplification into VVC for SbTMVP.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).