| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0605 |
| --- | --- |

| Title: | CE4-related: On exclusively applying LIC and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In core experiment (CE)4, local illumination compensation (LIC) and prediction refinement with optical flow (PROF) are two separate coding tools that are tested in CE4 test 3.1 and CE4 test 2.1. According to the current LIC and PROF designs, both two coding tools can be applied to affine coding units (CUs). However, simulation results show that although the coding gains of the LIC and the PROF are additive, jointly enabling the two tools on a same CU offers little help on coding performance.

To reduce implementation complexity, this contribution proposes to exclusively apply the LIC and the PROF for one affine CU. Simulations results reportedly show that such a simplification incurs {0.05%, 0.03%, -0.03%} and {0.00%, -0.07%, 0.01%} BD-rate impact for RA and LD configurations, respectively. Additionally, when comparing to the VTM-5.0 anchors, the overall BD-rate gains of the proposed method are {0.91%, 0.45%, 0.54%} for the RA and {0.68%, 0.28%, 0.60%} for the LD.

# Introduction

In [1], local illumination compensation (LIC) is proposed to compensate the local illumination changes in-between temporal neighboring pictures. The LIC is applied to both luma and chroma samples with the following main design features:

- Intra neighbor samples are not used in LIC parameter derivation and are replaced by the reference neighbor samples

- Disable for combined inter/intra prediction (CIIP) and intra block copy (IBC) blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- It is not applied to bi-prediction

- LIC is applied on 16x16 processing units, where LIC parameters are estimated for the first top left 16x16 unit and are used for other units within the coding unit (CU).

- LIC is enabled for affine mode

The LIC is now studied in core experiment (CE)4 test 3.1 [2].

In [3], prediction refinement with optical flow (PROF) is proposed to refine the subblock based affine motion compensation using the optical flow model. Specifically, after the sub-block based affine motion compensation is performed, luma prediction samples are refined by adding a delta derived by the optical flow equation. The PROF is now studied in CE4 test 2.1[2].

# Proposal

As described in the above, both the LIC and the PROF can be applied to affine CUs. However, because the LIC and the PROF were developed separately, the interaction between the two coding tools was not well studied. In this contribution, one solution is proposed to handle the interaction by exclusively applying the PROF mode and the LIC mode for one affine CU. Specifically, by the proposed solution, the decision on whether to apply the PROF for one affine CU is dependent on the value of the LIC flag. When the flag is equal to one, only the LIC is applied by adjusting the prediction samples of the CU block based on the LIC weight and offset. Otherwise (i.e., the LIC flag is equal to zero), the PROF is applied to refine the prediction samples of each subblock based on optical flow model.

# Simulation results

The proposed method was implemented based on the CE4-3.1a [1] and CE4-2.1a reference software [3], and tested following the CTCs as specified in [4].

Table 1 The LIC BD-rate performance of CE4-3.1a, compared to VTM-5.0 anchors

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.55% | -0.63% | 123% | 101% |
| Class A2 | -0.39% | -0.37% | -0.39% | 114% | 102% |
| Class B | -0.46% | -0.21% | -0.52% | 117% | 102% |
| Class C | -0.36% | -0.49% | -0.46% | 120% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.39% | -0.50% | 118% | 102% |
| Class D | -0.13% | -0.25% | -0.06% | 116% | 104% |
| Class F | -0.67% | -0.35% | -0.46% | 113% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.03% | -0.34% | 124% | 99% |
| Class C | -0.42% | -0.15% | -0.35% | 123% | 95% |
| Class E | -0.07% | 0.35% | -0.18% | 103% | 101% |
| Overall | -0.37% | 0.03% | -0.30% | 118% | 98% |
| Class D | -0.16% | -0.06% | 0.28% | 117% | 104% |
| Class F | -1.77% | -1.52% | -1.48% | 116% | 97% |

Table 2 The PROF BD-rate performance of CE4-2.1a, compared to VTM-5.0 anchors

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | 0.11% | 102% | 102% |
| Class A2 | -0.85% | 0.27% | 0.16% | 103% | 104% |
| Class B | -0.61% | -0.28% | -0.19% | 102% | 102% |
| Class C | -0.31% | -0.07% | -0.21% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -0.09% | -0.07% | 102% | 102% |
| Class D | -0.54% | -0.13% | 0.04% | 102% | 102% |
| Class F | -0.86% | -0.45% | -0.47% | 102% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.05% | -0.28% | 103% | 103% |
| Class C | -0.23% | 0.05% | -0.15% | 102% | 100% |
| Class E | -0.30% | -0.14% | -0.80% | 100% | 100% |
| Overall | -0.32% | -0.04% | -0.37% | 102% | 101% |
| Class D | -0.38% | -0.35% | 1.14% | 100% | 104% |
| Class F | -1.33% | -1.34% | -0.97% | 102% | 103% |

Table 3 shows the BD-rate performance of jointly enabling the LIC and the PROF for one affine CU. Specifically, in Table 3, the LIC is applied after the PROF to generate the prediction samples of one affine CU.

Table 3 The BD-rate performance of jointly applying the LIC and the PROF, compared to VTM-5.0 anchors

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | -0.64% | -0.56% | 124% | 100% |
| Class A2 | -1.27% | -0.07% | -0.31% | 117% | 103% |
| Class B | -1.11% | -0.56% | -0.50% | 118% | 102% |
| Class C | -0.66% | -0.55% | -0.64% | 120% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.97% | -0.47% | -0.51% | 120% | 102% |
| Class D | -0.72% | -0.16% | -0.28% | 115% | 101% |
| Class F | -1.53% | -0.87% | -0.82% | 113% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.88% | -0.44% | -0.72% | 127% | 101% |
| Class C | -0.69% | -0.41% | -0.55% | 130% | 105% |
| Class E | -0.33% | 0.44% | -0.48% | 106% | 108% |
| Overall | -0.68% | -0.21% | -0.60% | 123% | 104% |
| Class D | -0.46% | -0.35% | 0.38% | 119% | 103% |
| Class F | -3.07% | -2.65% | -2.31% | 121% | 102% |

Table 4 shows the BD-rate performance of the proposed solution where the LIC and the PROF are exclusively applied to one affine CU. To better understand the impact of the proposed solution, in Table 4, the anchors are the results of the joint applying the LIC and the PROF for affine mode in Table 3 and the test are the results of the proposed solution.

Table 4 The BD-rate performance of exclusively applying the LIC and the PROF, compared to jointly applying the LIC and the PROF

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Joint LIC and PROF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.02% | 0.04% | 100% | 101% |
| Class A2 | 0.08% | 0.01% | 0.05% | 101% | 100% |
| Class B | 0.09% | 0.06% | -0.08% | 99% | 100% |
| Class C | 0.00% | 0.03% | -0.09% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.03% | -0.03% | 100% | 100% |
| Class D | 0.03% | -0.16% | 0.01% | 99% | 99% |
| Class F | 0.02% | 0.06% | -0.06% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Joint LIC and PROF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.27% | 99% | 100% |
| Class C | 0.02% | 0.05% | -0.09% | 96% | 97% |
| Class E | -0.05% | -0.42% | -0.31% | 101% | 97% |
| Overall | 0.00% | -0.07% | 0.01% | 99% | 98% |
| Class D | -0.05% | -0.43% | -0.39% | 96% | 96% |
| Class F | 0.13% | 0.21% | -0.74% | 99% | 97% |

# Conclusions

In this proposal, one method is proposed to exclusively apply the LIC and the PROF for one affine CU to reduce the implementation complexity. Simulations results show that with such a simplification, coding gains from LIC and PROF are still very well maintained. The coding performance impact is negligible. We recommend to consider adoption of the proposed method in case we decide to include both LIC and PROF into VVC.

# References

V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, H. Chen, X. Ma, H. Yang, “CE4-3.1a and CE4-3.1b: Unidirectional local illumination compensation with affine prediction”, JVET-O0066, July 2019, Gothenburg, Sweden.

H. Yang, C.-C. Chen, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-N1024, March 2019, Geneva, Switzerland.

J. Luo, Y. He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, July 2019, Gothenburg, Sweden.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).