| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0606 |
| --- | --- |

| Title: | Non-CE3: Context Reduction for MPM Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Xiaoyu Xiu Yi-Wen Chen Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes reducing the number of contexts for MPM signaling in VVC. Specifically, the number of CABAC context modelling for planar mode flag, intra_luma_not_planar_flag, is reduced to 1. The results reportedly show that the proposed simplification incurs 0.00%, 0.00% and 0.00% BD-rate change for AI, RA and LDB configuration.

Introduction

In VTM5.0 [1], a unified 6-MPM list [2] is used for intra block mode signaling regardless of whether MRL and/or ISP coding tools are applied or not. The unified MPM list is constructed as follows:

- When a neighboring block is not available, its intra mode is set to Planar by default.

- If both modes Left and Above are non-angular modes:

  - MPM list  {Planar, DC, V, H, V-4, V+4}

- If one of modes Left and Above is angular mode, and the other is non-angular:

  - Set a mode Max as the larger mode in Left and Above

  - MPM list  {Planar, Max, DC, Max -1, Max +1, Max -2}

- If Left and Above are both angular and they are different:

  - Set a mode Max as the larger mode in Left and Above

  - if the difference of mode Left and Above is in the range of 2 to 62, inclusive

    - MPM list  {Planar, Left, Above, DC, Max -1, Max +1}

  - Otherwise

    - MPM list  {Planar, Left, Above, DC, Max -2, Max +2}

- If Left and Above are both angular and they are the same:

  - MPM list  {Planar, Left, Left -1, Left +1, DC, Left -2}

It can be seen that the first MPM is always defined as Planar mode. As a result, the first bin MPM0 is also referred as planar mode flag. According to the current VVC, MPM0 is context coded, with the other 5 MPM bins (MPM1-5) coded in bypass mode. Two contexts are used for coding the planar mode flag, depending on whether the current intra block is ISP enabled or a normal intra block. Furthermore, the planar mode flag is not coded and inferred as 0 in MRL mode.

Proposed Implementation

In this contribution, it is proposed to use only one context model for coding the planar mode flag, regardless of whether ISP is enabled or not for a current block.

Results

The proposed modification is integrated into VTM-5.0 [1] and its results and its results against VTM-5.0 under CTC [3] are shown in Table 1.

Table 1. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | -0.02% | 99% | 98% |
| Class A2 | 0.00% | -0.01% | -0.03% | 101% | 100% |
| Class B | 0.00% | -0.05% | 0.04% | 101% | 101% |
| Class C | 0.00% | -0.07% | 0.07% | 97% | 100% |
| Class E | 0.00% | -0.09% | -0.04% | 100% | 97% |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class D | 0.02% | 0.14% | -0.05% | 101% | 97% |
| Class F | -0.01% | -0.03% | -0.05% | 102% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.07% | 101% | 101% |
| Class A2 | 0.02% | 0.21% | 0.04% | 101% | 102% |
| Class B | 0.01% | 0.09% | -0.13% | 99% | 100% |
| Class C | -0.02% | 0.06% | -0.06% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | -0.04% | 100% | 100% |
| Class D | 0.01% | -0.11% | 0.11% | 102% | 101% |
| Class F | -0.06% | 0.09% | -0.03% | 101% | 101% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.30% | -0.11% | 100% | 104% |
| Class C | 0.01% | 0.19% | 0.00% | 99% | 101% |
| Class E | -0.03% | 0.41% | 0.78% | 103% | 103% |
| Overall | 0.00% | 0.29% | 0.15% | 100% | 103% |
| Class D | 0.05% | -0.17% | 0.16% | 98% | 96% |
| Class F | 0.00% | -0.16% | 0.79% | 100% | 101% |

Conclusion

In this proposal, we propose to use only one context model for coding the planar mode flag. The results show that there is essentially no coding performance impact. We recommend to consider adoption of such a simplification into VVC.

References

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Wang, A. M. Kotra, S. Esenlik, H. Gao and J. Chen , “ CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, the 14th JVET meeting, Mar. 2019, Geneva, CH.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Mar. 2019, Geneva, CH.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).