| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0607 |
| --- | --- |

| Title: | CE4-related: On handling the interaction of the mvd_l1_zero_flag and sym_mvd_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the VVC working draft 5, there are two control flags to indicate whether the L1 motion vector difference (MVD) syntax may be bypassed when parsing the bitstream. Firstly, when the flag mvd_l1_zero_flag in the current slice is equal to true, the L1 MVDs of bi-predicted coding units (CUs) in the slice will be always inferred as zero without signaling. Secondly, when the flag sym_mvd_flag of one bi-predicted CU is true, only the L0 MVD is signaled and the L1 MVD is inferred to be the opposite value to the L0 MVD. In this contribution, two methods are proposed to handle the interaction between the two control flags.

Firstly, it is reported that there is mismatch between the VVC working draft 5 and the test model VTM-5.0 on handling the interaction of the two control flags. Therefore, in the first method, it is proposed to align the test model with the specification text. In the second method, when the mvd_l1_zero_flag is true, it is proposed to condition the presence of the L0 MVD based on the value of sym_mvd_flag such that when sym_mvd_flag is true both the L0 and L1 MVDs are bypassed. It is asserted that the second method ccan fulfill the physical meanings of mvd_l1_zero_flag and sym_mvd_flag. The proposed methods have no performance impact under common test condition (CTC).

# Problem statement

In the VVC working draft 5 [1], there are two control flags to indicate whether to bypass the L1 motion vector difference (MVD) syntax. Firstly, when the flag mvd_l1_zero_flag of one slice is equal to true, the L1 MVDs of bi-predicted coding units (CUs) in the slice are not signaled but inferred to be zeros. Secondly, symmetrical MVD (SMVD) mode is enabled for one bi-predicted CU (indicated by setting the flag sym_mvd_flag to true), only the L0 MVD of the CU is signaled and the L1 MVD is inferred to be the opposite to the L0 MVD. In the VVC working draft 5 [1] and the test model VTM-5.0 [2], though the two flags are independently signaled, they cannot take in effect at the same time. Moreover, two different methods are applied in the specification text and the test model to handle the interaction of the two control flags, as illustrated as follows:

## The SMVD signaling in the VVC working draft 5

As shown in Table 1, in the VVC working draft 5, the mvd_l1_zero_flag has higher priority than the sym_mvd_flag when determining the inferred L1 MVD value. Specifically, only if the flag mvd_l1_zero_flag is false, the sym_mvd_flag flag is used to infer the value of L1 MVD. In such case, if sym_mvd_flag has a value of 1, the L1 MVD is set to be the opposite of the L0 MVD. Otherwise, the L1 MVD is explicitly signaled. But, when the flag mvd_l1_zero_flag is equal to true, the value of the L1 MVD is always forced to be zero without considering the sym_mvd_flag flag.

Table 1 The SMVD signaling design in the VVC working draft 5

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |

## The SMVD signaling in the VTM-5.0

Different from the VVC working draft, in the test model VTM-5.0, the sym_mvd_flag has higher priority than the mvd_l1_zero_flag flag. In details, the mvd_l1_zero_flag is only used to forcibly set the L1 MVD to zero, when the sym_mvd_flag flag. When the sym_mvd_flag is equal to true, the value of the L1 MVD will be always set to the opposite of the L0 MVD no matter what value the mvd_l1_zero_flag is.

# Proposal

In order to better handle the interaction between the mvd_l1_zero_flag flag and the sym_mvd_flag, the following solutions are proposed in this contribution.

In the first solution, it is proposed to align the VTM-5.0 reference software with the specification text in the VVC working draft 5.

In the first solution, when both the mvd_l1_zero_flag flag and the sym_mvd_flag flag are true, it may result in one non-zero L0 MVD (as signaled in bitstream) and one zero L1 MVD (as indicated by the mvd_l1_zero_flag). This is not in line with the physical meaning of the SMVD mode, i.e., the L0 and L1 MVDs are symmetrical. To address such issue, in the second solution, it is proposed to condition the presence of the L0 MVD based on the sym_mvd_flag, when the mvd_l1_zero_flag. Specifically, when the sym_mvd_flag flag is true, then the L0 MVD will not be signaled but inferred to be zero; otherwise (i.e., the sym_mvd_flag is false), the L0 MVD will be still signaled in bitstream. Correspondingly, by the second method, when the values of the mvd_l1_zero_flag flag and the sym_mvd_flag are true, both the L0 MVD and the L1 MVD are forcibly set to be zeros, which are symmetrical. Table 2 illustrates the modified SMVD signaling based on the second solution where the changes are highlighted in green.

Table 2 The SMVD signaling of the second solution

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| if(sym_mvd_flag[ x0 ][ y0 ] && mvd_l1_zero_flag) |  |
| { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| } |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |

# Simulation results

The proposed methods have no performance impact under the current common test conditions (CTCs).

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March. 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).