| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0609-r1 |
| --- | --- |

| Title: | CE9-related: On BDOF precision alignment for high internal bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Bi-directional optical flow (BDOF) is included in the VVC to enhance the granularity and accuracy of the motion vectors (MVs) used for motion compensation. It is reported that in the VVC working draft 5 the precisions of the intermediate parameters in the BDOF derivation are consistent when the internal bit-depth is 8-bit to 12-bit but inconsistent when the internal bit-depth is greater than 12-bit. In this contribution, it is proposed to unify the derivation precision of the BDOF for different internal bit-depths. Specifically, in the proposed method, the same BDOF derivation precision that is used for 8-bit to 12-bit is extended to be used for high internal bit-depth (i.e., > 12-bit). The proposed modification has no performance impact under the common test condition (CTC).

# Introduction

In the VVC working draft 5 [1], bi-directional optical flow (BDOF) is applied, which provides the motion refinements based on 4×4 subblocks for bi-predicted coding units (CUs) that has both forward and backward predictions. In general, the existing BDOF derivation can be summarized by the following steps:

- Gradient calculation: the horizonal and vertical gradients are calculated as the difference between two neighboring prediction samples, i.e.,

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx+1][vy] −predSampleL0[hx−1][vy ]) >> Max(6, bitDepth−6)	(8-783)

gradientVL0[ x ][ y ]  =  (predSampleL0[hx][vy+1] − predSampleL0[hx][vy−1] ) >> Max(6, bitDepth−6)	(8-784)

gradientHL1[ x ][ y ]  =  (predSamplesL1[hx+1][vy] − predSampleL1[hx−1][vy] ) >> Max(6, bitDepth−6)	(8-785)

gradientVL1[ x ][ y ]  =  (predSampleL1[hx][vy+1] − predSampleL1[hx][vy−1] ) >> Max(6, bitDepth−6)	(8-786)

- Correlation parameter calculation: the variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are firstly calculated as

diff[ x ][ y ] = (predSamplesL0[hx][vy] >> shift2 ) − ( predSamplesL1[hx][vy] >> Max(4, bitDepth−8 ))	(8-787)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> Max(1, bitDepth−11)	(8-788)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> Max(1, bitDepth−11)	(8-789)

Then, the BDOF parameters sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as

sGx2 = ij( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-790)

sGy2 = ij( tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-791)

sGxGy = ij( tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-792)

sGxGym = sGxGy >> 12		(8-793)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-794)

sGxdI = ij( −tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-795)

sGydI = ij( −tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-796)

- Motion refinement derivation: the horizontal and vertical motion refinements are derived as

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	(8-797)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

tempvy = ( sGydI << 3 ) −  ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) 	(8-798)

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, tempvy >> Floor( Log2( sGx2 ) ) )  :  0	(8-799)

where the variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ).

- BDOF sample refinement calculation:

bdofOffset =  ( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-800)
+  ( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

Table 1 illustrates the bit-widths (i.e., dynamic range) of the intermediate parameters that are used in each BDOF derivation stage when the internal bit-depth ranges from 8-bit to 12-bit. Table 2 and Table 3 illustrates the bit-widths of the corresponding BDOF intermediate parameters when the internal bit-depth is 14-bit and 16-bit, respectively.

Table 1 The bit-width of the BDOF intermediate parameters in the VVC working draft 5, for internal bit-depth from 8-bit to 12-bit

| Operation | Parameter | Bit-width |
| --- | --- | --- |
| Gradient calculation | gradientHL0, gradientVL0 gradientHL1, gradientVL1 | 11 |
| Correlation parameter calculation | diff | 13 |
|  | tempH, tempV | 11 |
|  | sGx2, sGxGy, sGy2 | 26 |
|  | sGxdI, sGydI | 28 |
| Motion refinement | tempvy | 31 |
|  | vx, vy | 6 |

Table 2 The bit-width of the BDOF intermediate parameters in the VVC working draft 5, for internal bit-depth of 14-bit

| Operation | Parameter | Bit-width |
| --- | --- | --- |
| Gradient calculation | gradientHL0, gradientVL0 gradientHL1, gradientVL1 | 11 |
| Correlation parameter calculation | diff | 13 |
|  | tempH, tempV | 9 |
|  | sGx2, sGxGy, sGy2 | 22 |
|  | sGxdI, sGydI | 28 |
| Motion refinement | tempvy | 31 |
|  | vx, vy | 8 |

Table 3 The bit-width of the BDOF intermediate parameters in the VVC working draft 5, for internal bit-depth of 16-bit

| Operation | Parameter | Bit-width |
| --- | --- | --- |
| Gradient calculation | gradientHL0, gradientVL0 gradientHL1, gradientVL1 | 11 |
| Correlation parameter calculation | diff | 13 |
|  | tempH, tempV | 7 |
|  | sGx2, sGxGy, sGy2 | 18 |
|  | sGxdI, sGydI | 28 |
| Motion refinement | tempvy | 31 |
|  | vx, vy | 10 |

Based on the comparison of Table 1 to Table 3, the following observations can be achieved:

- For internal bit-depth from 8-bit to 16-bit, the bit-widths of all the BDOF intermediate parameters do not exceed 32-bit, i.e., the existing BDOF design can guarantee 32-bit hardware implementation.

- The same intermediate precisions are used by the BDOF for the internal bit-depths from 8-bit to 12-bit. But, the precisions of partial intermediate parameters when the internal bit-depth is 14-bit or 16-bit are inconsistent with the corresponding precisions used for 8-bit to 12-bit. For example, in Table 1, the precision of the variables sGx2, sGxGy and sGy2 are 26-bit which however reduce to 22-bit and 18-bit for internal bit-depth of 14-bit (in Table 2) and 16-bit (in Table 3), respectively.

# Proposal

This contribution proposes to unify the precisions used for the BDOF derivation for different internal bit-depths. Specifically, the following modifications are proposed: 1) the right-shift used to calculate the prediction sample difference (i.e., diff[ x ][ y ] ) is fixed to 1; 2) the motion refinement threshold mvRefineThres is fixed to be 32; 3) one left shift is introduced to compensate the additional right shifts that are applied to calculate gradients for internal bit-depths beyond 12-bit. After the proposed method is applied, the same BDOF derivation precision that is used in Table 1 will be reused for high internal bit-depth (i.e., > 12-bit).

# Simulation results

The proposed modification has no performance impact for internal bit-depth from 8-bit to 12-bit. Therefore, the BD-rate performance of VTM-5.0 [2] under the common test condition (CTC) does not change. However, since the current VTM-5.0 does not support the coding with internal bit-depth larger than 12-bit, the experiments of the proposed modifications on 14-bit and 16-bit cannot be conducted.

# Proposed spec changes

#### 8.5.6.4 Bidirectional optical flow prediction process

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 ).

- The variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift3 is set to equal to Max( 1, bitDepth − 11 )1.

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 )32.

- The variable shift5 is set to equal to Max( 0, bitDepth - 12 )

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset =  ( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) << shift5) >> 1 )	(8-799)
	+  ( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) << shift5) >> 1 )

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March. 2019, Geneva, Switzerland.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).