| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0610 |
| --- | --- |

| Title: | AHG17: Association of VCL NAL units to coded pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Wade Wan | Email: | brian.heng@broadcom.com wade.wan@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution claims that the existing process required to map NAL units to pictures places an undue burden on decoders and bitstream parsers.  It recommends adding a first_slice_in_pic_flag at the start of each slice header to allow picture boundaries to be easily determined.

# Introduction

In addition to regular decoders, it is frequently necessary for light-weight bitstream parsers to quickly determine the boundaries of coded pictures. This is a typical requirement for trick-modes (fast-forward, reverse, etc…), bitstream multiplexing, and so on.

The process for mapping NAL units to access units and coded pictures is described in VVC Draft 5 v9, Section 7.4.2.4.4 and 7.4.2.4.5.  Note that currently, the latter section is empty (“TBD”).

Ultimately, the process for determining the start of a new picture involves finding the first VCL NAL unit of a picture.  To do this, the VTM software checks whether the next slice starts with CTU address equal to (0,0) or not.

This contribution asserts that determining whether the first CTU in a slice has address (0,0) requires an excessive amount of computation, all for the simple purpose of determining picture boundaries.

In the current VVC draft text, the slice header syntax table includes the following syntax elements.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |

First, the slice_pic_parameter_set_id is variable length coded.  It is not ideal to require light-weight parsers to parse and skip over variable length syntax elements.

Second, the slice_address is fixed length coded, however the number of bits used in this code requires knowledge of the full tile/brick/slice structure used for this picture. Specifically, to determine the number of bits used to code slice_address, the following processing must be done.

- Parse and store any SPS/PPS in the bitstream up to this point.

- Activate the PPS referenced in the slice header.

- Activate the SPS referenced in the PPS to determine the picture size.

- Fully determine the Tile/Brick/Slice structure from the PPS information.

- Different processing dependent on whether the picture is using raster or rectangular slices and  whether explicit slice IDs are signalled or not.  These may include:

  - Determine the number of tiles in the picture.

  - Determine the number of bricks in each tile and total number of bricks in the picture.

  - Determine the total number of slices in the picture.

  - Determine how many bits does it take to represent that many bricks or slices.

  - Determine how many bits are used for slice ID.

After doing all of the above, and parsing the slice_address code, when using rectangular slices and explicitly signalled slice IDs, the decoded slice_address ID needs to be mapped to an entry in the PPS slice array to figure out if the current slice is the first slice in the picture or not.

Access unit delimiters can make this process easier.  However, access unit delimiters are optional, and therefore they cannot be relied on.

In H.265/HEVC, each slice header included a 1-bit “first_slice_segment_in_pic_flag” at the very start of the slice header.  This made it easy to determine whether each slice started a new picture or not.

Similarly, H.264/AVC included a slice_address.  However, this slice_address was simply ue(v) coded, and the value 0 (a 1-bit code) could be easily checked to determine if the current slice was the first slice or not.

# Proposed Solution

This contribution proposes to add back the first_slice_in_pic_flag at the start of the slice header syntax.  This flag has been included in all previous versions of VTM software, up until it was removed on June 18, 2019 with JVET-N0857 related changes.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |

Restoring this flag requires only 1 additional bit in the slice header and greatly simplifies the parsing process.

Further optimization could be done, for example, to infer slice_address equal to 0 when first_slice_in_pic_flag is equal to 1.  However, to keep the changes minimal, this contribution does not pursue such additional optimization.

# Conclusions

Given the wide range of applications and usage modes that need to quickly determine picture boundaries, this contribution asserts that it is undesirable to require this much processing for what should be a basic task.  It is recommended to add back the first_slice_in_pic_flag to resolve this issue.

# References

[1] [2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.