| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0612 |
| --- | --- |

| Title: | Complexity Reduction for BDOF Parameters Calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone | Email: | Krit.panusopone@nokia.com |
| Source: | Nokia, USA |  |  |

_____________________________

# Abstract

This contribution proposes to reduce number of pixel gradients and pixel differences used in BDOF parameters calculation. The proposed method only computes pixel gradients and pixel differences for pixel within 4x4 block. The proposed changes reduce number of addition and multiplication from 180 to 80 for each 4x4 block. Moreover, padding step required to store gradients for pixels outside CU can be avoided.

# Introduction

BDOF (Bi-Directional Optical Flow) is an inter prediction tool that does not require any explicit signal to control its operation. BDOF utilizes gradients and differences between pixels in blocks determined by list 0 MV and list 1 MV to determine BDOF parameters. These parameters are used to compute MV refinement, which is then used to refine bi-prediction signal. These steps are performed independently in both encoder and decoder thereby overhead bit is not needed.

In BDOF, a CU is split into 4x4 blocks and BDOF parameters are derived for each 4x4 block. For each 4x4 block, a 6x6 surrounding region is defined as a block coincides with 4x4 block with one additional row to the top, one additional row to the bottom, one additional column to the left and one additional column to the left. Notice that there are two extra rows and two extra columns for each CU to support surrounding region of blocks along CU boundary.

# Block statistics calculation

Block statistics calculation step in BDF processes pixel gradient and pixel difference to compute BDOF parameters. Input to this step are pixel gradients and pixel difference for every pixel in 6x6 surrounding region. Equations (8-775) – (8-779) in [1] describes processing steps to compute BDOF parameters.

Using eqns. (8-775) – (8-779), there are a total of 36 additions and multiplications for each BDOF parameter, one for each pixel within 6x6 surrounding region. Hence, for a 4x4 block, a total of 180 additions and multiplications are required to compute 5 BDOF parameters. Additional memory is also required to store gradient information outside CU boundary.

# Proposal

This contribution aims to reduce complexity of BDOF parameters computation by reducing number of pixels involved in block statistics calculation step. Specifically, only pixel gradients and pixel differences within 4x4 block, instead of those within 6x6 surrounding region, are used. The proposed method modifies equations (8-775) –(8779) as shown below to accommodate the proposed change.

With this change, number of additions and multiplications to compute the 5 BDOF parameters is decreased to 80; 5 additions and multiplications for each pixel within 4x4 block, for every 4x4 block in a CU that uses BDOF. Additionally, there is no need to compute and to store gradients for pixels outside subblock boundary. Finally, padding step and memory to store gradient information outside CU boundary can be avoided.

Additionally, this contribution proposes to move sample padding process to before gradient calculation, instead of after it as in VTM5 [2]. This change allows a more accurate gradient calculation for pixel along CU boundary.

# Simulation results

The proposed modification is implemented in VTM-5.0 [2] and the results of the proposed method are shown in Table 1 against the VTM-5.0 under a modified RA CTC [3]. Notice that only results of the first 17 frames are reported in Table 1.

Table 1. Simulation result of the proposed method

|  | Short Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.50% | 0.15% | 103% | 101% |
| Class A2 | 0.00% | -0.09% | 0.15% | 104% | 103% |
| Class B | 0.08% | 0.03% | -0.23% | 103% | 102% |
| Class C | 0.02% | -0.19% | 0.03% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | -0.01% | 103% | 101% |
| Class D | -0.19% | -0.30% | 0.15% | 103% | 101% |
| Class F | 0.01% | 0.00% | 0.14% | 103% | 102% |

# Conclusions

Complexity reduction for BDOF parameters calculation is proposed in this contribution. Instead of using pixels gradients and pixels differences within 6x6 surrounding region to compute BDOF parameters, those statistics within 4x4 block are used instead.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding”, JVET- JVET-N1001-v6, March 2019.

- VTM-5.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).