| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0613 |
| --- | --- |

| Title: | CE7-related: Unification of Rice Parameter Tables in residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft 5, two Rice Parameter tables are used for residual coding, one table is used when transform skip is on, the other table is used when transform skip is off. This contribution proposes a method to use one Rice parameter table independently on the status of transform skip. With this method, one lookup table is removed and the amount of text description is reduced.

Compared to VTM-5.0 at CTC QPs with QP in [22, 27, 32, 37], experimental results show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

Compared to VTM-5.0 at low QPs with QP in [2, 7, 12, 17], experimental results show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

# Introduction

In VVC Draft 5, two Rice Parameter tables are used for residual coding, one table is used when transform skip is on, the other table is used when transform skip is off as depicted in Figure 1. This contribution proposes to remove the table for transform skip and reuse the table for the coding of transform coefficients.

As depicted in Figure 1 for VCC Draft 5 when transform skip is not activated, it first calculates a local sum of a maximum of five neighboring absolute levels to produce locSumAbs5. Then locSumAbs5 is modified and clipped to between 0 and 31 with baseLevel as input to produce locSumAbs:

locSumAbs = Clip3( 0, 31, locSumAbs5 − baseLevel * 5 )

which is used as input to the look up table LUT0 to determine the rice parameter cRiceParam.

When transform skip is activated, it first calculates a local sum of a maximum of two neighboring absolute levels to produce locSumAbs2. Then locSumAbs2 is clipped to between 0 and 31 to produce locSumAbs,

locSumAbs = Clip3( 0, 31, locSumAbs2 )

which is used as input to the look up table LUT1 to determine the rice parameter cRiceParam.

Figure 1: In VCC Drat 5, Rice parameter is determined with two look up tables, LUT0 and LUT. LUT1 is used with transform skip, otherwise LUT0 is used without transfrom skip.

Figure 2: In this contribution, the Rice parameter lookup table for transform coefficient coding is used independent on the status of transform skip. Consequently, the Rice parameter lookup table dedicated for transform skip is removed and the amount of text for the VVC draft is reduced.

# Proposed

As depicted in Figure 2, this contribution proposes a method to use one Rice parameter table independently on the status of transform skip while keeping other part unchanged. With this method, one lookup table is removed and the amount of text description is reduced.

In this proposal, when transform skip is not activated, it is the same as VVC Draft 5. It first calculates a local sum of a maximum of five neighboring absolute levels to produce locSumAbs5. Then this sum is modified and clipped to between 0 and 31 with baseLevel as input to produce locSumAbs:

locSumAbs = Clip3( 0, 31, locSumAbs5 − baseLevel * 5 )

which is used as input to the look up table LUT0 to determine the rice parameter cRiceParam. When transform skip is activated, .it first calculates a local sum of a maximum of two neighboring absolute levels to produce locSumAbs2. Then locSumAbs2 is modified by Adjust1 and clipped to between 0 and 31 to produce locSumAbs,

locSumAbs = Clip3( 0, 31, (locSumAbs2 >>1) + 1 )

which is used as input to the same look up table, LUT0, to determine the rice parameter cRiceParam. In this way, only one lookup table is used independently on the status of transform skip.

# Results

The testing condition of the proposed follows the CE7 testing conditions with regular QP range and low QP range. In addition, results from Class F and Class SCC are also reported. Since the anchor and the tests were simulated under different load conditions in the same computer cluster, some timing estimates may not be accurate.

Compared to VTM-5.0 at CTC QPs with QP in {22,27,32,37}, experimental results in Table 1 show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

Compared to VTM-5.0 at low QPs with QP in {2,7,12,17}, experimental results show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

Table 1: CTC QP Results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.01% | 0.01% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.04% | 100% | 102% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 101% |
| Class D | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class F | -0.01% | -0.02% | 0.00% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.04% | 0.01% | 0.06% | 100% | 101% |

Table 2: Low QP Results.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class F | -0.16% | -0.16% | -0.15% | 100% | 100% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.05% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class F | -0.24% | -0.14% | -0.21% | 100% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.02% | 0.01% | 100% | 102% |
| Class E | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.02% | -0.06% | -0.03% | 99% | 98% |
| Class F | -0.10% | -0.03% | -0.13% | 100% | 99% |

# Proposed Text

The proposed method is described in the form of text modifications relative to the specification text of VVC Draft v5 shown below. The text changes are highlighted in yellow and texts with strikethrough indicates deleted texts.

#### 9.5.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies:

locSumAbs2 = 0
if( xC > 0 )
	locSumAbs2 += AbsLevel[ xC − 1 ][ yC ]
if( yC > 0 )	(9-11)
	locSumAbs2 += AbsLevel[ xC ][ yC − 1 ]
locSumAbs = Clip3( 0, 31, (locSumAbs2 >>1) + 1 )

- Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies:

locSumAbs5 = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs5 += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs5 += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs5 += AbsLevel[ xC + 1 ][ yC + 1 ]	(9-12)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs5 += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAb5s += AbsLevel [ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs5 − baseLevel * 5 )

The following applies:

- If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState – 1 ) and given the variables locSumAbs and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-12.

- Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-12.

Table 9-12 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s

| s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
| 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
| 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | cRiceParam | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
| 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
| 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001-v7, 14th meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).