| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0615 |
| --- | --- |

| Title: | Non-CE9: Derivation of extended prediction samples for BDOF gradient calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification

##### 8.5.6.3.1	General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-720)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-721)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-722)

xFracL = refMvLX[ 0 ] & 15		(8-723)

yFracL = refMvLX[ 1 ] & 15		(8-724)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL+(xFracL>>3), yIntL +(yFracL>>3)), ( xFracL, yFracL ) and refPicLX as inputs:

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

    - The list padVal[ dir ] is derived as follows for dir = 0..1:

      - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xL : yL)	(8-725)

      - If disp is less than 0, padVal[ dir ] is set equal to disp.

      - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth : sbHeight ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth : sbHeight ) − 1).

      - Otherwise, padVal[ dir ] is set equal to 0.

    - The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, sbWidth, sbHeight, ( xSb, ySb ) and padVal as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-726)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-727)

xFracC = mvLX[ 0 ] & 31		(8-728)

yFracC = mvLX[ 1 ] & 31		(8-729)

  - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

  - The list padVal[ dir ] is derived as follows for dir = 0..1:

    - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xC : yC)	(8-730)

    - If disp is less than 0, padVal[ dir ] is set equal to disp.

    - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1).

    - Otherwise, padVal[ dir ] is set equal to 0.

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), refPicLX, and padVal as inputs.