| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0615-r1 |
| --- | --- |

| Title: | Non-CE9: Derivation of extended prediction samples for BDOF gradient calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In the current VVC BDOF process, the gradient calculation requires one extended row/column round the CU’s boundary. The extended prediction samples are generated by taking the reference samples at integer positions that rounded by floor() function. This contribution proposes to round the prediction sample position to the nearest integer position. The simulation result reportedly shows that average BD rate savings for RA are {-0.05%, 0.01%, -0.02%}. The difference in complexity is negligible.

# Introduction

In order to derive the gradient values, one additional row/column of prediction samples outside the current CU is required. In order to reduce the computational complexity, the current BDOF process take the reference samples at the nearby integer positions directly without interpolation. The current BDOF process derives the integer position by rounding the prediction sample position using floor() function.

This contribution proposes to modify the derivation of extend sample position by rounding to the nearest integer position. The rounding process is can be implemented as a right-shift operation.

# Simulation results

The proposed modification has been implemented on top of VTM-5.0. The test condition described in JVET-N1010 [1] is used in the simulation. The simulation result shows that on average the gain on RA is 0.05. In the CE4-2.1 [2], the prediction sample for extended boundary are generated by rounding the prediction sample position to nearest integer position. As supplementary information, simulation result on using floor() function in CE4-2.1 are provided in table 2. The result reportedly shows that on average 0.06 loss on both RA and LD.

Table 1.  Using nearest integer sample in BDOF extended prediction sample around CU boundary

|  | Over VTM 5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | 0.05% | 0.02% | 100% | 99% |
| Class B | -0.04% | 0.03% | -0.03% | 100% | 101% |
| Class C | -0.10% | -0.01% | -0.05% | 100% | 101% |
| Class E | - | - | - | - | - |
| Overall | -0.05% | 0.01% | -0.02% | 100% | 100% |
| Class D | -0.30% | -0.14% | -0.10% | 100% | 102% |
| Class F | -0.02% | 0.00% | -0.03% | 101% | 101% |

Table 2. Using floor() function in CE4-2.1a extended prediction sample around subblock boundary

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1a |  |  |  |  | Over CE4-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | 0.01% | 100% | 100% | - | - | - | - | - |
| Class A2 | 0.12% | 0.06% | -0.02% | 100% | 100% | - | - | - | - | - |
| Class B | 0.08% | 0.17% | 0.11% | 100% | 100% | 0.05% | 0.14% | 0.14% | 100% | 99% |
| Class C | 0.05% | -0.11% | 0.03% | 100% | 100% | 0.04% | -0.09% | 0.14% | 100% | 99% |
| Class E | - | - | - | - | - | 0.09% | -0.16% | 0.52% | 100% | 99% |
| Overall | 0.06% | 0.05% | 0.04% | 100% | 100% | 0.06% | -0.01% | 0.23% | 100% | 99% |
| Class D | 0.12% | -0.03% | -0.13% | 100% | 101% | 0.05% | -0.01% | -0.56% | 100% | 100% |
| Class F | 0.19% | 0.15% | 0.14% | 100% | 101% | 0.30% | 0.15% | -0.25% | 100% | 100% |

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

- J. Luo, Y. He, “CE4-2.1: Prediction refinement with optical flow for affine mode”, JVET-O0070, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).