The newly added texts on top of the draft provided by JVET-N1001-v8[2] are highlighted in green.

## 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | tu(v) |
| if( slice_alf_chroma_idc && ( slice_type != I ¦ ¦ slice_num_alf_aps_ids_luma != 1) ) |  |
| slice_alf_aps_id_chroma | u(5) |
| … |  |
| if( ChromaArrayType != 0 && !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| … |  |
| } |  |

slice_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. slice_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. slice_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. slice_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When slice_alf_chroma_idc is not present, it is inferred to be equal to 0. When slice_alf_chroma_idc is not present, it is inferred to be equal to 0.

The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the current slice. slice_ chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current slice. When slice_ chroma_residual_scale_flag is not present, it is inferred to be equal to 0. When slice_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

## 7.4.6.3	Adaptive loop filter data semantics

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag shall be equal to 0.