| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0616 |
| --- | --- |

| Title: | Non-CE: Various chroma format support in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang, Li Zhang, Hongbin Liu, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the last meeting, it is decided to support 4:0:0 color format in VVC. Several tools related to chroma coding have been modified to be conditionally enabled or signaled. However, it is noticed that some tools still need to be modified under the consideration of different color formats. In this contribution, signalling of a few syntax elements and semantics are further modified based on the condition of ChromaArrayType.

# Introduction

JVET-N0225 [1] was adopted in the last JEVT meeting to support various chroma formats. More specifically, the signalling of the following tools has been revised to support the conditions of ChromaArrayType equal or unequal to 0.

- qtbtt_dual_tree_intra_flag

- sps_cclm_enabled_flag

- alf_chroma_idc

- intra_chroma_pred_mode

- pcm_sample_chroma

- tu_cbf_cb and tu_cbf_cr

However, there are some other tools not revised due to recent changes of VVC.

# Proposed method

It is proposed to parse bit_depth_chroma_minus8, pcm_sample_bit_depth_chroma_minus1, slice_alf_chroma_idc, and slice_chroma_residual_scale_flag only when ChromaArrayType is not equal to 0. In addition, the semantics of alf_chroma_filter_signal_flag is modified to take care of the case that ChromaArrayType equal to 0.

The newly added texts on top of the draft provided by JVET-N1001-v8[2] are highlighted in green.

## 7.3.2.3	Sequence parameter set RBSP syntax

seq_parameter_set_rbsp( ) {

…

if( ChromaArrayType != 0 )

bit_depth_chroma_minus8

…

if( ChromaArrayType != 0 )

pcm_sample_bit_depth_chroma_minus1

…

}

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8                             (7-6)

QpBdOffsetC = 6 * bit_depth_chroma_minus8                                (7-7)

bit_depth_chroma_minus8 shall be in the range of 0 to 8, inclusive. When bit_depth_chroma_minus8 is not present, it is inferred to be equal to 0.

pcm_sample_bit_depth_chroma_minus1 specifies the number of bits used to represent each of PCM sample values of the chroma components as follows:

PcmBitDepthC = pcm_sample_bit_depth_chroma_minus1 + 1	(7-33)

The value of PcmBitDepthC shall be less than or equal to the value of BitDepthC. When pcm_sample_bit_depth_chroma_minus1 is not present, it is inferred to be equal to 0

## 7.3.5.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | tu(v) |
| if( slice_alf_chroma_idc && ( slice_type != I ¦ ¦ slice_num_alf_aps_ids_luma != 1) ) |  |
| slice_alf_aps_id_chroma | u(5) |
| … |  |
| if( ChromaArrayType != 0 && !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| … |  |
| } |  |

slice_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. slice_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. slice_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. slice_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When slice_alf_chroma_idc is not present, it is inferred to be equal to 0. When slice_alf_chroma_idc is not present, it is inferred to be equal to 0.

The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the current slice. slice_ chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current slice. When slice_ chroma_residual_scale_flag is not present, it is inferred to be equal to 0. When slice_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

## 7.4.6.3	Adaptive loop filter data semantics

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag shall be equal to 0.

- Conclusions

In this contribution, some aspects are proposed to support various chroma formats in VVC. It is recommended to adopt this proposal so that VVC could support various chroma formats.

# References

[1]	L. Li, J. Nam, J. Heo, J. Lim, “Various chroma format support in VVC,” JVET-N0225, Mar. 2019.

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001-v8, Mar. 2019.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).