| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0617 |
| --- | --- |

| Title: | CE7-related: Context Reduction of Transform Coefficient Significance Flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xin Zhao, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution presents three proposals to reduce the number of sig_coeff_flag context models from 90 contexts to 75, 60 and 48, respectively. The experiments were conducted following CE7 testing conditions.

For the 48 contexts (47% reduction) proposal, experimental results show CTC QP: 0.07% AI, 0.08% RA, 0.03% LB luma BD-rate and Low QP: 0.04% AI, 0.08% RA, 0.09% LB luma BD-rate.

For the 60 contexts (30% reduction) proposal, experimental results show CTC QP: 0.00% AI, 0.00% RA, -0.01% LB luma BD-rate and Low QP: 0.00% AI, -0.01% RA, -0.01% LB luma BD-rate.

For the 75 contexts (17% reduction) proposal, experimental results show CTC QP: 0.00% AI, 0.01% RA, -0.04% LB luma BD-rate and Low QP: -0.01% AI, -0.01% RA, -0.02% LB luma BD-rate.

# Introduction

In VVC Draft 5 [1], 90 context models are used to encode the sig_coeff_flag of transform coefficients. The selection of context model index is based on a sum of a maximum of five previously partially reconstructed absolute level called locSumAbsPass1 as in the following text from VCC Draft 5 [1] section 9.5.4.2.8:

- If cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = 18 * Max( 0, QState − 1) + Min( locSumAbsPass1, 5 ) + ( d < 2  ?  12  :  ( d < 5  ?  6  :  0 ) )	(9-51)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 54 + 12 * Max( 0, QState − 1) + Min( locSumAbsPass1, 5 ) + ( d < 2  ?  6  :  0 )	(9-52)

# Proposal

This contribution proposes to change Equation (9-51) and Equation (9-52) to reduce the number of contexts to produce various trade-off of coding efficiency and the number of contexts. The changes are highlighted in yellow. The context initializations were also modified to match the corresponding changes.

## A proposal with 42 contexts reduction

This section proposes to modify Equation (9-51) and Equation (9-52) to reduce the number of contexts to 48.

- If cIdx is equal to 0, ctxInc is derived as follows:

ctxInc =  8 * Max( 0, QState − 1) + Min( (locSumAbsPass1+1)>>1, 3 ) + ( d < 5  ?  4  :  0 )	(9-51)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 24 + 8 * Max( 0, QState − 1) + Min( (locSumAbsPass1+1)>>1, 3 ) + ( d < 2  ?  4  :  0 )	(9-52)

## A proposal with 30 contexts reduction

This section proposes to modify Equation (9-51) and Equation (9-52) to reduce the number of contexts to 60.

- If cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = 12 * Max( 0, QState − 1) + Min( (locSumAbsPass1+1)>>1, 3 ) + ( d < 2  ?  8  :  ( d < 5  ?  4  :  0 ) )	(9-51)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 36 + 8 * Max( 0, QState − 1) + Min( (locSumAbsPass1+1)>>1, 3 ) + ( d < 2  ?  4  :  0 )	(9-52)

## A proposal with 15 contexts reduction

This section proposes to modify Equation (9-51) and Equation (9-52) to reduce the number of contexts to 75.

- If cIdx is equal to 0, ctxInc is derived as follows:

scaledLocSumAbsPass1 = locSumAbsPass1 – (locSumAbsPass1>>2)

ctxInc = 15 * Max( 0, QState − 1) + Min( scaledLocSumAbsPass1, 4 ) + ( d < 2  ?  10  :  ( d < 5  ?  5  :  0 ) )	(9-51)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

scaledLocSumAbsPass1 = locSumAbsPass1 – (locSumAbsPass1>>2)

ctxInc = 45 + 15 * Max( 0, QState − 1) + Min( scaledLocSumAbsPass1, 4 ) + ( d < 2  ?  5  :  0 )	(9-52)

# Results

The testing condition of the proposals follows the CE7 testing conditions with CTC QP range and low QP range. Since the anchor and the tests were simulated under different load conditions in the same computer cluster, some timing estimates may not be accurate.

## Results with 42 contexts reduction

For the 48 contexts proposal, experimental results show CTC QP: 0.07% AI, 0.08% RA, 0.03% LB luma BD-rate, Low QP: 0.04% AI, 0.08% RA, 0.09% LB luma BD-rate.

Table 1: CTC QP range with 48 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.11% | -0.11% | 100% | 101% |
| Class A2 | 0.12% | -0.17% | -0.11% | 99% | 100% |
| Class B | 0.05% | -0.21% | -0.15% | 99% | 98% |
| Class C | 0.04% | -0.24% | -0.09% | 99% | 101% |
| Class E | 0.05% | -0.17% | -0.14% | 98% | 99% |
| Overall | 0.07% | -0.19% | -0.12% | 99% | 100% |
| Class D | 0.05% | -0.01% | -0.30% | 98% | 101% |
| Class F | 0.03% | 0.03% | 0.00% | 100% | 104% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.29% | 0.00% | 100% | 100% |
| Class A2 | 0.11% | -0.18% | -0.17% | 100% | 100% |
| Class B | 0.11% | -0.36% | -0.39% | 100% | 99% |
| Class C | 0.03% | -0.29% | -0.27% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.29% | -0.24% | 100% | 100% |
| Class D | 0.03% | -0.31% | 0.16% | 100% | 100% |
| Class F | 0.04% | -0.07% | 0.08% | 100% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.04% | -0.42% | 100% | 101% |
| Class C | -0.02% | -0.19% | -0.17% | 100% | 96% |
| Class E | 0.08% | -0.13% | 0.18% | 101% | 97% |
| Overall | 0.03% | -0.11% | -0.19% | 100% | 99% |
| Class D | -0.03% | 0.09% | 0.29% | 99% | 101% |
| Class F | -0.03% | -0.42% | -0.56% | 101% | 102% |

Table 2: Low QP range with 48 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.08% | -0.08% | 98% | 101% |
| Class A2 | 0.02% | -0.02% | -0.04% | 99% | 100% |
| Class B | 0.07% | -0.07% | -0.06% | 99% | 100% |
| Class C | 0.02% | -0.09% | -0.07% | 99% | 100% |
| Class E | 0.04% | -0.09% | -0.09% | 99% | 100% |
| Overall | 0.04% | -0.07% | -0.07% | 99% | 100% |
| Class D | -0.01% | -0.05% | -0.08% | 99% | 101% |
| Class F | 0.05% | -0.16% | -0.18% | 99% | 101% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.23% | -0.22% | 99% | 99% |
| Class A2 | 0.10% | -0.07% | -0.07% | 99% | 100% |
| Class B | 0.11% | -0.23% | -0.18% | 100% | 102% |
| Class C | 0.06% | -0.29% | -0.25% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.21% | -0.18% | 99% | 101% |
| Class D | 0.01% | -0.35% | -0.33% | 99% | 101% |
| Class F | 0.19% | -0.10% | -0.19% | 99% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.21% | -0.16% | 99% | 102% |
| Class C | 0.05% | -0.26% | -0.27% | 99% | 101% |
| Class E | 0.09% | -0.29% | -0.42% | 99% | 102% |
| Overall | 0.09% | -0.25% | -0.26% | 99% | 101% |
| Class D | 0.01% | -0.39% | -0.35% | 100% | 100% |
| Class F | 0.05% | -0.31% | -0.18% | 99% | 100% |

## Results with 30 contexts reduction

For the 60 contexts proposal, experimental results show CTC QP: 0.00% AI, 0.00% RA, -0.01% LB luma BD-rate, Low QP: 0.00% AI, -0.01% RA, -0.01% LB luma BD-rate.

Table 3: CTC QP range with 60 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.07% | 0.07% | 99% | 100% |
| Class A2 | 0.00% | -0.04% | -0.01% | 99% | 101% |
| Class B | 0.01% | -0.03% | 0.03% | 99% | 98% |
| Class C | 0.00% | -0.01% | 0.07% | 99% | 100% |
| Class E | 0.00% | -0.12% | 0.13% | 99% | 99% |
| Overall | 0.00% | -0.03% | 0.06% | 99% | 100% |
| Class D | -0.02% | 0.22% | -0.07% | 98% | 98% |
| Class F | 0.00% | 0.11% | 0.13% | 101% | 105% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.18% | 99% | 99% |
| Class A2 | 0.03% | 0.02% | 0.05% | 99% | 99% |
| Class B | -0.01% | 0.07% | -0.09% | 100% | 99% |
| Class C | -0.02% | -0.07% | -0.16% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.03% | 99% | 99% |
| Class D | 0.02% | 0.21% | 0.07% | 100% | 99% |
| Class F | -0.03% | -0.06% | 0.13% | 100% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.07% | 0.34% | 99% | 100% |
| Class C | -0.03% | 0.03% | -0.17% | 100% | 95% |
| Class E | 0.09% | 0.90% | 0.46% | 101% | 96% |
| Overall | -0.01% | 0.27% | 0.20% | 100% | 97% |
| Class D | -0.03% | 0.30% | 0.16% | 99% | 101% |
| Class F | -0.07% | -0.18% | -0.19% | 100% | 100% |

Table 4: Low QP range with 60 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.03% | 99% | 100% |
| Class A2 | -0.01% | 0.03% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Class C | -0.01% | 0.01% | 0.01% | 99% | 99% |
| Class E | 0.01% | 0.01% | -0.04% | 99% | 99% |
| Overall | 0.00% | 0.01% | 0.00% | 99% | 100% |
| Class D | -0.05% | 0.04% | -0.04% | 99% | 102% |
| Class F | 0.02% | -0.01% | -0.03% | 99% | 99% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.04% | 99% | 101% |
| Class A2 | -0.02% | 0.05% | 0.04% | 99% | 99% |
| Class B | 0.01% | -0.02% | -0.01% | 99% | 99% |
| Class C | -0.05% | -0.06% | -0.02% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 99% | 100% |
| Class D | -0.06% | -0.06% | -0.07% | 99% | 101% |
| Class F | 0.06% | -0.07% | -0.06% | 99% | 99% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.03% | -0.01% | 99% | 98% |
| Class C | -0.04% | -0.03% | -0.05% | 99% | 100% |
| Class E | 0.00% | -0.07% | -0.12% | 99% | 99% |
| Overall | -0.01% | -0.04% | -0.05% | 99% | 99% |
| Class D | -0.07% | -0.13% | -0.13% | 100% | 101% |
| Class F | 0.04% | -0.04% | -0.02% | 99% | 98% |

## Results with 15 contexts reduction

For the 75 contexts proposal, experimental results show CTC QP: 0.00% AI, 0.01% RA, -0.04% LB luma BD-rate, Low QP: -0.01% AI, -0.01% RA, -0.02% LB luma BD-rate.

Table 5: CTC QP range with 75 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | -0.05% | 99% | 99% |
| Class B | -0.01% | -0.06% | 0.00% | 99% | 98% |
| Class C | 0.01% | -0.01% | 0.09% | 99% | 102% |
| Class E | 0.00% | -0.06% | -0.06% | 99% | 98% |
| Overall | 0.00% | -0.03% | 0.01% | 99% | 99% |
| Class D | 0.00% | 0.26% | -0.04% | 99% | 99% |
| Class F | -0.01% | -0.05% | 0.05% | 100% | 105% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.04% | 99% | 99% |
| Class A2 | 0.03% | -0.01% | -0.05% | 99% | 99% |
| Class B | 0.00% | 0.16% | -0.17% | 100% | 99% |
| Class C | -0.01% | -0.07% | -0.04% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | -0.07% | 99% | 100% |
| Class D | -0.01% | 0.11% | 0.04% | 100% | 100% |
| Class F | -0.04% | 0.02% | 0.10% | 100% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.16% | 0.07% | 99% | 100% |
| Class C | 0.00% | 0.07% | -0.09% | 100% | 96% |
| Class E | -0.11% | 0.64% | 1.24% | 100% | 95% |
| Overall | -0.04% | 0.25% | 0.31% | 100% | 98% |
| Class D | -0.02% | 0.21% | 0.98% | 100% | 100% |
| Class F | 0.01% | -0.56% | -0.66% | 100% | 98% |

Table 6: Low QP range with 75 contexts for encoding transform coefficients significant flag.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.03% | 99% | 100% |
| Class A2 | -0.01% | 0.01% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.02% | 99% | 100% |
| Class C | -0.02% | 0.03% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.02% | 0.01% | 99% | 100% |
| Overall | -0.01% | 0.01% | 0.01% | 99% | 100% |
| Class D | -0.04% | 0.03% | -0.02% | 99% | 101% |
| Class F | -0.02% | -0.03% | -0.01% | 98% | 99% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.03% | 99% | 100% |
| Class A2 | -0.02% | 0.00% | -0.01% | 99% | 100% |
| Class B | -0.01% | -0.02% | -0.02% | 99% | 100% |
| Class C | -0.02% | -0.04% | -0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.02% | 99% | 100% |
| Class D | -0.06% | -0.05% | -0.05% | 99% | 100% |
| Class F | 0.02% | 0.03% | -0.10% | 99% | 99% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 LowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 98% |
| Class C | -0.03% | -0.01% | -0.02% | 99% | 99% |
| Class E | -0.02% | -0.02% | -0.17% | 99% | 99% |
| Overall | -0.02% | -0.01% | -0.05% | 99% | 98% |
| Class D | -0.02% | -0.04% | -0.06% | 100% | 99% |
| Class F | 0.04% | 0.13% | 0.15% | 98% | 98% |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001-v7, 14th meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).