| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0619-v2 |
| --- | --- |

| Title: | Non-CE7: Simplification of transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In order to unify the coefficient scanning order between transform residual and transform skip residual coding, this contribution proposes reverse scanning order for residual coding of the transform skip (TS) residual block. It also proposes two options of a single pass coding of gt3, gt5, gt7 and gt9 flag of TS residual block in order to  reduce the number of coding passes from 6 to 3. In addition to the coding pass reduction options, a unified by-pass coding method between transform and TS residual coding is also proposed. Following results (RA) are reported.

- Test 1: Reverse scanning order:

  - 0.13%/0.13%/0.04% Y/Cb/Cr BD-rate for TGM

- Test 2: Forward scanning order + single pass gt1 coding – Method 1

  - -0.09%/-0.02%/-0.05% Y/Cb/Cr BD-rate for TGM

- Test 3: Forward scanning order + single pass gt1 coding – Method 2

  - 0.00%/0.04%/-0.09% Y/Cb/Cr BD-rate for TGM

- Test 4: Test 2 + unified by-pass coding:

  - 0.15%/0.23%/0.12% Y/Cb/Cr BD-rate for TGM

# Introduction

In the March 2019 meeting, the JVET adopted a new residual coding process for the transform skip residual block. The transform skip (TS) residual levels of a coefficient group (CG) are coded in six passes over the scan positions as follows:

- Pass 1: Following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2: greater than 3 flag (abs_level_gtx_flag[1]) is signaled

- Pass 3: greater than 5 flag (abs_level_gtx_flag[2]) is signaled

- Pass 4: greater than 7 flag (abs_level_gtx_flag[3]) is signaled

- Pass 5: greater than 9 flag (abs_level_gtx_flag[4]) is signaled

- Pass 6: coding of remaining absolute level (abs_remainder) is processed for all scan positions with greater than 9 flag (abs_level_gtx_flag[4]) equal to 1. The non-binary syntax element is binarized with Golomb-Rice code and the resulting bins are coded in the bypass mode of the arithmetic coding engine.

This TS residual coding process is significantly different from the transform residual coding process of a transform block. The major differences between TS residual coding and transform  residual coding are listed as follows:

- The coefficient scanning of the TS residual coding is forward scan whereas in case of transform residual coding the scanning is in reverse order. The difference in scanning order between transform and TS block impacts hardware implementation complexity.

- The number of coding passes of the TS residual coding differs from that of transform residual coding (6 passes vs 3 passes) which can affect hardware implementation complexity.

- The by-pass coding process of transform and TS residual coding is different. In transform residual coding case, by-pass coding is performed in a single pass (abs_remainder and dec_abs_level using Golomb-Rice code). However, in TS residual coding, by-pass coding can be more than one pass and can have sig_coeff_flag, coeff_sign_flag, gt1_flag, par_level_flag, gt3_flag, gt5_flag, gt7_flag, gt9_flag and abs_remainder. Figure 1 shows an illustration of the by-pass coding of TS residual coding and Figure 2 shows the illustration the transform residual coding. In both cases, it is assumed that the number of context coded bins reached to maximum limit after coding of the red coefficient. The multi-pass by-pass coding not only increases the number of coding passes, but also increases the number of comparisons. For instance, in case of pass 2 (of Figure 1), the decoder requires 16 comparisons (one comparison for each coefficient, assume a CG has 16 coefficients) to check if the gt1 flag of the corresponding coefficient is zero or not. Similarly, pass 3, 4, 5, and 6 need 64 comparisons (16 comparisons for each coding pass). In case of transform residual coding, the by-pass coding requires only 16 comparisons.

Figure 1: An illustration of multi-pass bypass coding in TS residual coding

Figure 2: An illustration of single pass by-pass coding of transform residual coding

# Proposed methods

## Reverse scanning order

In order to unify the scanning order of the transform coding and TS residual coding processes, it is proposed to change the scanning order of the TS residual block from the forward scanning to the reverse scanning order.

## Single pass coding of abs_level_gtx_flag[x], x = 1 to 4

In VVC 5, in total 4 coding passes are used to code greater than 3, greater than 5, greater than 7 and greater than 9 flag (that is, each flag is coded as separate coding pass). We propose to code all of those flags in a single pass. This contribution following proposes two methods:

### Method 1:

In this method, greater than 3, greater than 5 and greater than 7 and greater than 9 flags are single pass coded. Position of parity, greater than 1 and all other flags remain the same as VTM-5.0.

- Pass 1: following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2: following flags are signalled

  - greater than 3 flag (abs_level_gtx_flag[1])

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding of remaining absolute level (abs_remainder) using Golomb-Rice coding

### Method 2:

In this method, the position of parity flag is moved to second pass and the position of the greater than 3 flag is moved to the first pass.

- Pass 1: following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - greater than 3 flag (abs_level_gtx_flag[1])

- Pass 2: following flags are signalled

  - parity (par_level_flag) flag

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding of remaining absolute level (abs_remainder) using Golomb-Rice coding

Both of the method 1 and method 2 require only 3 coding passes (same as transform residual coding) to code a CG, resulting in a reduction of 3 coding passes, which can improve CABAC throughput for hardware implementations.

## Unified by-pass coding

In order to unify the by-pass coding process of transform and TS residual coding, this contribution proposes to modify the by-pass coding method of TS residual coding. The proposed by-pass coding is implemented on top of method 1 (described in the previous 2.2.1). Below is the pseudo code:

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 4; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

iFirstPassBypassPos = n;

Pass 2: for(n = 0; n < iFirstPassBypassPos && remainingCtxBin >= 4; n++ )

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] //gt5

decode abs_level_gtx_flag[3] //gt7

decode abs_level_gtx_flag[4] //gt9

iSecondPassBypassPos = n;

Pass 3 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

baseLevel = (n >= iSecondPassBypassPos ? 2 : 10);

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

Pass 3 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

baseLevel = 0

decode_bypass_sig_flag

if (sig_flag){

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

}

The Rice parameter is derived based on locSumAbs and baseLevel.

locSumAbs  += (10 – baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)

Figure 3: An illustration of the proposed by-pass coding method of TS residual coding

# Experimental results

The proposed methods are implemented on top of VTM-5.0. Following two tests are performed under common test conditions (CTC)[1].

- Test 1: reverse scanning order described in section 2.1

- Test 2: forward scanning + single pass coding of abs_level_gtx_flag – Method 1

- Test 3: forward scanning + single pass coding of abs_level_gtx_flag – Method 2

- Test 4: Test 2 + unified by-pass coding

Table 1, 2, 3, and 4 show the simulation results of test 1, test 2, test 3 and test 4, respectively.

Table 1: Simulation results of test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.02% | 99% | 99% |
| Class A2 | 0.01% | -0.01% | -0.03% | 99% | 99% |
| Class B | 0.03% | -0.02% | 0.04% | 99% | 99% |
| Class C | 0.08% | 0.03% | 0.11% | 100% | 99% |
| Class E | 0.06% | 0.07% | 0.03% | 99% | 99% |
| Overall | 0.04% | 0.01% | 0.04% | 99% | 99% |
| Class D | 0.09% | 0.34% | 0.11% | 99% | 98% |
| Class F | 0.25% | 0.16% | 0.16% | 99% | 99% |
| TGM | 0.31% | 0.27% | 0.24% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | 0.11% | 99% | 100% |
| Class A2 | 0.01% | 0.08% | 0.08% | 99% | 99% |
| Class B | 0.00% | 0.02% | -0.19% | 99% | 99% |
| Class C | 0.00% | -0.29% | -0.38% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.13% | 99% | 99% |
| Class D | 0.04% | 0.14% | 0.02% | 99% | 98% |
| Class F | 0.17% | 0.14% | 0.23% | 99% | 99% |
| TGM | 0.13% | 0.13% | 0.04% | 99% | 100% |

Table 2: Simulation results of test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.02% | 0.00% | 99% | 99% |
| Class C | 0.00% | -0.01% | 0.01% | 99% | 99% |
| Class E | 0.00% | 0.01% | -0.04% | 99% | 99% |
| Overall | 0.00% | 0.01% | -0.01% | 99% | 99% |
| Class D | 0.00% | 0.01% | -0.08% | 98% | 98% |
| Class F | 0.00% | -0.03% | 0.02% | 99% | 99% |
| TGM | 0.01% | 0.01% | 0.01% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.03% | 100% | 99% |
| Class A2 | 0.01% | 0.00% | -0.01% | 100% | 99% |
| Class B | 0.01% | 0.05% | -0.06% | 99% | 99% |
| Class C | -0.03% | -0.22% | -0.09% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.05% | 99% | 99% |
| Class D | 0.05% | -0.02% | 0.18% | 99% | 98% |
| Class F | -0.01% | -0.17% | 0.08% | 99% | 99% |
| TGM | -0.09% | -0.02% | -0.05% | 100% | 99% |

Table 3: Simulation results of test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 99% | 99% |
| Class A2 | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Class B | 0.00% | 0.01% | -0.01% | 99% | 99% |
| Class C | 0.00% | 0.02% | -0.01% | 99% | 99% |
| Class E | 0.00% | 0.04% | 0.04% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.00% | 99% | 99% |
| Class D | -0.01% | 0.07% | -0.13% | 98% | 98% |
| Class F | -0.04% | 0.09% | 0.13% | 99% | 99% |
| TGM | -0.04% | -0.01% | 0.00% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.01% | 99% | 99% |
| Class A2 | 0.00% | 0.15% | 0.08% | 99% | 99% |
| Class B | 0.00% | 0.00% | -0.04% | 99% | 99% |
| Class C | -0.04% | -0.21% | -0.22% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.06% | 99% | 99% |
| Class D | 0.01% | -0.05% | -0.13% | 99% | 97% |
| Class F | -0.01% | -0.04% | 0.19% | 99% | 99% |
| TGM | 0.00% | 0.04% | -0.09% | 99% | 99% |

Table 4: Simulation results of test 4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.01% | -0.01% | 99% | 99% |
| Class B | 0.00% | -0.01% | 0.02% | 99% | 100% |
| Class C | 0.02% | 0.00% | 0.03% | 99% | 99% |
| Class E | 0.00% | 0.06% | 0.03% | 99% | 99% |
| Overall | 0.00% | 0.01% | 0.01% | 99% | 99% |
| Class D | 0.02% | 0.12% | -0.01% | 99% | 97% |
| Class F | 0.14% | 0.12% | 0.20% | 99% | 99% |
| TGM | 0.34% | 0.30% | 0.30% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.04% | 99% | 100% |
| Class A2 | 0.01% | -0.08% | 0.06% | 99% | 99% |
| Class B | 0.00% | -0.02% | -0.22% | 99% | 100% |
| Class C | -0.03% | -0.13% | -0.06% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.07% | 99% | 99% |
| Class D | 0.00% | 0.17% | 0.20% | 99% | 98% |
| Class F | 0.11% | 0.09% | 0.14% | 99% | 99% |
| TGM | 0.15% | 0.23% | 0.12% | 99% | 100% |

# Conclusions

This contribution proposes simplification of TS residual coding. Table 1 shows the performance of reverse scanning order. Although the reverse scanning order introduces a coding loss of 0.31% for AI and 0.13% for RA  for screen content sequences, the performance loss is negligible for mainstream content. Therefore, it is asserted to be an unified solution that can reduce hardware implementation cost. Table 2 and 3 shows the performance of test 2 and 3, respectively. In test 2, single pass coding of gt3, gt5, gt7 and gt9 flags are applied. The position of all other flags in test 2 remain the same as VTM-5.0. In test 3, the position of parity flag is moved to second pass and the position of the greater than 3 flag is moved to the first pass. Both test 2 and test 3 reduce the number of coding passes from 6 to 3. Compared to VTM-5.0, the test 2, and test 3 reportedly achieves -0.09% and 0.00% luma gain of TGM sequences in RA condition, respectively. In test 4, the proposed by-pass coding is implemented on top of test 2. Test 4 is an unified solution between transform residual and TS residual coding and may save hardware complexity with expanse of 0.15% (RA) luma coding loss of TGM sequences.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Specification Changes

## Draft Specification for Test 1

7.3.7.11	Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| if( ( i != 0 ¦ ¦ !inferSbCbf ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && i > 0 ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| for( n = numSbCoeff − 1; n >= 0; n-- ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n != 0 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| MaxCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| MaxCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPassX[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| } |  |
| /* Greater than X scan passes (numGtXFlags=5) */ |  |
| for( j = 1; j < 5; j++ ) { |  |
| for( n = numSbCoeff − 1; n >= 0; n-- ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ j − 1 ] ) |  |
| abs_level_gtx_flag[ n ][ j ] | ae(v) |
| MaxCcbs− − |  |
| AbsLevelPassX[ xC ][ yC ] + = 2 * abs_level_gtx_flag[ n ][ j ] |  |
| } |  |
| } |  |
| /* remainder scan pass */ |  |
| for( n = numSbCoeff − 1; n >= 0; n-- ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 4 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| } |  |
| } |  |

      - Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) 
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
if( yC < (1 << log2TbHeight) − 1 )	(9-11)
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
locSumAbs = Clip3( 0, 31, locSumAbs )

- Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(9-12)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel [ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

The following applies:

- If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-12.

- Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-12.

Table 9-12 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s

| trafoSkip | s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| 1 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
|  | 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
|  | 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  |  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 0 |  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| 1 |  | cRiceParam | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
|  | 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

##### Derivation process for the variables locNumSig, locSumAbsPass1

Inputs to this process are the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Outputs of this process are the variables locNumSig and locSumAbsPass1.

Given the syntax elements sig_coeff_flag[ x ][ y ] and the array AbsLevelPass1[ x ][ C ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variables locNumSig and locSumAbsPass1 are derived as specified by the following pseudo code:

locNumSig          = 0
locSumAbsPass1 = 0
if( transform_skip_flag[ xS ][ yS ] ) {
	if( xC < (1 << log2TbWidth) − 1 ) {
		locNumSig          += sig_coeff_flag[ xC + 1 ][ yC ]
		locSumAbsPass1 += AbsLevelPass1[ xC + 1 ][ yC ]
	}
	if( yC < (1 << log2TbHeight) − 1 ) {
		locNumSig           += sig_coeff_flag[ xC ][ yC + 1 ] 
		locSumAbsPass1 += AbsLevelPass1[ xC ][ yC + 1 ]
	}
} else {
	if( xC < (1 << log2TbWidth) − 1 ) {
		locNumSig          += sig_coeff_flag[ xC + 1 ][ yC ]
		locSumAbsPass1 += AbsLevelPass1[ xC + 1 ][ yC ]
		if( xC < (1 << log2TbWidth) − 2 ) {
			locNumSig           += sig_coeff_flag[ xC + 2 ][ yC ] 
			locSumAbsPass1 += AbsLevelPass1[ xC + 2 ][ yC ]
		}
		if( yC < (1 << log2TbHeight) − 1 ) {
			locNumSig           += sig_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-49)
			locSumAbsPass1 += AbsLevelPass1[ xC + 1 ][ yC + 1 ]
		}
	}
	if( yC < (1 << log2TbHeight) − 1 ) {
		locNumSig           += sig_coeff_flag[ xC ][ yC + 1 ] 
		locSumAbsPass1 += AbsLevelPass1[ xC ][ yC + 1 ]
		if( yC < (1 << log2TbHeight) − 2 ) {
			locNumSig          += sig_coeff_flag[ xC ][ yC + 2 ] 
			locSumAbsPass1 += AbsLevelPass1[ xC ][ yC + 2 ]
		}
	}
}

## Draft Specification for Test 2

7.3.7.11	Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| if( ( i != lastSubBlock ¦ ¦ !inferSbCbf ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && i < lastSubBlock ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| MaxCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| MaxCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPassX[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| } |  |
| /* Greater than X scan pass (numGtXFlags=5) */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| for( j = 1; j < 5; j++ ) { |  |
| if( abs_level_gtx_flag[ n ][ j − 1 ] ) |  |
| abs_level_gtx_flag[ n ][ j ] | ae(v) |
| MaxCcbs− − |  |
| AbsLevelPassX[ xC ][ yC ] + = 2 * abs_level_gtx_flag[ n ][ j ] |  |
| } |  |
| } |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 4 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| } |  |
| } |  |

## Draft Specification for Test 3

7.3.7.11	Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| if( ( i != lastSubBlock ¦ ¦ !inferSbCbf ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && i < lastSubBlock ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| MaxCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| MaxCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| abs_level_gtx_flag[ n ][1 ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPassX[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + abs_level_gtx_flag[ n ][ 0 ] + abs_level_gtx_flag[ n ][ 0 ] << 1 |  |
| } |  |
| /* Greater than X scan pass (numGtXFlags=5) */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| AbsLevelPassX[ xC ][ yC ] + = par_level_flag[ n ] |  |
| } |  |
| for( j = 2; j < 5; j++ ) { |  |
| if( abs_level_gtx_flag[ n ][ j − 1 ] ) |  |
| abs_level_gtx_flag[ n ][ j ] | ae(v) |
| MaxCcbs− − |  |
| AbsLevelPassX[ xC ][ yC ] + = 2 * abs_level_gtx_flag[ n ][ j ] |  |
| } |  |
| } |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 4 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| } |  |
| } |  |

## Draft Specification for Test 4

7.3.7.11	Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| if( ( i != lastSubBlock ¦ ¦ !inferSbCbf ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && i < lastSubBlock ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| for( n = 0; n <= numSbCoeff – 1 && MaxCcbs >= 4; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| MaxCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| MaxCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPassX[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| } |  |
| iFirstPassBypassPos = n; |  |
| /* Greater than X scan pass (numGtXFlags=5) */ |  |
| for( n = 0; n < iFirstPassBypassPos && MaxCcbs >= 4; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| for( j = 1; j < 5; j++ ) { |  |
| if( abs_level_gtx_flag[ n ][ j − 1 ] ) |  |
| abs_level_gtx_flag[ n ][ j ] | ae(v) |
| MaxCcbs− − |  |
| AbsLevelPassX[ xC ][ yC ] + = 2 * abs_level_gtx_flag[ n ][ j ] |  |
| } |  |
| } |  |
| iSecondPassBypassPos = n; |  |
| /* abs_remainder scan pass */ |  |
| for( n = 0; n < iFirstPassBypassPos ; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| baseLevel = (scanPos >= iSecondPassBypassPos ? 2 : 10); |  |
| if(AbsLevelPassX[ xC ][ yC ] >= baseLevel) |  |
| abs_remainder[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| /* dec_abs_level scan pass */ |  |
| for( n = iFirstPassBypassPos; n <= numSbCoeff – 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| sig_coeff_flag[ xC ][ yC ] |  |
| if(sig_coeff_flag[ xC ][ yC ]) |  |
| dec_abs_level[ n ] | ae(v) |
| coeff_sign_flag[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * (sig_coeff_flag[ xC ][ yC ] + dec_abs_level[ n ]) |  |
| } |  |
| } |  |
| } |  |

      - Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) 
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
if( yC < (1 << log2TbHeight) − 1 )	(9-11)
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
locSumAbs = Clip3( 0, 31, locSumAbs + 10 - baseLevel  )

- Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(9-12)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel [ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

The following applies:

- If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-12.

- Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-12.

Table 9-12 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s

| trafoSkip | s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| 1 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
|  | 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
|  | 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  |  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 0 |  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| 1 |  | cRiceParam | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
|  | 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

# References

F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “ JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March, 2019.