| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0621_r2 |
| --- | --- |

| Title: | Non-CE3: MIP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai 1-9-2, Nakase, Mihama-ku, Chiba-shi, Chiba, Japan | Tel: Email: | +81-50-5213-6336 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp |  |  |

_____________________________

- Abstract

This contribution proposes a method to reduce the table size in matrix intra prediction (MIP). In VTM 5, the weight matrices of MIP are implemented as tables that is about 7 KB in total. The proposed method derives a part of weight matrices using linear interpolation of neighbouring weight matrices, and reduces the table size by about 40%. The experimental result reportedly show the luma BD rate performance on average are 0.04%, 0.03% and 0.07% in AI, RA and LB conditions respectively.

Introduction

In VVC Draft 5, MIP uses the following table. Total size is 55040 bits, which takes large amount of storage space.

| sizeId | input | output | #Mode | # table | #weight per table | tblSize |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 4 | 4 | 35 | 18 | 64 | 11520 |
| 1 | 8 | 8 | 19 | 10 | 128 | 12800 |
| 2 | 16 | 16 | 11 | 6 | 512 | 30720 |
|  |  |  |  |  | total | 55040 |

On the our understanding,

- sizeId = 0 shows relatively small gain, which can be reduced.

- sizeId = 1 shows relatively large gain, it might be ok for relatively large table

- sizeId = 2 needs the largest table size, whose table should be reduced.

Proposal

Taking account for the above observation, we propose to reduce the table of sizeId = 0 and 2. Specifically, we only keep half of tables and derive other half of table (even mode numbers) using linear interpolation of the tables of neighboring modes.

As shown below, the table size is reduced by 38 % (55040 to 33920).

| sizeId | input | output | #Mode | # table | #weight per table | tblSize |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 4 | 4 | 35 | 9 | 64 | 5760 |
| 1 | 8 | 8 | 19 | 10 | 128 | 12800 |
| 2 | 16 | 16 | 11 | 3 | 512 | 15360 |
|  |  |  |  |  | total | 33920 |

The precision of all weight and bias values are adjusted to 8 bits (and 1 bit for sign) for simple interpolation. The type of the table elements is not changed.

Experimental results

The proposed method was implemented on top of the VTM 5.0[1] software and the experiments were conducted under the common test conditions [2].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 (Sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | -0.01% | 100% | 100% |
| Class A2 | 0.03% | 0.08% | 0.00% | 100% | 100% |
| Class B | 0.04% | -0.03% | 0.10% | 100% | 100% |
| Class C | 0.04% | 0.05% | 0.11% | 101% | 100% |
| Class E | 0.02% | -0.03% | -0.19% | 100% | 100% |
| Overall | 0.04% | 0.02% | 0.02% | 100% | 100% |
| Class D | 0.07% | 0.31% | 0.07% | 101% | 100% |
| Class F | 0.02% | 0.11% | 0.14% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 (Sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.30% | 0.08% | 100% | 101% |
| Class A2 | 0.03% | -0.04% | -0.01% | 100% | 101% |
| Class B | 0.04% | 0.01% | -0.27% | 100% | 100% |
| Class C | 0.01% | -0.06% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.08% | -0.08% | 100% | 100% |
| Class D | 0.06% | -0.23% | 0.10% | 100% | 100% |
| Class F | 0.04% | 0.02% | 0.12% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 (Sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.23% | -0.28% | 100% | 100% |
| Class C | 0.06% | 0.12% | -0.04% | 100% | 99% |
| Class E | 0.16% | -0.34% | 0.18% | 100% | 100% |
| Overall | 0.07% | 0.05% | -0.09% | 100% | 100% |
| Class D | 0.02% | 0.11% | 0.59% | 100% | 100% |
| Class F | 0.03% | 0.72% | -0.49% | 100% | 100% |

Conclusion

We propose to reduce MIP table size by deriving half of tables by interpolating weight of other tables. This applies to sizeId=0 and 2 table (not apply sizeId=1 table). This proposal reduces the table size by 38%. It is reported the BD-rate is 0.04%, 0.03 % and 0.07 % on average in AI, RA and LB conditions respectively.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Mar. 2019, Geneva, CH.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).