| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0625 |
| --- | --- |

| Title: | Non-CE5: Padding method for samples at variant boundaries in ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Na Zhang, Jizheng Xu Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, ALF virtual boundary is not applied to CTUs with their bottom boundary are slice/brick boundaries. Disabling the application of ALF virtual boundary may create pipeline bubble or require processing more lines. In addition, no padding is performed for samples at the slice/brick boundaries even if the ALF operations are disallowed to across the slice/brick boundaries. Moreover, there are variant boundaries (including ALF virtual boundaries, slice/brick boundaries, 360-degree video virtual boundaries) defined and multiple padding methods are employed for handling those boundaries. This contribution proposes to two aspects. Firstly, the ALF virtual boundary is applicable to CUs with bottom boundaries are slice/brick/virtual boundaries regardless the usage of filtering crossing boundaries. Secondly, on top of the first aspect, it is further proposed to use the symmetric 2-side padding method used by ALF virtual boundaries to pad samples outside those horizontal boundaries in the ALF process.

# Introduction

Three issues related to boundary handling are identified in the ALF process according to the latest VVC working draft [1].

Firstly, ALF virtual boundaries are added to a CTU except one of the following conditions are true:

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture.

  - The bottom boundary of the current coding tree block is the bottom boundary of the brick and loop_filter_across_bricks_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is the bottom boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is one of the bottom virtual boundaries of the picture and pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1.

Therefore, the ALF virtual boundary (VB) is disabled for CTUs at slice/brick/360-degree video virtual boundaries. Disabling the ALF VB along those boundaries would create pipeline bubble or require processing more lines per Virtual Pipeline Data Units (VPDU). Since the decoders may not be able to know the slice/brick/tile boundaries upfront, the ALF line buffers need to be restored.

Secondly, according to the current specification, no padding is performed for samples at the slice/brick boundaries, i.e., samples in a different slice/brick may be used in ALF process of the current slice/brick, even if the ALF operations are not be allowed to across the slice/brick boundaries.

Thirdly, different padding methods are used for different boundaries. for samples at the 360-degree virtual boundaries, a 1-side padding method is used. On the other hand, for samples at the ALF virtual boundaries, a symmetric 2-side padding method [2] is used. An example is shown in Figure 1.

Figure 1. Different padding methods in ALF for luma samples at different boundaries.

# Proposed method

To solve the abovementioned problem in ALF, this contribution proposes two aspects which will be described in the following sub-sections.

## First proposed aspect

ALF virtual boundary is applicable to CUs with bottom boundaries are slice/brick/virtual boundaries regardless the usage of samples across boundaries.

The proposed changes to the ALF filtering process are as follows:

8.8.5.2	Coding tree block filtering process for luma samples

…

  - The variable applyVirtualBoundary is derived as follows:

  - If one or more of the following conditions are true, applyVirtualBoundary is set equal to 0:

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture.

  - The bottom boundary of the current coding tree block is the bottom boundary of the brick and loop_filter_across_bricks_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is the bottom boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is one of the bottom virtual boundaries of the picture and pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1.

  - Otherwise, applyVirtualBoundary is set equal to 1.

  - The reconstructed sample offsets r1, r2 and r3 are specified in Table 8-22 according to the horizontal luma sample position y and applyVirtualBoundary.

  - The variable curr is derived as follows:

…

## Second proposed aspect

It is proposed that the symmetric 2-side padding method is applied to samples at all horizontal boundaries in ALF.

In the symmetric 2-side padding method, when a sample is at two boundaries, e.g., one boundary in the above side and one boundary in the below side, how many samples are padded is decided by the nearer boundary as shown in Figure 2. Meanwhile, when deriving the classification information, only the 4 lines between the two boundaries in Figure 2 are used.

Figure 2. Padded neighboring samples in ALF for luma samples at two boundaries.

The proposed changes to the working draft could refer to the attached document.

# Experimental results

Two tests were performed to evaluate the proposed method.

- Test #1: the proposed method was implemented on top of the reference software VTM-5.0 [2], and simulations were performed following the common test conditions [3]. Meanwhile, a picture is split into two slices. Each slice contains at most ((W + 127)/128) * ((H + 127)/128 + 1)/2 CTUs, wherein W and H are the picture width and height respectively. The results are shown in Table 1 with the anchor using VTM5.0 and two slices coding enabled.

- Test #2: the proposed method was implemented on top of the master branch [5] (SHA-1: 8b33ef53c5225cdedf87850bd977ce05cbf846ad) of the reference software on top of which the concept of 360-degree virtual boundaries is integrated. Simulations were performed following the common test conditions [6] of 360-degree video.

Table 1: Coding performance of Test #1.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.08% | 0.07% | 102% | 101% | 0.01% | 0.06% | -0.01% | 99% | 100% |
| Class A2 | 0.02% | 0.07% | 0.04% | 101% | 101% | 0.01% | 0.07% | 0.13% | 102% | 103% |
| Class B | 0.03% | 0.06% | 0.08% | 98% | 100% | 0.05% | -0.01% | -0.03% | 100% | 101% |
| Class C | 0.05% | 0.10% | 0.14% | 99% | 98% | 0.07% | 0.20% | 0.00% | 101% | 102% |
| Class E | 0.10% | 0.07% | 0.17% | 100% | 101% |  |  |  |  |  |
| Overall | 0.05% | 0.08% | 0.10% | 100% | 100% | 0.04% | 0.07% | 0.01% | 100% | 101% |
| Class D | 0.07% | 0.19% | 0.30% | 100% | 100% | 0.11% | 0.03% | 0.01% | 100% | 102% |
| Class F | 0.04% | 0.07% | 0.06% | 100% | 101% | 0.04% | 0.03% | 0.06% | 100% | 101% |

# Conclusions

This contribution proposes two aspects to unify the ALF filtering process for samples at variant boundaries. It is recommended to adopt the proposed methods into VVC.

# References

- “Versatile Video Coding (Draft 5)”, B. Bross, J. Chen, S. Liu, Doc. JVET-N1001, Mar. 2019.

- “CE5-2: Loop filter line buffer reduction	”, A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao, Doc. JVET-N0180, Mar. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-N1010, Mar. 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.

- “JVET common test conditions and evaluation procedures for 360° video”, P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, Doc. JVET-L1012.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).