| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0627-v3 |
| --- | --- |

| Title: | CE2-related: luma-chroma dependency reduction on chroma scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ruling Liao, Yan Ye, Jiancong Luo Radiance Builidng Beijing China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

At the 14th JVET meeting, several contributions proposed to simplify the chroma scaling factor derivation to reduce the dependency between luma component and chroma component, and a CE was set up to further study it. This contribution proposes to use CTU neighboring reconstructed samples to derive chroma scaling factor to solve the dependency issue. It is reported that the performance impact for {Y, U, V} is {0.00%, -0.07%, -0.06%} for RA and {0.02%, 0.00%, -0.20%} for LDB in the case of using 4 left and 4 up neighbors, and {0.00%, -0.09%, -0.13%} for RA and {-0.01%, -0.07%, -0.19%} for LDB in the case of using 16 left and 16 up neighbors if the CTU-level derivation method is applied only in case of single tree. And by additionally applying the proposed method in the case of dual tree, and enabling the chroma scaling in dual tree, the total coding performance is {0.05%, -0.49%, -0.65%} for AI, {0.03%, -0.65%, -0.66%} for RA and {-0.02%, -0.59%, -0.51%} for LDB in the case of using 4 left and 4 up neighbors, and {0.04%, -0.43%, -0.59%} for AI, {0.03%, -0.67%, -0.67%} for RA and {0.02%, -0.47%, -0.53%} for LDB in the case of using 16 left and 16 up neighbors.

In the v3 revision, the proposed method is tested on top of CRS lambda quantization bug fixed anchor which is proposed in JVET-O0429. And it is reported that by combining with bug fixing, the coding performance impact for {Y, U, V} is {0.06%, -0.67%, -0.83%} for AI, {0.08%, -1.32%, -1.26%} for RA and {0.02%, -0.59%, -0.89%} for LDB in the case of using 4 left and 4 up neighbors, {0.05%, -0.53%, -0.76%} for AI, {0.07%, -1.27%, -1.25%} for RA and {-0.01%, -0.61%, -0.74%} for LDB in the case of using 16 left and 16 up neighbors, if both apply to single tree and dual tree.

Chroma scaling in VVC draft 5

Chroma residual scaling depends on the average value of the corresponding luma prediction block. In VVC draft 5, the average value of the luma prediction block, avgY, is computed using all samples in the luma prediction block. Once the average luma value is determined, the chroma scaling factor is computed by finding the piece index of the piecewise linear model to which avgY belongs. So chroma residue scaling cannot start until the prediction of the corresponding luma block is finished, which causes latency issue between luma and chroma. Especially in the dual tree case, the latency issue is more serious since the luma partitioning and chorma partitioning is not guaranteed to be aligned. In the worst case, chroma scaling has to wait until the prediction of an entire 64x64 luma block is finished. Therefore, in VVC draft 5, chroma scaling is disabled if dual tree partition is used.

Proposal

It is proposed to derive chroma scaling factor at the CTU level instead of at the chorma block level. For each CTU, the left and top neighboring reconstructed luma samples are used. First, the average value of M left neighboring luma samples and M top neighboring luma samples which are the closest to the top-left position, as shown in Figure 1, avgY, is calculated. And then the same method in VVC draft 5 is used to derive the chroma scaling factor based on avgY. The factor derived on CTU level is used for all the chroma blocks in this CTU. So there is no additional dependency between luma and chroma when decoding the current CTU.

Figure 1. CTU neighboring samples used in chroma scaling factor derivation

On the picture boundary, for the rightmost column CTU, if the upper neighboring samples within the picture is less than M, the picture boundary sample is padded until M; for the bottom row CTU, if the left neighboring samples within the picture is less than M, the picture boundary samples is padded until M. By doing it, the number of averaged samples is always 2M where M is set as power of 2, so division operation is avoided in this contribution.

Simulation results

The proposed modifications are implemented on top of VTM5.0. Simulation results of the M being 4 and M being 16 are summarized in the following sections. Both single tree and dual tree cases are tested, and in the dual tree test case, the anchor VTM5.0 disables chroma scaling in the case of dual tree, but  whereas we enable chroma scaling with the proposed method.

In the case of M being 4, the overall impact for {Y, U, V} is

- Single tree only: {0.00%, -0.07%, -0.06%} for RA and {0.02%, 0.00%, -0.20%} for LDB

- Both single and dual trees: {0.05%, -0.49%, -0.65%} for AI, {0.03%, -0.65%, -0.66%} for RA and {-0.02%, -0.59%, -0.51%} for LDB.

In the case of M being 16, the overall impact for {Y, U, V} is

- Single tree only: {0.00%, -0.09%, -0.13%} for RA and {-0.01%, -0.07%, -0.19%} for LDB

- Both single and dual trees: {0.04%, -0.43%, -0.59%} for AI, {0.03%, -0.67%, -0.67%} for RA and {0.02%, -0.47%, -0.53%} for LDB.

Additionally, a lambda quantization bug for CRS is fixed in JVET-O0429, this contribution also integrates the bug fixing to the proposed method. By combining the bug fixing, the coding performance impact is provided in this section as well.

In the case of M being 4, if apply the proposed method both to single tree and dual tree, the overall impact for {Y, U, V} is {0.06%, -0.67%, -0.83%} for AI, {0.08%, -1.32%, -1.26%} for RA and {0.02%, -0.59%, -0.89%} for LDB if both apply to single tree and dual tree.

In the case of M being 16, if apply the porsed method both to single tree and dual tree, the overall impact for {Y, U, V} is {0.05%, -0.53%, -0.76%} for AI, {0.07%, -1.27%, -1.25%} for RA and {-0.01%, -0.61%, -0.74%} for LDB if both apply to single tree and dual tree.

Using 4 top and 4 left neighbors (M=4)

The simulation results of M being 4 are summarized in this section. Table 1 give the coding performance impact if we only apply the method in single tree (B slice). Table 2 give the coding performance impact if we apply method both in single tree and dual tree (all slice). The anchor in both tables are VTM5.0.

- Single tree

Table 1. Simulation results of single tree (M=4)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.03% | 99% | 99% |
| Class A2 | 0.03% | -0.03% | 0.01% | 99% | 99% |
| Class B | 0.01% | -0.08% | -0.04% | 99% | 99% |
| Class C | -0.03% | -0.10% | -0.18% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.06% | 99% | 99% |
| Class D | 0.00% | 0.11% | 0.17% | 99% | 95% |
| Class F | 0.00% | 0.02% | 0.02% | 99% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.04% | -0.41% | 99% | 99% |
| Class C | 0.06% | -0.24% | 0.05% | 99% | 98% |
| Class E | -0.04% | 0.38% | -0.17% | 99% | 99% |
| Overall | 0.02% | 0.00% | -0.20% | 99% | 98% |
| Class D | -0.02% | -0.15% | 0.16% | 99% | 98% |
| Class F | 0.05% | -0.06% | -0.17% | 100% | 97% |

- Single tree + dual tree

Table 2. Simulation results of single tree + dual tree (M=4)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.42% | 0.24% | 100% | 99% |
| Class A2 | -0.11% | 0.99% | 1.09% | 99% | 99% |
| Class B | 0.05% | -0.35% | -0.48% | 99% | 98% |
| Class C | 0.20% | -1.54% | -1.71% | 101% | 98% |
| Class E | 0.16% | -1.74% | -2.14% | 100% | 98% |
| Overall | 0.05% | -0.49% | -0.65% | 100% | 98% |
| Class D | 0.13% | -0.93% | -1.16% | 99% | 99% |
| Class F | 0.32% | -1.50% | -1.65% | 101% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.03% | 99% | 99% |
| Class A2 | 0.03% | -0.03% | 0.01% | 99% | 99% |
| Class B | 0.01% | -0.52% | -0.50% | 98% | 98% |
| Class C | 0.08% | -1.71% | -1.86% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.65% | -0.66% | 99% | 97% |
| Class D | 0.07% | -1.28% | -0.94% | 99% | 92% |
| Class F | 0.27% | -1.66% | -1.75% | 100% | 97% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.11% | -0.38% | 99% | 98% |
| Class C | -0.02% | -0.31% | -0.42% | 99% | 97% |
| Class E | -0.09% | -1.75% | -0.85% | 99% | 98% |
| Overall | -0.02% | -0.59% | -0.51% | 99% | 98% |
| Class D | 0.01% | -0.49% | 0.69% | 100% | 99% |
| Class F | 0.09% | -0.76% | -0.69% | 99% | 98% |

Using 16 top and 16 left neighbors (M=16)

The simulation results of M being 16 are summarized in this section. Table 3 give the coding performance impact if we only apply the method in single tree (B slice). Table 4 give the coding performance impact if we apply method both in single tree and dual tree (all slice). The anchor in both tables are VTM5.0.

- Single tree

Table 3. Simulation results of single tree (M=16)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | -0.02% | -0.06% | 100% | 100% |
| Class B | 0.01% | -0.10% | -0.17% | 99% | 100% |
| Class C | -0.02% | -0.15% | -0.26% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.13% | 99% | 99% |
| Class D | 0.03% | -0.06% | 0.23% | 99% | 99% |
| Class F | 0.02% | 0.04% | -0.03% | 99% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.05% | -0.13% | 100% | 100% |
| Class C | 0.02% | -0.02% | 0.13% | 99% | 99% |
| Class E | -0.07% | -0.15% | -0.71% | 99% | 100% |
| Overall | -0.01% | -0.07% | -0.19% | 100% | 100% |
| Class D | -0.01% | -0.20% | 1.14% | 99% | 99% |
| Class F | 0.01% | 0.01% | 0.17% | 100% | 99% |

- Single tree + dual tree

Table 4. Simulation results of single tree + dual tree (M=16)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.54% | 0.32% | 99% | 100% |
| Class A2 | -0.12% | 1.07% | 1.15% | 100% | 100% |
| Class B | 0.04% | -0.27% | -0.37% | 100% | 100% |
| Class C | 0.20% | -1.62% | -1.73% | 101% | 100% |
| Class E | 0.16% | -1.59% | -2.10% | 100% | 100% |
| Overall | 0.04% | -0.43% | -0.59% | 100% | 100% |
| Class D | 0.14% | -0.87% | -1.14% | 100% | 99% |
| Class F | 0.31% | -1.53% | -1.61% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | -0.02% | -0.06% | 100% | 100% |
| Class B | 0.02% | -0.49% | -0.61% | 99% | 100% |
| Class C | 0.07% | -1.85% | -1.73% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.67% | -0.67% | 99% | 100% |
| Class D | 0.07% | -1.34% | -1.12% | 99% | 98% |
| Class F | 0.27% | -1.69% | -1.67% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.09% | -0.31% | 100% | 100% |
| Class C | 0.02% | -0.38% | -0.46% | 100% | 99% |
| Class E | 0.04% | -1.24% | -0.99% | 99% | 100% |
| Overall | 0.02% | -0.47% | -0.53% | 99% | 100% |
| Class D | -0.02% | -0.80% | 0.45% | 99% | 99% |
| Class F | 0.12% | -0.76% | -0.62% | 100% | 99% |

Results with bug fixing from JVET-O0429

In this section, the CRS lambda quantization bug fixing in JVET-O0429 is integrated to the proposed method. And the coding results are summarized in Table 5 and Table 6.

- M being 4, single tree + dual tree, bug fix

Table 5. Simulation results of single tree + dual tree (M=4)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.72% | 0.57% | 99% | 100% |
| Class A2 | -0.16% | 1.57% | 1.83% | 99% | 100% |
| Class B | 0.07% | -0.44% | -0.65% | 99% | 100% |
| Class C | 0.27% | -2.25% | -2.37% | 100% | 99% |
| Class E | 0.23% | -2.59% | -3.13% | 100% | 100% |
| Overall | 0.06% | -0.67% | -0.83% | 100% | 100% |
| Class D | 0.17% | -1.30% | -1.55% | 99% | 98% |
| Class F | 0.48% | -2.11% | -2.32% | 100% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.93% | -0.49% | 99% | 100% |
| Class A2 | 0.03% | -0.12% | -0.20% | 100% | 100% |
| Class B | 0.05% | -1.03% | -1.05% | 99% | 100% |
| Class C | 0.15% | -2.87% | -2.89% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -1.32% | -1.26% | 99% | 99% |
| Class D | 0.14% | -1.60% | -2.20% | 99% | 97% |
| Class F | 0.44% | -2.58% | -2.50% | 99% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.06% | -0.46% | 100% | 99% |
| Class C | 0.04% | -0.55% | -0.70% | 99% | 99% |
| Class E | -0.01% | -1.75% | -1.86% | 99% | 100% |
| Overall | 0.02% | -0.59% | -0.89% | 99% | 99% |
| Class D | 0.06% | -0.20% | 0.59% | 99% | 98% |
| Class F | 0.10% | -1.06% | -0.75% | 99% | 98% |

- M being 16, single tree + dual tree, bug fix

Table 6. Simulation results of single tree + dual tree (M=16)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.96% | 0.64% | 100% | 100% |
| Class A2 | -0.18% | 1.70% | 1.90% | 100% | 100% |
| Class B | 0.06% | -0.31% | -0.49% | 100% | 100% |
| Class C | 0.27% | -2.21% | -2.40% | 101% | 100% |
| Class E | 0.21% | -2.37% | -3.10% | 100% | 100% |
| Overall | 0.05% | -0.53% | -0.76% | 100% | 100% |
| Class D | 0.16% | -1.19% | -1.47% | 100% | 98% |
| Class F | 0.46% | -2.12% | -2.25% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.96% | -0.49% | 99% | 100% |
| Class A2 | 0.03% | -0.14% | -0.20% | 100% | 100% |
| Class B | 0.04% | -0.88% | -1.00% | 99% | 100% |
| Class C | 0.14% | -2.86% | -2.93% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -1.27% | -1.25% | 99% | 100% |
| Class D | 0.15% | -1.86% | -2.18% | 99% | 98% |
| Class F | 0.40% | -2.45% | -2.45% | 99% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.03% | -0.77% | 100% | 100% |
| Class C | 0.01% | -0.68% | -0.46% | 99% | 99% |
| Class E | -0.10% | -1.58% | -1.08% | 99% | 99% |
| Overall | -0.01% | -0.61% | -0.74% | 99% | 99% |
| Class D | 0.08% | -0.79% | 0.68% | 98% | 98% |
| Class F | 0.10% | -0.88% | -0.96% | 99% | 99% |

Additional results with modified anchor

In this section, we enable chroma residue scaling in dual tree on top of VTM5.0 and use this version of software as the anchor to check the coding performance impact of scaling factor derivation method itself.

- M being 4

Table 7. simulation results with modified anchor (M being 4)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.20% | -0.17% | 100% | 99% |
| Class A2 | 0.04% | -0.21% | -0.25% | 100% | 99% |
| Class B | 0.03% | -0.35% | -0.31% | 100% | 98% |
| Class C | 0.02% | 0.04% | 0.01% | 100% | 98% |
| Class E | 0.03% | -0.29% | -0.30% | 99% | 98% |
| Overall | 0.03% | -0.21% | -0.21% | 100% | 98% |
| Class D | 0.00% | -0.08% | 0.08% | 100% | 100% |
| Class F | 0.04% | -0.14% | -0.12% | 101% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.03% | 99% | 100% |
| Class A2 | 0.03% | -0.03% | 0.01% | 100% | 100% |
| Class B | 0.01% | -0.13% | -0.13% | 99% | 98% |
| Class C | 0.00% | 0.13% | -0.03% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.04% | 99% | 98% |
| Class D | -0.02% | 0.13% | 0.31% | 100% | 93% |
| Class F | 0.09% | -0.10% | -0.21% | 100% | 98% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.28% | -0.06% | 99% | 99% |
| Class C | -0.02% | -0.26% | 0.14% | 100% | 98% |
| Class E | -0.17% | 0.07% | 0.26% | 100% | 98% |
| Overall | -0.04% | -0.19% | 0.09% | 100% | 98% |
| Class D | -0.02% | -0.24% | 0.44% | 101% | 100% |
| Class F | 0.07% | -0.27% | -0.35% | 100% | 100% |

- M being 16

Table 8. simulation results with modified anchor (M being 16)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | -0.09% | 100% | 100% |
| Class A2 | 0.04% | -0.13% | -0.20% | 100% | 100% |
| Class B | 0.02% | -0.28% | -0.20% | 100% | 100% |
| Class C | 0.02% | -0.04% | -0.01% | 100% | 100% |
| Class E | 0.02% | -0.14% | -0.26% | 100% | 100% |
| Overall | 0.02% | -0.14% | -0.15% | 100% | 100% |
| Class D | 0.01% | -0.03% | 0.09% | 101% | 100% |
| Class F | 0.03% | -0.16% | -0.08% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.06% | 100% | 100% |
| Class B | 0.01% | -0.10% | -0.24% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.06% | 100% | 100% |
| Class D | -0.02% | 0.07% | 0.13% | 100% | 99% |
| Class F | 0.09% | -0.13% | -0.12% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over (VTM-5.0+CS in dual tree) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.26% | 0.01% | 100% | 101% |
| Class C | 0.02% | -0.32% | 0.09% | 100% | 100% |
| Class E | -0.04% | 0.58% | 0.14% | 100% | 100% |
| Overall | 0.00% | -0.07% | 0.07% | 100% | 100% |
| Class D | -0.05% | -0.55% | 0.21% | 100% | 100% |
| Class F | 0.09% | -0.26% | -0.28% | 100% | 101% |

Conclusion

By using CTU level derivation, the latency between luma and chroma decoding is removed. According to the experiments, this modification has ignorable coding performance impact. Thus, it is suggested to adopt it in the next version of VVC draft.

The modification of the spec.

#### 8.7.5.3 Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the transform block width,

- a variable nCurrSh specifying the transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

Output of this process is a reconstructed chroma picture sample array recSamples.

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one of the following conditions is true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1C( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- slice_chroma_residual_scale_flag is equal to 0

- nCurrSw * nCurrSh is less than or euqal to 4

- tu_cbf_cb [ xCurr ][ yCurr ] is equal to 0 and tu_cbf_cr [ xCurr ][ yCurr ] is equal to 0

- Otherwise, the following applies:

- For the derivation of the variable varScale the following ordered steps apply:

- The  variable xCtu and yCtu is derived as follows:

xCtu=(xCurr*SubWidthC)/ CtbSizeY* CtbSizeY

yCtu=(yCurr*SubHeightC / CtbSizeY* CtbSizeY

- The array recLuma[i] with i=0,1,2,3 and the variable cnt are derived as follows:

- cnt is set to 0

- when xCtu is larger than 0, the luma sample at the location ( xCtu-1, yCtu ) is assigned to recLuma[0], the luma sample at the location (  xCtu-1, yCtu+1  ) is assigned to recLuma[1], the luma sample at the location ( xCtu-1, yCtu+2 ) is assigned to recLuma[2], the luma sample at the location ( xCtu-1, yCtu +3) is assigned to recLuma[3], cnt is set to 4

- when xCtu is larger than 0, the luma sample at the location ( xCtu, yCtu-1 ) is assigned to recLuma[cnt+0], the luma sample at the location (  xCtu+1, yCtu-1  ) is assigned to recLuma[cnt+1], the luma sample at the location ( xCtu+2, yCtu-1 ) is assigned to recLuma[cnt+2], the luma sample at the location ( xCtu+3, yCtu -1) is assigned to recLuma[cnt+3], cnt is set to cnt+4

- The variable invAvgLuma is derived as follows:

-

if(cnt>0)

invAvgLuma= Clip1Y( )

else

invAvgLuma=1  <<  ( bitDepth − 1 )

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with the variable lumaSample set equal to invAvgLuma as the input and idxYInv as the output.

- The variable varScale is derived as follows:

varScale = ChromaScaleCoeff[ idxYInv ]	(8-997)

- The recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepthC ), ( 1 << BitDepthC ) − 1, resSamples[ i ][ j ] )	(8-998)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C( predSamples[ i ][ j ] + 	(8-999)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tu_cbf is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C(predSamples[ i ][ j ] )

- Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).