| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0628-v1 |
| --- | --- |

| Title: | Non-CE9: SATD based DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ru-Ling Liao, Yan Ye, Jiancong Luo Radiance Building Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In VVC draft 5, DMVR searches for the refined motion vectors by minimizing the SAD of two predictors at the decoder side. In this contribution, it is proposed to use SATD instead of SAD as the criterion to refine the motion vectors. The refinement process itself is unchanged in this contribution. It is reported that, under the CTC, the overall performance impact achieves -0.2%(Y) for RA using the first proposed method.

Introduction

In VVC draft 5, DMVR refines the motion vectors in merge mode or combined inter and intra prediction mode at the decoder side. In DMVR, refined MVs are searched around the initial MVs in the reference picture list L0 and reference picture list L1. The SAD of the two predictors of L1 and L0 are calculated during search process. The position with minimum SAD in the search window is chosen as the best integer position. An error surface is estimated based on the best integer position and fractional refinement is conducted based on the error surface.

Proposed method

SAD is used in the current DMVR design as an estimator of coding cost. To improve the accuracy of coding cost estimation, it is proposed to use SATD as a criterion in the searching process of DMVR. The position with minimum SATD in the search window is chosen as the best integer position. The search process and the fractional refinement process are not changed in this contribution.

Three methods are proposed in the contribution.

In the first method, the DMVR block (16x16/16x8/8x16 in VTM5.0) is split into 4x4 sub-blocks. And 4x4 Hadamard transform is applied on each sub-block. After primary Hadamard transform, a secondary Hadamard transform is applied on the DC coefficient of each 4x4 sub-block. The procedure is shown in Figure 1. Compared with directly applying Hadamard transform on the whole DMVR block, splitting into small sub-blocks and applying smaller Hadamard transform reduces the computation complexity.

Figure 1. Method 1

To reduce the complexity of method 1, in method 2, a vertical 2:1 downsampling is performed before primary Hadamard transform, and the secondary transform is only applied to the DMVR block whose original size is 16x16. The procedure is shown in Figure 2. In this contribution, the vertical downsampling is performed using decimation without additional filtering.

Figure 2. Method 2

To further reduce the complexity, in method 3, the secondary transform is removed on top of method 2. So a vertical 2:1 downsampling is performance first, and then the downsampled block is split into 4x4 sub-block to perform 4x4 Hadamard transform. The procedure is shown in Figure 3.

Figure 3. Method 3

.

Simulation results

The proposed methods are implemented on top of VTM-5.0. The results are listed in the following tables. Table 1 gives the coding results of method 1. It gives {} coding gain with about 4% increase of decoding time. Table 2 gives the coding results of method 2. It gives {} coding gain with about 2% increase of decoding time. Table 3 gives the coding results of method 3. It gives {} coding gain with about1% increase of decoding time.

Table 1. Summary results of method 1

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.20% | -0.16% | -0.16% | 100% | 105% |
| Class C | -0.17% | -0.21% | -0.26% | 99% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.22% | -0.21% | #NUM! | #NUM! |
| Class D | -0.11% | -0.19% | -0.03% | 100% | 104% |
| Class F | -0.12% | -0.09% | -0.09% | 99% | 103% |

Table 2. Summary results of method 2

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.15% | -0.12% | -0.13% | 99% | 102% |
| Class C | -0.16% | -0.13% | -0.14% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.05% | 0.03% | 99% | 100% |
| Class F | -0.10% | -0.04% | -0.03% | 99% | 101% |

Table 3. Summary results of method 3

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.12% | -0.09% | -0.09% | 98% | 101% |
| Class C | -0.13% | -0.11% | -0.23% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | -0.05% | 0.01% | 99% | 99% |
| Class F | -0.08% | -0.04% | -0.05% | 99% | 100% |

Conclusion

In this contribution, it is proposed to use SATD as a criterion to refine the motion vector in DMVR process. It is reported that using the proposed method 1, -0.2% average coding gain is obtained. Thus, it is suggested to consider adoption of SATD based DMVR in VVC standard.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).