| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0629-v1 |
| --- | --- |

| Title: | Non-CE4: Simplification of motion field for triangle partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Jie Chen Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In this contribution, the motion field storage for triangle partition mode is simplified. Firstly, 8x8 subblock is used to store motion instead of 4x4 subblock in VVC draft 5. Secondly, the bi-prediction motion generation process for subblocks along the diagonal/anti-diagonal is modified: when both of the uni-prediction motion vectors of the two triangle partitions are from the same reference picture list, the process of converting one of them to another reference picture list is removed. Instead, the motion vector of the bottom triangle partition is stored. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.03%, 0.04%, 0.00%} for RA and {0.04%, 0.17%, -0.05%} for LDB when 8x8 subblock is used. {0.00%, -0.05%, -0.02%} for RA and {0.09%, 0.02%, -0.38%} for LDB when the motion vector of bottom triangle partition is used. {0.02%, -0.02%, 0.01%} for RA and {0.09%, 0.08%, 0.06%} for LDB when both simplifications are applied.

Introduction

In VVC draft 5, the motion field for triangle partition mode (TPM) is stored in 4x4 subblock units. Depending on the position of each 4x4 subblock, either uni-prediction or bi-prediction motion vectors are stored. As shown in Figure 1, if a 4x4 subblock is located in the uni-predicted area, a uni-prediction motion vector is stored. Otherwise, bi-prediction motion vectors are generated and stored for a 4x4 subblock located in the bi-predicted area.

Figure 1. An illustration of 4x4 motion field for TPM

The bi-prediction motion is generated using the following process:

- Denote the motion vectors of top and bottom triangle partitions as Mv1 and Mv2, respectively. If Mv1 and Mv2 are from different reference picture lists, Mv1 and Mv2 are simply combined to form the bi-prediction motion.

- Otherwise, if Mv1 and Mv2 are from the same list, and without loss of generality, assume they are both from L0. In this case,

  - If the reference picture of either Mv2 (or Mv1) appears in L1, then that Mv2 (or Mv1) is converted to a L1 motion vector using that reference picture in L1. Then the two motion vectors are combined to form the bi-prediction motion vector;

  - Otherwise, instead of bi-prediction motion, only uni-prediction motion Mv1 is stored.

There are two complicated processes in this design. Firstly, for each 4x4 subblock, it may require two times comparison when storing the motion vectors. One comparison is to determine whether the subblock is located in the bi-predicted area. The other comparison is to determine whether the subblock is located in the top-right (diagonal split) or top-left (anti-diagonal split) uni-predicted area. In total, 2016 comparisons are needed for a 128x128 block predicted using TPM. Secondly, in the bi-prediction generation process, the reference pictures of uni-prediction motion vectors are examined to determine if the reference pictures appear in another reference picture list. That means, in the worst case, the reference pictures of the uni-prediction motion vectors need to be compared with all the reference pictures in the list.

Proposed methods

To simply the motion filed storage for TPM, two methods are proposed.

Firstly, the subblock size for motion storage is enlarged from 4x4 to 8x8, as shown in Figure 2. In total, only 496 times comparison are needed for a 128x128 block predicted using TPM. That means, roughly 75% of comparison is reduced as compared to the VVC draft 5.

Secondly, the process of bi-prediction motion vector generation is simplified. Specifically, the process of converting one uni-prediction motion vector to another reference picture list is removed from the bi-prediction generation process. Instead, the motion vector of bottom triangle partition is used when both of the two uni-prediction motion vectors are from the same reference picture list. That is, the bi-prediction motion vector is generated using the following process:

- Denote the motion vector of top and bottom triangle partitions as Mv1 and Mv2, respectively. If Mv1 and Mv2 are from different reference picture lists, Mv1 and Mv2 are simply combined to form the bi-prediction motion vector.

- Otherwise, if Mv1 and Mv2 are from the same list, only uni-prediction motion Mv2 is stored.

Figure 2. An illustration of 8x8 motion field for TPM

Simulation results

The proposed modification is implemented on top of VTM-5.0. Three tests are evaluated:

Test 1: 8x8 motion field

Test 2: Simplified bi-prediction motion vector generation

Test 3: combination of test 1 and test 2

The results showed that there is negligible impact on coding performance in random access condition for all three tests. For low delay B condition, 0.04% to 0.09% coding performance loss is observed. Besides, there is no impact on both the encoding time and decoding time.

Table 1. Summary results of all tests

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TPM off | 0.35% | 0.63% | 0.67% | 92% | 100% |
| Test 1 | 0.03% | 0.04% | 0.00% | 99% | 99% |
| Test 2 | 0.00% | -0.05% | -0.02% | 99% | 100% |
| Test 3 | 0.02% | -0.02% | 0.01% | 99% | 99% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TPM off | 0.88% | 1.34% | 1.02% | 87% | 101% |
| Test 1 | 0.04% | 0.17% | -0.05% | 100% | 99% |
| Test 2 | 0.07% | 0.26% | 0.05% | 99% | 99% |
| Test 3 | 0.09% | 0.08% | 0.06% | 99% | 99% |

Table 2. 8x8 motion field

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.12% | 0.05% | 100% | 99% |
| Class A2 | 0.02% | 0.14% | 0.06% | 100% | 99% |
| Class B | 0.02% | 0.05% | 0.01% | 99% | 99% |
| Class C | 0.05% | 0.08% | -0.10% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.00% | 99% | 99% |
| Class D | 0.03% | 0.10% | 0.28% | 99% | 98% |
| Class F | 0.06% | 0.04% | 0.07% | 99% | 99% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.32% | 0.19% | 100% | 100% |
| Class C | 0.05% | 0.12% | -0.23% | 100% | 99% |
| Class E | 0.05% | -0.01% | -0.20% | 99% | 98% |
| Overall | 0.04% | 0.17% | -0.05% | 100% | 99% |
| Class D | 0.05% | -0.32% | -0.14% | 99% | 98% |
| Class F | 0.06% | 0.11% | -0.10% | 99% | 98% |

Table 3. Simplified bi-prediction motion vector generation

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | 0.07% | 99% | 100% |
| Class A2 | 0.02% | 0.06% | 0.05% | 99% | 100% |
| Class B | 0.02% | -0.06% | 0.00% | 99% | 100% |
| Class C | -0.03% | -0.02% | -0.16% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.02% | 99% | 100% |
| Class D | -0.02% | 0.07% | 0.08% | 99% | 99% |
| Class F | 0.00% | 0.03% | 0.00% | 99% | 100% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.39% | 0.06% | 99% | 99% |
| Class C | 0.09% | 0.09% | 0.21% | 99% | 99% |
| Class E | 0.03% | 0.28% | -0.19% | 99% | 99% |
| Overall | 0.07% | 0.26% | 0.05% | 99% | 99% |
| Class D | 0.12% | -0.02% | 1.33% | 99% | 99% |
| Class F | 0.09% | 0.02% | -0.38% | 99% | 99% |

Table 4. Combination of Test 1 and Test 2

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.18% | 99% | 99% |
| Class A2 | 0.02% | -0.04% | 0.01% | 99% | 99% |
| Class B | 0.02% | 0.05% | -0.04% | 99% | 99% |
| Class C | 0.01% | -0.03% | -0.06% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | 0.01% | 99% | 99% |
| Class D | 0.02% | 0.14% | 0.36% | 99% | 98% |
| Class F | 0.02% | 0.05% | 0.06% | 99% | 99% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.23% | 0.58% | 99% | 99% |
| Class C | 0.14% | 0.06% | -0.09% | 99% | 98% |
| Class E | 0.06% | -0.13% | -0.59% | 99% | 98% |
| Overall | 0.09% | 0.08% | 0.06% | 99% | 99% |
| Class D | 0.10% | 0.12% | 0.53% | 99% | 98% |
| Class F | 0.06% | -0.14% | 0.67% | 99% | 98% |

Conclusion

In this contribution, two methods are proposed to simplify the motion storage process for TPM: 1) 8x8 subblock motion field, and 2) simplified bi-prediction motion vector generation. Compared to VTM-5.0, less than 0.1% coding performance loss is observed when the proposed methods are applied. The proposed method 1 can reduce TPM motion storage complexity by removing roughly 75% of comparisons for a 128x128 block predicted using TPM, and the proposed method 2 can remove the need to examine reference pictures for the bi-predicted subblocks. It is asserted that the VVC specification text is also simplified. Considering the trade-off between complexity and coding performance, we recommend to adopt this contribution to VVC.

Proposed text

The proposed text change is highlighted in yellow.

- 8.5.7.3  Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 MxN blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = Max( 1, cbWidth >> 23) and numSbY = Max( 1, cbHeight >> 23). M is set equal to Min( 8, cbWidth ) and N is set equal to Min( 8, cbHeight ).

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-814)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4MxN subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-815)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-816)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-817)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-818)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0	(8-819)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1	(8-820)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-821)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-822)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-823)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-824)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-825)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-826)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0	(8-827)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1	(8-828)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-829)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-830)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-831)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-832)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-833)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-834)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-835)

predFlagL1 = 1		(8-836)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-837)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-838)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-839)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-840)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-841)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-842)

  - Otherwise (if predListFlagA + predListFlagB is not equal to 01), the following applies:

predFlagL0 = 1( predListFlagB = = 0 )  ?  1  :  0	(8-843)

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1
            1 − predFlagL0                  	(8-844)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-845)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )
         ( predListFlagB = = 0 )  ?  refIdxB : −1

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-846)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )
         ( predListFlagB = = 0 )  ?  −1  :  refIdxB

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-847)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )
          ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-848)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )
          ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-849)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )
          ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-850)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )
          ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-851)

predFlagL1 = 1		(8-852)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-853)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-854)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-855)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-856)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-857)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-858)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

- The following assignments are made for x = 0..3M-1 and y = 0..3N-1:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-859)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-860)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-861)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-862)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-863)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(8-864)

- 8.5.7.4  Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

The variable refPicPoc is derived as follows:

refPicPoc = ( X = = 0 )  ?  RefPicList[ 0 ][ refIdxN ]  :  RefPicList[ 1 ][ refIdxN ]	(8-865)

The reference picture list refPicListTemp is derived as follows:

refPicListTemp = ( X = = 0 )  ?  RefPicList[ 1 ]  :  RefPicList[ 0 ]	(8-866)

The variable refIdxTemp is derived as follows:

- The variable mapStop is set equal to FALSE.

- For the variable refIdxm with m = 0..NumRefIdxActive[ 1 ] − 1, the following applies until mapStop is equal to FALSE:

refIdxTemp = ( refPicListTemp[ refIdxm ] = = refPicPoc )  ?  refIdxm  :  −1	(8-867)

mapStop = ( refIdxTemp != −1 )  ?  TRUE  :  FALSE	(8-868)

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).