| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0630-v2 |
| --- | --- |

| Title: | Non-CE4: DMVR on merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In VVC draft 5, DMVR is applied to a block predicted using regular merge mode or CIIP mode. There is no flag to turn off DMVR. In this contribution, it is proposed to signal one flag to disable DMVR for regular merge mode. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.07%, 0.06%, 0.05%} for RA with no impact on encoding time and decoding time.

Introduction

In VVC draft 5, DMVR is always applied to a block predicted using regular merge mode or combined inter and intra prediction mode (CIIP) if the following conditions are all satisfied:

- CU level merge mode with bi-prediction MV or combined inter and intra prediction mode (in this case DMVR is applied to the inter part of the CIIP mode)

- The block is predicted using bi-prediction motion vector with equal weights. That is, BCW is not applied to the block

- One reference picture is in the past and another reference picture is in the future with respect to the current picture

- The distances (i.e. POC difference) from both reference pictures to the current picture are the same

- The block has at least 128 luma samples and the block width and height are both larger than or equal to 8 luma samples

In this contribution, we propose a flag to turn on/off DMVR. For a block predicted using regular merge mode or CIIP mode, the decision of DMVR is inferred from its neighboring block. When the block is not coded using the regular merge mode, a flag followed by an index is signaled to provide the opposite option of DMVR in the regular merge mode – that is, if DMVR is turned off for a given merge candidate (e.g. merge candidate 1) in the regular merge mode, then in this alternative merge process, DMVR is turned on for the given merge candidate (e.g. merge candidate 1). An example is shown in Figure 1. When a block coded using the regular merge mode with merge candidate 1, DMVR is turned off. When a block coded using the alternative merge mode with merge candidate 1, DMVR is turned on.

Figure 1. An example of merge candidates for the regular merge mode and alternative merge mode

The merge candidate list for the alternative merge mode is derived from that of the regular merge mode using the following process:

- If a merge candidate in the list of regular merge mode is bi-predicted and satisfied the DMVR conditions, the merge candidate is directly copied with the decision of DMVR inverted.

- Otherwise (the merge candidate is uni-predicted), the merge candidate is converted into bi-predicted candidate to avoid wasting the signaling overhead of the alternative merge mode flag using the following process:

  - DMVR is set to be on for the converted bi-predicted merge candidate.

  - Without loss of generality, assuming that the merge candidate is L0 predicted, and the first available L1 predicted motion vector in the list of regular merge mode is searched.

  - If the first available L1 predicted motion vector exists, it is directly copied and combined with the merge candidate to form the bi-predicted merge candidate.

  - Otherwise, the L1 motion vector of the converted bi-predicted merge candidate is set to be the same as L0 motion vector with reference index set to 0.

After decoding and reconstructing a block, a DMVR on/off flag is set to indicate whether DMVR is turned on/off for the block according to the following process:

- If the block is predicted using regular merge mode, CIIP mode or the proposed alternative merge mode, the DMVR on/off flag is set to be the same as the DMVR on/off flag used in these modes.

- Otherwise, the DMVR on/off flag is set to be on.

When coding future blocks, the DMVR on/off flag of this block is inherited if the future block is coded using the regular merge mode and CIIP mode, and inverted if the future block is coded using the alternative merge mode.

Simulation results

The proposed method is implemented on top of VTM-5.0. The results showed that there is 0.07% coding gain. In addition, there is no impact on encoding and decoding time.

Table 1. Summary results of all tests

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.01% | 101% | 99% |
| Class A2 | -0.08% | -0.04% | -0.02% | 101% | 99% |
| Class B | -0.07% | 0.00% | -0.03% | 100% | 99% |
| Class C | -0.08% | -0.05% | -0.08% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.04% | 100% | 99% |
| Class D | -0.11% | -0.09% | 0.02% | 100% | 97% |
| Class F | -0.05% | 0.04% | 0.00% | 100% | 99% |

Conclusion

In this contribution, it is proposed to disable DVMR for regular merge mode by signaling a CU-level flag. It is reported that 0.07% coding gain is obtained under CTC condition. Considering this interesting results, we recommend to study this contribution in CE.

Proposed text

The proposed syntax change is highlighted in yellow.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| … |  |
| } else { |  |
| if( cbWidth >= 8 && cbHeight >= 8 && cbWidth * cbHeight >= 128 ) |  |
| oppo_dmvr_flag[ x0 ][ y0 ] | ae(v) |
| if( oppo_dmvr_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| alt_merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |
| } |  |

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).