| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0632 |
| --- | --- |

| Title: | Non-CE3: WAIP restriction for square sub-CU blocks partitioned by ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankit Kumar Sandeep Shrestha Bumshik Lee Yonggil Lee Junsuk Park | Email: | ankitku@chosun.kr sandeepshrestha407@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In current VTM-5.0 [1], for the ISP mode, the selection of wide-angle mode solely relies on the shape of current CU rather than sub-CU. It is also proclaimed that when the wide-angle mode is enabled for a sub-CU for the ISP mode, it needs to access more neighboring pixels than the normal intra CU in the same size. In this contribution, it is proposed to harmonize the wide angle intra prediction and ISP by selecting the wide-angle based on the size of sub-CUs in the ISP mode only for the non-square sub-CU’s.

Introduction

WAIP [2] was proposed to allow extended intra prediction angles beyond the traditional 45-degree top-right and bottom-left directions to improve the accuracy of prediction blocks using closer reference samples for non-square blocks. WAIP replaces several conventional angular intra prediction modes with wide angular modes according to aspect ratio of non-square block.

ISP [3] divides an intra luma coding block vertically or horizontally into 2 or 4 sub-partitions which share the same intra mode. Each sub-partition is sequentially encoded/decoded to generate a reconstructed block and used as reference samples of the next sub-partition. All sub-partitions share the same intra mode which is determined based on a coding block.

In the VTM-5.0, WAIP determines the intra mode considering the aspect ratio of the current CU. However, when the aspect ratio of the CU and sub-CU partitioned by ISP is different, it is not appropriate to apply the intra mode for sub-CU, derived from CU. Therefore, this contribution proposes a method of determining the intra mode of sub-partition CU partitioned by ISP.

Proposed method

We propose a WAIP restriction condition for the ISP block when a CU block is partitioned into non-square sub-CU blocks. We restrict the WAIP mode for square sub-CU blocks, partitioned by ISP from CU.

| (a) A 8×4 CU block partitioned to two 4×4 square sub-blocks | (b) A 4×8 CU block partitioned to two 4×4 square sub-blocks |
| --- | --- |

Fig. 1. An example of WAIP restriction. When a non-square CU block of 8×4 (or 4×8) is partitioned to two 4x4 sub-CU block, the WAIP is restricted for each square 4×4 sub-CU block

The CU block sizes for the proposed WAIP restriction for sub-CU blocks can be 4×8, 8×4, 4×16 and 16×4 because these non-square CU blocks can produce the square sub-CU blocks by ISP.

For the consistency of applying the WAIP for non-square sub-CU, we propose to determine the WAIP based on the non-square blocks only.

Experimental Results

The proposed modification is integrated into VTM-5.0 and the results of the proposed implementation are shown below against the VTM-5.0 under CTC test conditions.

Table 1. Performance of the proposed modification under All Intra

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | OverVTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Class B | 0.00% | -0.03% | -0.03% | 98% | 100% |
| Class C | 0.00% | 0.02% | 0.07% | 100% | 100% |
| Class E | 0.02% | -0.11% | -0.08% | 100% | 99% |
| Overall | 0.00% | -0.03% | 0.00% | 99% | 99% |
| Class D | 0.02% | 0.04% | -0.02% | 100% | 99% |
| Class F | -0.02% | 0.02% | 0.02% | 100% | 100% |

Table 2. Performance of the proposed modification under Random Access

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.07% | 0.17% | 99% | 99% |
| Class A2 | 0.05% | 0.07% | 0.16% | 99% | 100% |
| Class B | 0.09% | 0.04% | 0.02% | 100% | 100% |
| Class C | 0.03% | -0.02% | 0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.01% | 0.10% | 100% | 100% |
| Class D | 0.06% | 0.16% | 0.14% | 100% | 100% |
| Class F | 0.08% | 0.05% | 0.12% | 100% | 101% |

Table 3. Performance of the proposed modification under Low Delay B

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  | OverVTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.27% | 0.08% | 97% | 100% |
| Class C | 0.07% | 0.28% | 0.04% | 100% | 100% |
| Class E | -0.07% | 0.30% | 0.21% | 100% | 99% |
| Overall | 0.02% | 0.28% | 0.10% | 99% | 99% |
| Class D | 0.04% | 0.04% | 0.53% | 100% | 99% |
| Class F | 0.01% | 0.24% | 0.32% | 100% | 100% |

Conclusion

In this contribution, harmonization method between WAIP and ISP is proposed, which provides an efficient breakthrough for the inconsistency design. It is recommended to adopt the proposed method in the next VTM.

References

| [1] | "VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | Fabien Racapé, et al., “CE3-related: Wide-angle intra prediction for non-square blocks”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0500, 11th meeting, Ljubljana, SI, 10–18 July 2018 |
| [3] | Santiago De-Luxán-Hernández, et al., “CE3: Intra Sub-Partitions Coding Modes”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0102, 13th meeting, Marrakech, MA, 9–18 Jan. 2019 |
| [4] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

Patent rights declaration(s)

Chosun University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).