| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0634 |
| --- | --- |

| Title: | BDOF simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Haitao Yang | Tel: Email: | +86-13480990067 anderson.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution studies the performance impact on BDOF when meet conditions as follow.

- Not applied in Nx8 CUs case.

- Not applied in 8x8/4xN CUs case.

3.  Turn BDOF off when DMVR is activated for small blocks (8x16/16x8 CUs).

Test1 is to further check the performance impact on BDOF when Nx8 CUs are removed. Test2 is to check the performance impact on BDOF when 8x8/4xN CUs are removed. Test3 is to check the case that BDOF is off when DMVR is activated for 8x16/16x8 CUs. Experiments were conducted on top of the VTM-5.0 with Random-Access testing condition. The performance results are summarized as follows.

Test1. VTM: (Y) 0.06%, (U) -0.03%, (V) -0.01%, (Enc.) 100%, (Dec.) 101%.

Test2. VTM: (Y) 0.04%, (U) -0.02%, (V) -0.04%, (Enc.) 100%, (Dec.) 100%.

Test3. VTM: (Y) 0.02%, (U) -0.01%, (V) 0.01%, (Enc.) 101%, (Dec.) 104%.

Introduction

BDOF refines the luma sample predictors for a CU when the CU is coded with bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order. A CU-size constraint is imposed on BDOF, so BDOF performs only for CUs with 8 pixels in height at least.

DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) when the CU is coded with regular merge/skip mode and bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture. CU-size constraints are also imposed on DMVR, so DMVR performs only for CUs with 8 pixels in height and width at least and height x width is greater than or equal to 128.

Proposed Methods

BDOF removal of Nx8 CU

This coding tool imposes an additional CU-size constraint on BDOF to avoid using CUs of size Nx8 for BDOF, which has 11.4% multiplication reduction to current BDOF design.

Spec modification as follow

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than 8

      - cIdx is equal to 0.

BDOF Removal of 8x8 and 4xN CUs

This coding tool imposes an additional CU-size constraint on BDOF to avoid using CUs of size 8x8/4xN for BDOF, which is similar as current DMVR size constraint design.

Spec modification as follow

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cbWidth is greater than or equal to 8

      - cbHeight*cbWidth is greater than or equal to 128

      - cIdx is equal to 0.

Turn BDOF off when DMVR is activated for small CUs (8x16/16x8 CUs)

To reduce calculation complexity when BDOF and DMVR are both activated for small CUs, this coding tool imposes an additional CU-size constraint on BDOF to avoid using CUs of size 8x16/16x8 for BDOF when DMVR is activated, which also can reduce power consumption as DMVR and BDOF not in same pipeline.

Spec modification as follow

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cbHeight*cbWidth is greater than 128 when dmvrFlag is equal to 1.

      - cIdx is equal to 0.

Experimental Results

The experiments for BDOF with the proposed methods were conducted on top of the VTM-5.0[1] software with the common test conditions [2] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio.

Coding Performance

Three tests were conducted. The respective performance results are summarized in Table1-3.

Test1: Removing Nx8 CUs from BDOF;

Test2: Removing 8x8/4xN CUs from BDOF;

Test3: Set BDOF off when DMVR is on for small (8x16/16x8) CUs;

- Performance results of Test2 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.01% | 100% | 100% |
| Class A2 | 0.07% | -0.04% | -0.03% | 100% | 100% |
| Class B | 0.06% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.10% | 0.01% | 0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.03% | -0.01% | 100% | 101% |
| Class D | 0.20% | -0.02% | 0.04% | 100% | 103% |
| Class F (optional) | 0.05% | 0.03% | 0.00% | 100% | 102% |

- Performance results of Test3 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.05% | 100% | 100% |
| Class A2 | 0.04% | -0.02% | -0.07% | 100% | 100% |
| Class B | 0.03% | 0.00% | 0.02% | 100% | 100% |
| Class C | 0.08% | 0.04% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.04% | 100% | 100% |
| Class D | 0.19% | -0.01% | 0.12% | 100% | 98% |
| Class F (optional) | 0.02% | 0.02% | 0.00% | 100% | 101% |

- Performance results of Test4 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | -0.01% | 98% | 98% |
| Class A2 | 0.01% | -0.01% | 0.00% | 107% | 117% |
| Class B | 0.02% | 0.02% | 0.01% | 102% | 105% |
| Class C | 0.02% | 0.01% | 0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.01% | 101% | 104% |
| Class D | 0.09% | 0.07% | 0.10% | 99% | 99% |
| Class F (optional) | 0.02% | 0.04% | 0.02% | 100% | 101% |

Computational Complexity

- The generation of the L0 and L1 prediction samples

To derive the BDOF motion refinement of each block, the numbers of multiplications and additions that are needed to generate L0 and L1 prediction samples are equal to (W×(H+7)×8+W×H×8)×2.

- Gradient calculation

Only one addition is needed per sample to derive one gradient. Considering both horizontal and vertical gradients in one extended region W×H are needed for both L0 and L1, the total number of additions that are required for the gradient derivation is equal to W×H×2×2.

- Correlation parameter calculation

The total numbers of multiplications and additions for calculating the correlation parameters are equal to ((W+2)×(H+2))×5 and ((W+2)×(H+2))×3, respectively.

- Summation

The motion refinements are separately derived for each 4x4 block within one BIO CU. To derive the motion refinement of each 4x4 sub-block, it needs to calculate the sums of the five correlation parameters inside the 6x6 surrounding area. Therefore, the summation of the five correlation parameters needs in total (W/4)×(H/4)×6×6×5 additions.

- Motion refinement derivation

To derive one local motion refinement vx,vy , there are two additions for adding the regulation parameter r to S1 and S3. Additionally, there are one multiplication and one addition needed to calculate the value of vy. So, to derive the motion refinements for all 4x4 blocks inside the CU, the numbers of multiplications and additions are equal to (W/4)×(H/4) and (W/4)×(H/4)×3, respectively.

- Generation of the bi-prediction signal

Given the motion refinements, two multiplications and six additions are further needed to derive the final predication sample at each position. Therefore, there are in total W×H×2 multiplications and W×H×6 additions that need to be performed at this stage.

- Early termination

BDOF can be early skipped based on the SAD between L0 and L1 prediction samples of the CU. Therefore, there are W×H ×2 -1 additions that are needed to calculate the SAD value.

Table 4 shows the numbers of multiplications and additions per samples that are performed with BDOF. As can been seen, the BDOF worst-case complexity happens at 8x8 CUs.

Table 4 The number of multiplications per sample performed by method 1 compared to 16×4 bi-pred

| (W×H) |  |  | BDOF |  |  |  |  |  |  | Percentage to 16x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×16 | 39.0 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 49.5 | 82.5% |
| 8×8 | 46.0 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 55.9 | 93.2% |
| 16×8 | 46.0 | 0.0 |  | 7.0 | 0.0 | 0.1 | 2.0 | 0.0 | 55.1 | 91.8% |
| 8×16 | 39.0 | 0.0 |  | 7.0 | 0.0 | 0.1 | 2.0 | 0.0 | 48.1 | 80.2% |
| 16×16 | 39.0 | 0.0 |  | 6.3 | 0.0 | 0.1 | 2.0 | 0.0 | 47.4 | 79% |

References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).