| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0635 |
| --- | --- |

| Title: | AHG 10-Related: Optimized CTU Level Bit Allocation in Inter Frames |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye Li, Ning Yan Dong Liu, Zhibo Chen,USTC Li Li, Zhu Li, UMKC | Tel: Email: | liye0713@mail.ustc.edu.cn nyan@mail.ustc.edu.cn chenzhibo@mail.ustc.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

- Abstract

This contribution proposes a modification to the CTU level bit allocation method for rate control in VVC reference software, i.e. VTM. In the current software VTM-5.0, the frame level lambda value is calculated by the frame level alpha and beta parameters as well as the frame level target bpp, according to the R-lambda model. Then the initial bit weight of each CTU within the frame is obtained by the CTU level parameters and the frame level lambda. Due to the inaccuracy of the updated frame level alpha and beta value, the calculated frame level lambda may be not accurate. Thus the CTU level initial bit weight is not optimized in terms of coding efficiency. In this contribution, we first allocate a constant target bit for skip CTUs, then we propose to use a bisection method to calculate the frame level lambda. With this, the CTU level bit allocation can be optimized for inter frame rate control. Compared with VTM-5.0, an average 0.44% BD-rate reduction can be achieved under low delay configuration.

Introduction

In inter frame rate control of VTM-5.0, the frame level lambda is calculated by the R-lambda model [1],

Where bpp is frame level target bpp and are the two parameters decided by the video content.

The R-lambda model can be rewritten as follows:

Then before encoding the whole frame, the initial bit weight of each CTU is obtained as follows [2]:

Where  are related to the i-th CTU R-lambda model parameters, and  means the number of pixels in the i-th CTU.

Proposed Rate Control Modification Methods

Bit Allocation of Skip CTUs

Since for skip CTUs, the original R-lambda model can not correctly characterize the true R-lambda relationship, so for skip CTUs, a constant bit is allocated:

And the target bpp of skip CTU is set as 0.001. The skip CTU is determined by the CTU level rate control parameter skip_ratio( with skip_ratio=1 means the current CTU is a skip CTU).

Frame Level Lambda Calculation

Considering the sum of CTU-level target bits should be equal to the frame level target bits, an equation can be obtained [3],

In the above equation only the frame level lambda, i.e.  is unknown. Due to the monotone of the left-side function,  can be obtained through bisection method, with only a few round of iterations. Besides, another two termination condition are set as follows:

- If  for the current solution of . Where  is set as  and

- If

Simulation results

Table 1. Simulation results of Test 1

|  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  | -0.38% |  |  |  |  | -1.24% |  |  |  | -1.35% |  |  |  | 100% |  |  |  | #DIV/0! |
| Class C |  |  | -0.31% |  |  |  |  | -0.56% |  |  |  | -0.57% |  |  |  | 100% |  |  |  | #DIV/0! |  |
| Class E |  |  | -0.72% |  |  |  |  | -1.15% |  |  |  | -1.84% |  |  |  | 98% |  |  |  | #DIV/0! |  |
| Overall |  |  | -0.44% |  |  |  |  | -0.99% |  |  |  | -1.21% |  |  |  | 100% |  |  |  | #DIV/0! |  |
| Class D |  | -0.30% |  |  |  |  | -0.52% |  |  |  | -0.67% |  |  |  | 100% |  |  |  | #DIV/0! |  |  |
| Class F |  |  | 2.09% |  |  |  |  | -0.40% |  |  |  | -0.14% |  |  |  | 98% |  |  |  | #DIV/0! |  |

Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC K0103, Oct. 2012.

- B. Li, H. Li, and L. Li, “Adaptive bit allocation for R-lambda model rate control in HM,” JCT-VC M0036, Apr. 2013.

- Y. Li, Z. Chen, X. Li and S. Liu, “Rate Control for VVC,” JVET-K0390, Jul. 2018.