| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0636_r1 |
| --- | --- |

| Title: | Cross-Component Adaptive Loop Filter for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Frank Bossen2, Andrew Segall1 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | 1Sharp Labs of America 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

The contribution proposes a tool called the Cross-Component Adaptive Loop Filter (CC-ALF). CC-ALF operates as part of the adaptive loop filter process and makes use of luma sample values to refine each chroma component.   The tool is controlled by information in the bit-stream, and this information includes both (a) filter coefficients for each chroma component and (b) a mask controlling the application of the filter for blocks of samples. The filter coefficients are signalled in the APS, while block sizes and mask are signalled at the slice-level.

The proposed approach was evaluated using the CTC and resulted in the average BD rate improvement:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| All-Intra | 0.13% | -8.54% | -6.65% | 102% | 105% |
| Random Access | 0.16% | -14.36% | -13.16% | 103% | 105% |
| Low Delay B | 0.21% | -20.11% | -15.39% | 104% | 109% |

Furthermore, initial experiments to shift the gains from chroma to luma were also performed and resulted in the average BD rate improvement:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| All-Intra | -0.93% | 0.82% | 2.55% | 102% | 104% |
| Random Access | -0.95% | -5.07% | -3.82% | 102% | 105% |
| Low Delay B | -0.12% | -14.81% | -9.24% | 103% | 110% |

Revision 1 of the document provides updates the experimental results and provides more signaling details in the appendix

# Proposal

The contribution proposes a tool called Cross-Component Adaptive Loop Filter (CC-ALF).  CC-ALF makes use of luma sample values to refine each chroma component.  Fig. 1a below illustrates the placement of CC-ALF with respect to the other loop filters.

| (a) | (b) |
| --- | --- |

Fig. 1: (a) Placement of CC ALF with respect to other loop filters (b) Diamond shaped filter

CC-ALF operates by applying a linear, diamond shaped filter (Fig 1b) to the luma channel for each  chroma component. The filter coefficients are transmitted in the APS, scaled by a factor of 210, and rounded for fixed point representation. The application of the filters is controlled on a variable block size and signalled by a context-coded flag received for each block of samples. The block size along with an CC-ALF enabling flag is received at the slice-level for each chroma component. Syntax and semantics for CC-ALF are provided in the Appendix.  In the contribution the following block sizes (in chroma samples) were supported 16x16, 32x32, 64x64, 128x128.

# Experimental Results

The proposed method was implemented in VTM5 and evaluated for CTC [1].  A summary of the results is provided in Section 2.1 below.  As can be seen in the section, the tool provides coding efficiency improvements in the chroma channel for All Intra, Random Access, and Low Delay configurations.  Furthermore, the gains range from approximately 3.5% to over 25% for the different Classes.  Detailed results can be found in the attached Excel files.

Additionally, results are shown in Section 2.2 where we provide a first method for shifting the gains from the chroma channels to luma.  This was accomplished by increasing the chroma QP offset by 1 and modifying the loop filter lambda to correspond to (chroma QP - 1). As can be seen in the results, the overall luma gain is .9% for both All Intra and Random Access configurations.  Detailed results can be found in attached Excel files.

## Summary of results without shifting chroma gains (CTC)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -3.66% | -4.62% | 103% | 105% |
| Class A2 | 0.12% | -8.98% | -9.46% | 103% | 105% |
| Class B | 0.10% | -11.49% | -9.68% | 102% | 105% |
| Class C | 0.14% | -5.15% | -4.59% | 102% | 104% |
| Class E | 0.21% | -12.58% | -3.59% | 102% | 104% |
| Overall | 0.13% | -8.54% | -6.65% | 102% | 105% |
| Class D | 0.16% | -3.91% | -4.28% | 101% | 101% |
| Class F | 0.23% | -7.64% | -7.99% | 102% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | -8.96% | -7.59% | 103% | 106% |
| Class A2 | 0.09% | -14.72% | -19.43% | 103% | 108% |
| Class B | 0.24% | -23.01% | -18.32% | 104% | 106% |
| Class C | 0.13% | -7.34% | -6.18% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -14.36% | -13.16% | 103% | 105% |
| Class D | 0.14% | -6.69% | -5.38% | 103% | 102% |
| Class F | 0.30% | -7.80% | -7.45% | 104% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | -26.96% | -24.33% | 102% | 115% |
| Class C | 0.24% | -10.68% | -8.72% | 105% | 108% |
| Class E | 0.21% | -21.27% | -9.37% | 107% | 102% |
| Overall | 0.21% | -20.11% | -15.39% | 104% | 109% |
| Class D | 0.17% | -11.09% | -6.98% | 104% | 105% |
| Class F | 0.36% | -14.15% | -12.35% | 105% | 110% |

## Summary of results after shifting chroma gains to luma

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | 7.35% | 4.69% | 103% | 105% |
| Class A2 | -1.88% | 0.69% | 0.29% | 101% | 104% |
| Class B | -0.72% | -1.67% | 0.34% | 102% | 104% |
| Class C | -0.67% | 3.48% | 4.01% | 101% | 103% |
| Class E | -0.41% | -4.99% | 4.37% | 102% | 103% |
| Overall | -0.93% | 0.82% | 2.55% | 102% | 104% |
| Class D | -0.62% | 4.57% | 4.35% | 101% | 101% |
| Class F | -0.62% | -1.09% | -1.57% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43% | -0.26% | 0.64% | 101% | 106% |
| Class A2 | -1.88% | -5.91% | -11.28% | 101% | 106% |
| Class B | -0.36% | -13.10% | -8.01% | 102% | 105% |
| Class C | -0.64% | 1.98% | 3.67% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.95% | -5.07% | -3.82% | 102% | 105% |
| Class D | -0.53% | 2.99% | 4.42% | 103% | 102% |
| Class F | -0.69% | -1.53% | -1.17% | 103% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -22.39% | -19.00% | 103% | 117% |
| Class C | -0.24% | -5.87% | -3.86% | 103% | 110% |
| Class E | 0.03% | -14.10% | -0.14% | 104% | 101% |
| Overall | -0.12% | -14.81% | -9.24% | 103% | 110% |
| Class D | -0.15% | -6.35% | -1.62% | 104% | 102% |
| Class F | -0.03% | -11.14% | -9.21% | 104% | 109% |

As can be observed from results above it is possible to move more of the average chroma gains to luma for the random-access and low delay B configuration.

# Conclusion

The contribution proposes a tool called Cross-Component Adaptive Loop Filter (CC-ALF).  The tool operates as part of the adaptive loop filter and uses samples from the luma channel to further refine the chroma channel.  Results show chroma gains of approximately 7.6%, 13.8% and 17.8% for All Intra, Random Access, and Low Delay, respectively.  Furthermore, results are provided that show that it is possible to shift these gains to the luma channel, if so desired.

It is recommended to formally study the method in a Core Experiment.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring " JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Changes to VVC WD5

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_cross_component_alf_enabled_flag ) { |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_reuse_temporal_layer_filter | u(1) |
| if (!slice_cross_component_alf_cb_reuse_temporal_layer_filter) |  |
| slice_cross_component_alf_cb_aps_id | u(5) |
| slice_cross_component_alf_cb_log2_control_size_minus4 | ue(v) |
| } |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_ct_reuse_temporal_layer_filter | u(1) |
| if (!slice_cross_component_alf_cr_reuse_temporal_layer_filter) |  |
| slice_cross_component_alf_cr_aps_id | u(5) |
| slice_cross_component_alf_cr_log2_control_size_minus4 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

slice_cross_component_alf_cb_enabled_flag equal to 0 specifies that the cross-component Cb filter is not applied to Cb colour component. slice_ cross_component_alf_cb_enabled_flag equal to 1 indicates that the cross-component Cb filter is applied to the Cb colour component.

slice_cross_component_alf_cr_enabled_flag equal to 0 specifies that the cross-component Cr filter is not applied to Cr colour component. slice_ cross_component_alf_cb_enabled_flag equal to 1 indicates that the cross-component Cr filter is applied to the Cr colour component.

slice_cross_component_alf_cb_reuse_temporal_layer_filter equal to 1 specifies that the cross-component Cb filter coefficients, with j=0..13, inclusive is set equal to AlfCCTemporalCoeffCb[ TemporalId ][ j ]. slice_cross_component_alf_cb_reuse_temporal_layer_filter equal to 0 and slice_cross_component_alf_cb_enabled_flag is equal to 1 specifies that the syntax element  slice_cross_component_alf_cb_aps_id is present in slice header.

When slice_cross_component_alf_cb_enabled_flag  is equal to 1, and slice_cross_component_alf_cb_reuse_temporal_layer_filter is equal to 0, the elements of AlfCCTemporalCoeffCb[ TemporalId ][ j ], with j = 0..13 are derived as follows:

AlfCCTemporalCoeffCb[ TemporalId ][ j ] = AlfCCCoeffCb[ slice_cross_component_alf_cb_aps_id ][ j ]

slice_cross_component_alf_cr_reuse_temporal_layer_filter equal to 1 specifies that the cross-component Cr filter coefficients, with j=0..13, inclusive is set equal to AlfCCTemporalCoeffCr[ TemporalId ][ j ]. slice_cross_component_alf_cr_reuse_temporal_layer_filter equal to 0 and slice_cross_component_alf_cr_enabled_flag is equal to 1 specifies that the syntax element  slice_cross_component_alf_cr_aps_id is present in slice header.

When slice_cross_component_alf_cr_enabled_flag  is equal to 1, and slice_cross_component_alf_cr_reuse_temporal_layer_filter is equal to 0, the elements of AlfCCTemporalCoeffCr[ TemporalId ][ j ], with j = 0..13 are derived as follows:

AlfCCTemporalCoeffCr[ TemporalId ][ j ] = AlfCCCoeffCr[ slice_cross_component_alf_cr_aps_id ][ j ]

slice_cross_component_alf_cb_aps_id specifies the adaptation_parameter_set_id that the Cb colour component of the slice refers to for cross-component Cb filter. When slice_cross_component_alf_cb_aps_id is not present, it is inferred to be equal to slice_alf_aps_id_luma[ 0 ]. The TemporalId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_cross_component_alf_cb_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

slice_cross_component_alf_cr_aps_id specifies the adaptation_parameter_set_id that the Cr colour component of the slice refers to for cross-component Cr filter. When slice_cross_component_alf_cr_aps_id is not present, it is inferred to be equal to slice_alf_aps_id_luma[ 0 ]. The TemporalId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_cross_component_alf_cr_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

slice_cross_component_alf_cb_log2_control_size_minus4 specifies the value of the square block sizes in number of samples as follows:

AlfCCSamplesCbW = AlfCCSamplesCbH = 2( slice_cross_component_alf_cb_log2_control_size_minus4 + 4 )

slice_cross_component_alf_cb_log2_control_size_minus4 shall be in the range 0 to 3, inclusive.

slice_cross_component_alf_cr_log2_control_size_minus4 specifies the value of the square block sizes in number of samples as follows:

AlfCCSamplesCrW = AlfCCSamplesCrH = 2( slice_cross_component_alf_cr_log2_control_size_minus4 + 4 )

slice_cross_component_alf_cr_log2_control_size_minus4 shall be in the range 0 to 3, inclusive.

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_cross_component_alf_enabled_flag) { |  |
| alf_cross_component_cb_filter_signal_flag | u(1) |
| alf_cross_component_cr_filter_signal_flag | u(1) |
| } |  |
| if( alf_luma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| … |  |
| } |  |
| if( alf_cross_component_cb_filter_signal_flag ) { |  |
| alf_cross_component_cb_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_cross_component_cb_eg_order_increase_flag[ i ] | u(1) |
| for ( j = 0; j < 14; j++ ) { |  |
| alf_cross_component_cb_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cb_coeff_abs[ j ] ) |  |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| if( alf_cross_component_cr_filter_signal_flag ) { |  |
| alf_cross_component_cr_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_cross_component_cr_eg_order_increase_flag[ i ] | u(1) |
| for ( j = 0; j < 14; j++ ) { |  |
| alf_cross_component_cr_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cr_coeff_abs[ j ] ) |  |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| } |  |

alf_luma_filter_signal_flag equal to 1 specifies that a luma filter set is signalled. alf_luma_filter_signal_flag equal to 0 specifies that a luma filter set is not signalled. When alf_luma_filter_signal_flag is not present, it is inferred to be equal 0.

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When alf_chroma_filter_signal_flag is not present, it is inferred to be equal 0.

alf_cross_component_cb_filter_signal_flag equal to 1 specifies that a cross-component Cb filter set is signalled. alf_cross_component_cb_filter_signal_flag equal to 0 specifies that a cross-component Cb filter set is not signalled. When alf_cross_component_cb_filter_signal_flag is not present, it is inferred to be equal 0.

alf_cross_component_cr_filter_signal_flag equal to 1 specifies that a cross-component Cr filter set is signalled. alf_cross_component_cb_filter_signal_flag equal to 0 specifies that a cross-component Cr filter set is not signalled. When alf_cross_component_cr_filter_signal_flag is not present, it is inferred to be equal 0.

alf_cross_component_cb_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for cross-component Cb filter coefficient signalling. The value of alf_cross_component_cb_min_eg_order_minus1 shall be in the range of 0 to 9, inclusive.

alf_cross_component_cr_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for cross-component Cr filter coefficient signalling. The value of alf_cross_component_cb_min_eg_order_minus1 shall be in the range of 0 to 9, inclusive.

alf_cross_component_cb_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for cross-component Cb filter coefficient signalling is incremented by 1. alf_cross_component_cb_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for cross-component Cb filter coefficient signalling is not incremented by 1.

The order expGoOrderCb[ i ] of the exp-Golomb code used to decode the values of alf_cross_component_cb_coeff_abs[ j ] is derived as follows:

expGoOrderCb[ i ] = ( i = = 0  ?  alf_cross_component_cb_min_eg_order_minus1 + 1  :  expGoOrderCb[ i − 1 ] ) + alf_cross_component_cb_eg_order_increase_flag[ i ]

alf_cross_component_cr_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for cross-component Cr filter coefficient signalling is incremented by 1. alf_cross_component_cr_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for cross-component Cr filter coefficient signalling is not incremented by 1.

The order expGoOrderCr[ i ] of the exp-Golomb code used to decode the values of alf_cross_component_cb_coeff_abs[ j ] is derived as follows:

expGoOrderCr[ i ] = ( i = = 0  ?  alf_cross_component_cr_min_eg_order_minus1 + 1  :  expGoOrderCr[ i − 1 ] ) + alf_cross_component_cr_eg_order_increase_flag[ i ]

alf_cross_component_cb_coeff_abs[ j ] specifies the absolute value of the j-th coefficient of the signalled cross-component Cb filter. When alf_cross_component_cb_coeff_abs[ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxCb[ ] = {0,2,2,2,1,2,2,2,2,2,2,1,2,1} [these may be Categorize coefficient into 3 categories, each category uses the same order k exp-Golomb code]

k = expGoOrderCb[ golombOrderIdxCb[ j ] ]

alf_cross_component_cr_coeff_abs[ j ] specifies the absolute value of the j-th coefficient of the signalled cross-component Cr filter. When alf_cross_component_cr_coeff_abs[ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxCr[ ] = {0,1,2,1,0,1,2,2,2,2,2,1,2,1}[these may be Categorize coefficient into 3 categories, each category uses the same order k exp-Golomb code]

k = expGoOrderCr[ golombOrderIdxCr[ j ] ]

alf_cross_component_cb_coeff_sign[ j ] specifies the sign of the j-th cross-component Cb filter coefficient as follows:

- If alf_cross_component_cb_coeff_sign[ j ] is equal to 0, the corresponding cross-component Cb filter coefficient has a positive value.

- Otherwise (alf_cross_component_cb_coeff_sign[ j ] is equal to 1), the corresponding cross-component Cb filter coefficient has a negative value.

When alf_cross_component_cb_coeff_sign[ j ] is not present, it is inferred to be equal to 0.

The cross-component Cb filter coefficients AlfCCCoeffCb[ adaptation_parameter_set_id ] with elements AlfCCCoeffCb[ adaptation_parameter_set_id ][ j ], with j = 0..13 are derived as follows:

AlfCCCoeffCb[ adaptation_parameter_set_id ][ j ] = alf_cross_component_cb_coeff_abs[ j ] * 	
	( 1 − 2 * alf_cross_component_cb_coeff_sign[ j ] )

It is a requirement of bitstream conformance that the values of AlfCCCoeffCb[ adaptation_parameter_set_id ][ j ] with j = 0..13 shall be in the range of −210 − 1 to 210 − 1, inclusive.

alf_cross_component_cr_coeff_sign[ j ] specifies the sign of the j-th cross-component Cr filter coefficient as follows:

- If alf_cross_component_cr_coeff_sign[ j ] is equal to 0, the corresponding cross-component Cr filter coefficient has a positive value.

- Otherwise (alf_cross_component_cr_coeff_sign[ j ] is equal to 1), the corresponding cross-component Cr filter coefficient has a negative value.

When alf_cross_component_cr_coeff_sign[ j ] is not present, it is inferred to be equal to 0.

The cross-component Cr filter coefficients AlfCCCoeffCr[ adaptation_parameter_set_id ] with elements AlfCCCoeffCr[ adaptation_parameter_set_id ][ j ], with j = 0..13 are derived as follows:

AlfCCCoeffCr[ adaptation_parameter_set_id ][ j ] = alf_cross_component_cr_coeff_abs[ j ] * 	
	( 1 − 2 * alf_cross_component_cr_coeff_sign[ j ] )

It is a requirement of bitstream conformance that the values of AlfCCCoeffCr[ adaptation_parameter_set_id ][ j ] with j = 0..13 shall be in the range of −210 − 1 to 210 − 1, inclusive.

TemporalId is the temporal identifier of the current NAL unit

It is asserted that for the random-access configuration, additional chroma gains could be shifted to the luma channel.