| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0639 |
| --- | --- |

| Title: | Non-CE4: Context reduction for merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, Cheung Auyeung, Shan Liu | Tel: Email: | {jingye, xlxiangli, cauyeung, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the number of contexts for CABAC in VVC. In this contribution, the CABAC context modelling for the CABAC context coded bins ciip_flag and regular_merge_flag is investigated. It is proposed to reuse the context of regular_merge_flag for ciip_flag. The proposal reports 0.01% BD-Rate changing on top of VTM5 for RA configuration under common test condition.

# Proposal

Two contexts are used for signaling regular_merge_flag in VVC [1], and it is proposed to reuse the first context of regular_merge_flag for signaling ciip_flag.

# Results

The proposed method has been tested with CTC defined in [2]. Table 1 show the results with CTC.

Table 1: Coding performance with anchor VTM-5.0 under CTC.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.04% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.01% | 0.09% | 0.00% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.06% | -0.06% | 100% | 99% | 0.02% | 0.37% | 0.07% | 100% | 101% | 0.04% | 0.21% | 0.11% | 99% | 102% |
| Class C | 0.00% | -0.09% | -0.14% | 101% | 100% | 0.03% | 0.12% | 0.11% | 100% | 97% | 0.05% | 0.05% | 0.19% | 100% | 95% |
| Class E |  |  |  |  |  | 0.07% | 0.10% | -0.67% | 99% | 100% | -0.01% | 1.00% | -0.10% | 100% | 96% |
| Overall | 0.01% | 0.00% | -0.05% | 100% | 100% | 0.04% | 0.22% | -0.10% | 100% | 99% | 0.03% | 0.36% | 0.09% | 100% | 98% |
| Class D | 0.01% | -0.05% | 0.17% | 100% | 102% | 0.03% | -0.84% | 1.03% | 100% | 99% | 0.04% | -0.07% | 0.33% | 100% | 97% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution proposed to reduce the context for coding ciip_flag, thus simplified the CABAC coding for VVC. The proposal reports 0.01% coding loss on top of VTM5 for RA configuration under common test condition. It is recommended to adopt the proposed method.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, ISO/IEC JTC1/SC29/WG11 JVET-N1001, Geneva, CH, 19–27, March 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).