| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0640-v1 |
| --- | --- |

| Title: | CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, aramasub, vseregin, martak}@qti.qualcomm.com, |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a restriction on the picture size to avoid 2x2/2x4/4x2 intra chroma blocks at the corner of pictures. On top of Core Experiment (CE) 3-2.1, the proposed restriction completely removes 2x2/2x4/4x2 intra chroma blocks and increases the worst-case processing throughput of VTM5.0 by 4x.

# Introduction

In VTM5.0, it is possible to have small chroma blocks of size smaller than 16 samples including 2x2, 2x4 and 4x2. In a typical hardware video codec, the processing throughput is reduced when a picture is coded using many small blocks. This reduction mainly comes from small intra blocks, because intra blocks have data dependency between neighbouring blocks (e.g., the prediction of a block uses top and left boundary reconstructed samples from neighbouring blocks as reference); hence the reconstruction must be performed sequentially. In order to increase the worst-case processing throughput for VTM5.0, the CE3-2 subtest has been established in Geneva meeting in March 2019.

The CE3-2.2 [1] enables parallel generation of multiple chroma intra coding blocks within a parallel-processable region (PPR) in the single tree case. All the data dependencies between the chroma blocks in the same PPR are eliminated, thus they can be reconstructed in parallel.

In an alternative approach, CE3-2.1 [2] proposes to disallow chroma intra CBs smaller than 16 chroma samples by constraining the partitioning of chroma intra CBs.

# Proposed Method

In the current VVC WD5, the picture width and height in luma samples are restricted to be a multiple of the minimum coding unit size while the minimum luma coding unit size may be 4:

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

When the picture with or height is multiple of 4 but not a multiple of 8, there are 4x4, 4x8/8x4 luma blocks at the corner of the picture. In this setting, when the CE3-2.1 method is applied, there are still 2x2, 2/4 or 4x2 chroma blocks at the corner of the picture.

To completely remove 2x2, 2/4 and chroma blocks for CE3-2.1, this proposal proposes a restriction on the width and height of pictures. The proposed restriction is that the width and height of the picture shall not be equal to 0 and shall be a multiple of the maximum of the value 8 and the minimum coding unit size. The specification text is modified accordingly as follows:

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of the maximum of 8 and MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of the maximum of 8 and MinCbSizeY.

Since VTM5.0 already imposes the proposed restriction (multiple of 8), the experiential results are similar to the results in [2] and they are not presented in this proposal.

# Conclusions

This contribution proposes a restriction on the size of picture to avoid 2x2/2x4/4x2 intra chroma blocks at the corner of pictures.

[1] L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, T. Hsieh, M. Karczewicz (Qualcomm), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), “CE3-2.2: Enabling parallel reconstruction of small intra-coded chroma blocks”, JVET-O0118.

[2] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), Y. Zhao, H. Yang (Huawei), “CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs”, JVET-O0050.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).