| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0641-v2 |
| --- | --- |

| Title: | AHG 8: JVET Common test conditions for adaptive resolution change |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Jie Chen Daniel Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes the common test conditions for adding adaptive resolution change support into VVC. It is asserted that establishing the adaptive resolution change CTC enables the JVET to select coding tools with better performance vs. complexity trade-offs. This contribution further suggests to test both modes of resampling, i.e. picture-based and block-based, in a core experiment before selecting the solution that offers better performance vs. complexity trade-off.

Introduction

One of the key requirements of the VVC standard [1] is to offer video conferencing applications the ability to tolerate diversity of networks and devices, and to be able to rapidly adapt to varying network environments, including rapidly reducing encoded bit rate when network conditions deteriorate, and rapidly increasing video quality when network conditions improve. The VVC standard is expected to allow video conferencing applications to do so efficiently in a wide range of bit rates and video quality.

Adaptive resolution change (ARC) allows a stream to change spatial resolution between coded pictures within the same video sequence, without requiring a new IRAP picture and without requiring multi-layers as in scalable video codec. Instead, at a switch point, pictures change resolution and may be predicted either from pictures of the same resolution (if available) or from pictures of a different resolution by resampling the reference pictures. An illustration of adaptive resolution change is shown in Figure 1 where resolution of Ref 0 is same as the current picture. However, resolutions of Ref 1 and Ref 2 are different from that of the current picture. In order to generate motion compensated prediction signal for coding of the current picture, reference signal from both Ref 1 and Ref 2 require to be resampled to align with the resolution of the current picture.

Figure 1: A scenario where resolutions of reference pictures are different from that of the current picture

It is suggested that supporting ARC with high coding efficiency is very important for video conferencing applications. And in order to evaluate different ARC-related coding technology proposals, the first step should be defining common test conditions, such that important factors such as coding efficiency and encoding/decoding complexity can be measured. Establishing CTC for ARC allows future experiments to be conducted in a well defined environment, and allows the JVET committee to effectively make decisions. This contribution proposes common test conditions for ARC.

Proposed test conditions

We see the most important target applications of ARC to be video conferencing. Therefore, this contribution proposes to use the same low delay configurations (LDB and LDP) as in SDR CTC [2] for ARC. All low-delay sequences in SDR CTC are suggested to be tested, this includes classes B, C and E as mandatory classes, and classes D and F as optional classes. LD-B configuration is proposed to be mandatory, and LD-P configuration is proposed to be optional. All pictures shall be encoded for all sequences. And unless otherwise specified, all other test conditions should be kept the same as the CTC for SDR video.

In terms of resolution change, we proposed to switch the resolution of coded video multiple times, and include both upscaling and downscaling in the test conditions. It is proposed to use straightforward scaling ratios such as 2:1 or 1:2, and 1.5:1 or 1:1.5 in the ARC CTC. It is further proposed to use the same scaling ratio in both dimensions in the ARC CTC. It is suggested that these are more likely to be used in practical applications.

The picture resolution is reduced at a first time instance and is enlarged at a second time period. Figure 2 shows an example of resolution change where, at 3rd second the resolution is reduced to half and enlarged to full resolution at 6th second.

Figure 2: resolution switching

Test pipeline

Figure 3 shows the illustration of the test pipeline. For performance evaluation, we note that there are two ways for PSNR computation.

- The first PSNR (PSNR1) in Figure 3 is computed between the resampled source and reconstructed sequence.

- The second PSNR (PSNR2) is computed between the original resolution source and the resampled (upsample only in this case) version of the reconstructed sequence whose resolution is restored to the original resolution.

Figure 3: Test pipeline

In order to downsample the source before encoding, we proposed to use SHM down-sampling filter [3]. For PSNR2 computation, to upscale the decoded sequence, this contribution proposed to use SHM up-sampling filter[3]. The details of the filter coefficients are given below.

const int srcDownSampleFilter[8][13] =

{

{0,  0, 5, -6, -10, 37, 76, 37, -10,  -6, 5, 0,  0},

{0, -1, 5, -3, -12, 29, 75, 45,  -7,  -8, 5, 0,  0},

{0, -1, 4, -1, -13, 22, 73, 52,  -3, -10, 4, 1,  0},

{0, -1, 4,  1, -13, 14, 70, 59,   2, -12, 3, 2, -1},

{0, -1, 3,  2, -13,  8, 65, 65,   8, -13, 2, 3, -1},

{0, -1, 2,  3, -12,  2, 59, 70,  14, -13, 1, 4, -1},

{0,  0, 1,  4, -10, -3, 52, 73,  22, -13,-1, 4, -1},

{0,  0, 0,  5,  -8, -7, 45, 75,  29, -12,-3, 5, -1}

};

Const int upSampleFilter[16][8] =

{

{  0, 0,   0, 64,  0,   0,  0,  0 },

{  0, 1,  -3, 63,  4,  -2,  1,  0 },

{ -1, 2,  -5, 62,  8,  -3,  1,  0 },

{ -1, 3,  -8, 60, 13,  -4,  1,  0 },

{ -1, 4, -10, 58, 17,  -5,  1,  0 },

{ -1, 4, -11, 52, 26,  -8,  3, -1 },

{ -1, 3,  -9, 47, 31, -10,  4, -1 },

{ -1, 4, -11, 45, 34, -10,  4, -1 },

{ -1, 4, -11, 40, 40, -11,  4, -1 },

{ -1, 4, -10, 34, 45, -11,  4, -1 },

{ -1, 4, -10, 31, 47,  -9,  3,  1  },

{ -1, 3,  -8, 26, 52, -11,  4,  -1 },

{  0, 1,  -5, 17, 58, -10,  4,  -1 },

{  0, 1,  -4, 13, 60,  -8,   3,  -1  },

{  0, 1,  -3,  8, 62,  -5,   2,  -1  },

{  0, 1,  -2,  4, 63,  -3,   1,   0  }

};

Preliminary results on picture-based resampling

In terms of the resampling process, there have been some discussions at prior meeting and this meeting on whether picture-based resampling or block-based resampling should be applied. The following table attempts to summarize some pros and cons of the two methods:

|  | Picture-based | Block-based |
| --- | --- | --- |
| Pros | Resampling before decoding and no/little need for block-level changes | On-the-fly resampling, only resample when needed Can combine the filtering process of resampling and MC |
| Cons | Increase memory footprint When reference picture is downsampled, information of some samples is lost before current picture is decoded (this is the same for block-based if MC and resampling filtering is performed separately) | Block level changes are needed Some tools need to be modified or disabled Increase the block area to be fetched from the reference picture when downsampling the reference Interpolation filter phases could change per sample position in the block when the filtering process of resampling and MC is combined For encoder, motion search could become more complicated if combined MC/resampling filtering is used |

Some performance data had been provided in other contributions but no systematic comparison of the two methods in terms of their complexity and performance had been conducted under any previously agreed-upon CTCs. Therefore, it is asserted that it might be too early to select a specific method, and instead it is suggested to establish the ARC CTC and conduct more experiments before educated decisions can be made.

In such spirit this contribution attempted to collect some preliminary results of picture-based resampling. In order to do this, the VTM-5.0 is modified to add basic ARC support:

- Encoder config file parameters are added to specify when and how resolution changes (start from which frame, end at which frame, and the scaling ratios in horizontal/vertical direction);

- At the beginning of coding a picture, the reference pictures are examined, resampling is performed if the resolution of a reference picture is different from that of the current picture. For resampling filters, the software used motion compensated interpolation filters for upsampling and the SHM downsampling filters for downsampling;

- Source downsampling uses SHM downsampling filter;

- Resolution signaling is the same as the JVET-N0279 software

We evaluate the picture-based resampling method by examining temporal motion vector prediction (TMVP) more closely. It is asserted that TMVP affects three coding tools in ARC as follows. When the resolution of a reference picture is different from that of current picture, the motion vectors from that reference picture need to be scaled before TMVP and subblock-based temporal motion vector prediction (SbTMVP) can be applied and used in merge mode and AMVP mode. In addition, TMVP affects affine because it is used as one of the candidates in constructed affine merge mode. To evaluate the performance related to TMVP, we perform three tests:

Test 1: The collocated reference picture is set to the first reference picture in L0. When the resolution of first reference picture in L0 is not the same as that of current picture, TMVP is disabled.

Test 2: The collocated reference picture is set to the first reference picture in L0 whose resolution is the same as that of current picture. If all reference pictures have different resolution from the current one, TMVP is disabled.

Test 3: motion field of a reference picture with different resolution is resampled in the same way as in SHVC. TMVP is performed without any further changes to VVC draft 5.

These three tests are evaluated using low delay B condition with downsampling the resolution to half at 3rd second and upsampling the resolution to original resolution at 6th second (e.g. for a sequence with 60 fps, the resolution is downsampling at POC 180 and is upsampling at POC 360). It is noted that in this version only first 400 frames are coded if the sequence has more than 400 frames. The anchor is VTM-5.0 with TMVP, SbTMVP and affine mode turned off. These three tools are turned on in the tests. In Table 1 - 3, PSNR1 computation is used. On the other hands, PSNR2 computation is used in Table 4 - 6.

Table 1. Simulation results of Test 1 with PSNR1 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.81% | -3.64% | -3.82% | 125% | 106% |
| Class C | -3.03% | -2.22% | -2.68% | 116% | 103% |
| Class E | -4.31% | -3.01% | -2.72% | 129% | 102% |
| Overall | -4.09% | -3.01% | -3.16% | 123% | 104% |
| Class D | -3.92% | -2.98% | -2.41% | 113% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2. Simulation results of Test 2 with PSNR1 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.81% | -3.64% | -3.82% | 125% | 106% |
| Class C | -3.03% | -2.22% | -2.68% | 116% | 103% |
| Class E | -4.31% | -3.01% | -2.72% | 129% | 102% |
| Overall | -4.09% | -3.01% | -3.16% | 123% | 104% |
| Class D | -3.92% | -2.98% | -2.41% | 113% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3. Simulation results of Test 3 with PSNR1 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.81% | -3.82% | -3.61% | 124% | 106% |
| Class C | -3.03% | -2.27% | -2.65% | 115% | 103% |
| Class E | -4.40% | -3.03% | -2.81% | 128% | 103% |
| Overall | -4.11% | -3.11% | -3.09% | 122% | 104% |
| Class D | -3.91% | -2.48% | -2.74% | 111% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4. Simulation results of Test 1 with PSNR2 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.98% | -3.77% | -3.92% | 125% | 106% |
| Class C | -3.21% | -2.28% | -2.78% | 116% | 102% |
| Class E | -4.55% | -3.00% | -2.77% | 130% | 103% |
| Overall | -4.28% | -3.08% | -3.25% | 123% | 104% |
| Class D | -4.13% | -3.03% | -2.40% | 113% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5. Simulation results of Test 2 with PSNR2 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.98% | -3.77% | -3.92% | 125% | 106% |
| Class C | -3.21% | -2.28% | -2.78% | 116% | 102% |
| Class E | -4.55% | -3.00% | -2.77% | 130% | 103% |
| Overall | -4.28% | -3.08% | -3.25% | 123% | 104% |
| Class D | -4.13% | -3.03% | -2.40% | 113% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6. Simulation results of Test 3 with PSNR2 computation

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.98% | -3.93% | -3.71% | 125% | 106% |
| Class C | -3.21% | -2.32% | -2.76% | 116% | 103% |
| Class E | -4.54% | -3.02% | -2.83% | 130% | 103% |
| Overall | -4.28% | -3.16% | -3.18% | 123% | 104% |
| Class D | -4.13% | -2.61% | -2.74% | 113% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Discussion

In this contribution, we proposed a few considerations for establishing ARC CTC. It is asserted that establishing the ARC CTC first is crucial for the JVET to select coding tools with better performance vs. complexity trade-offs. The following items are suggested to be further discussed at the meeting:

- Selection of resampling ratios. A further consideration is if we should use the same resampling ratios for all classes of sequences, or should adjust the resampling ratios according to the input video resolution;

- Selection of resampling time instances. For example, the input 10-second video may be partitioned into 3 equal chunks (3.3 seconds per chunk), and downsampled at the start of the second chunk and upsampled at the start of the third chunk;

- Selection of PSNR computation method(s). Options include using just one of the two PSNR computation methods, or using both of them.

This contribution further suggests to test both modes of resampling, i.e. picture-based and block-based, in a core experiment before selecting the solution that offers better performance vs. complexity tradeoff.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

“Requirements for a Future Video Coding Standard v5”, ISO/IEC JTC1/SC29/WG11 N17074, July 2017.

F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakesh, MA, Jan, 2019.

SHM reference software, https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/SHM-12.4/

P. Chen, T. Hellman, B. Heng, W. Wan, M. Zhou, “AHG 19: Adaptive Resolution Change”, JVET-N0279, Geneva, CH, March 2019.