| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0643 |
| --- | --- |

| Title: | Modification on Delta QP for Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Geert Van Der Auwera, Muhammed Coban, Alican Nalci Marta Karczewicz | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com geertv@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents a modification on delta QP which restricts the delta QP not to apply to the previous CUs in different VPDUs. This modification removes the tracing back operations across different VPDUs such as deblocking filtering which improves the efficiency of codec. The simulation results show no quality effect.

# Introduction

In the current draft of VVC standard and VTM5.0 reference software, the delta QP is signaled in the first CB of which CBF flag equals to 1, and is applied to all of the CBs in the QG. For example, Fig.1 shows a QG including a 128x128 CU split into 4 64x64 TUs. The CBF flags of TU0, TU1 and TU2 equal to 0, the CBF flag of TU3 equals to 1. In this case, the delta QP of this QG is signaled in TU3. The first 3 CBs can’t get QP value until decode the delta QP in the last CB. Since deblocking filtering needs the QP values to calculate the deblocking parameters, TU0, TU1 and TU2 can’t perform deblocking until decode TU3. In the VVC, the size of VPDU is 64x64, tracing back to different VPDUs is even less efficient for hardware.

Figure 1: a QG including a 128x128 CU, split into 4 TUs

# Technical description

In this method, a restriction on delta QP is added. In this restriction, delta QP can only be applied to the CB within the current VPDU and the following CBs, it can’t be applied to the previous CBs in different VPDUs. The previous CBs which can’t use delta QP use qPy_pred instead of CU QP to calculate deblocking parameters.

The syntax modification on VVC draft is as below:

cu_qp_delta_abs specifies the absolute value of the difference CuQpDeltaVal between the quantization parameter of the current coding unit and its prediction.

specifies the absolute value of the difference CuQpDeltaVal between the quantization parameter of the current coding unit with luma coding block width smaller or equal to 64 and height smaller or equal to 64 and its prediction, or the current transform unit with luma transform block width equal to 64 and height equal to 64 and its prediction and the following transform units in decoding order within the current coding unit.

# Simulation results

## Test 1

The simulation was based on VTM5.0 software. The simulations were performed following the JVET common test conditions (CTC). Set MaxCuDQPSubdiv=2 and MaxDeltaQp=1 in both anchor and test. Results are shown in the following tables. No BD-rate difference was observed in the summary table. 0.01% gain for V component of class A “CampfireParty2” and class F “ChinaSpeed” can be found in in RA case in the attached spreadsheet

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 101% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 103% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test 2

The simulation was based on VTM5.0 software. The simulations were performed following the JVET common test conditions (CTC). In following tests, the delta QP syntax was enabled (cu_qp_enabled_flag = 1), and MaxCuDQPSubdiv was set equal to 10 in both anchor and test. Results are shown in the following tables:

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class SCC 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution presents a modification on delta QP which restricts the delta QP not to apply to the previous CUs in different VPDUs. This modification removes the tracing back operations across different VPDUs such as deblocking filtering which improves the efficiency of codec. The simulation results show no quality effect.

# References

- B. Bross, J. Chen, S. Liu, JVET-N1001, “Versatile Video Coding (Draft 5)”, Geneva, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).