| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–17 July 2019 | Document: JVET-N0645-v2 |
| --- | --- |

| Title: | Non-CE4/8: Blending-off Switch for TPM Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Yan Zhang, Kevin Reuze, Yao-Jen Chang, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal disables the blending operation of Triangle Prediction Merge (TPM) mode for screen content sequences. With it, the weighting values in the blending area are all clipped to be either 0/8 or 8/8, depending on whether they are closer to 0/8 or 8/8. Thus, none of the final prediction samples are generated by weighted-averaging two different ones which would blur out certain video contents with extremely sharp signals (e.g., screen contents). It is noted that a slice-level flag is introduced to indicate whether the blending operation of TPM is enabled or not, and a sequence-level flag is introduced to indicate whether the mentioned slice-level flag is present is the bitstream. Experiments were conducted on top of the VTM-5.0 with Random Access (RA) and Low Delay B (LDB) testing conditions for both CTC and CE8 testing sequences. Performance results show that the average BD-rate impact on CTC sequences is 0.00% and 0.01% respectively for RA and LDB. For CE8 testing sequences, the average BD-rate gain is 0.20% and 0.34% respectively for RA and LDB. It is also reported that Class TGM1080p444 achieves the highest coding among all, with 0.60% and 0.91% BD-rate gain in RA and LDB, respectively. The average runtime change observed respectively from encoder and decoder is around ±1%.

# Introduction

Triangle Partition Merge (TPM) mode is a merge mode that splits a coding unit diagonally (i.e., from upper-left to bottom-right corner) or anti-diagonally (i.e., from upper-right to bottom-left corner) into two equal-sized halves, each with a triangle shape. Each triangle partition is assigned with a uni-prediction MV derived from merge candidate list. During the motion compensation stage, the two triangle partitions are both unit-predicted except for the samples along the boundaries between the two partitions. A weighted-averaging operation (as shown in the below figure) is applied to blend those samples to form the resulting signals of motion compensation.

However, the performance-wise benefit of this blending operation may be reduced if the coded video sequences contain extremely sharp signals (e.g., screen contents). Prediction samples along the boundary of two triangle partitions inside a CU would be blurred, resulting in less accurate prediction for those sharp signals.

# Technical Description

Accordingly, this contribution introduces a slice-level flag (triangle_blending_off_flag) to indicate whether the blending operation of TPM has to be disabled. If yes, the weighting values (i.e., w) assigned to the samples sitting on the blending area are all clipped to be either 0/8 or 8/8, depending on whether they are closer to 0/8 or 8/8.

- If w < 4/8, the value of w is clipped to 0/8;

- Otherwise, if w > 4/8, the value of w is clipped to 8/8;

- Otherwise, if w = 4/8, the value of w is clipped as follows:

- If the CU is split diagonally, w is clipped to 8/8;

- Otherwise, w is clipped to 0/8.

The below figure at the left-hand side visualizes the map of sample-by-sample weighting values when blending operation is enabled. The other figure at the right-hand side illustrates the map of sample-by-sample weighting values when blending operation is disabled.

| (a) VTM | (b) Proposed |
| --- | --- |

Figure 1. (a) Weighting map when blending is enabled; (b) weighting map when blending is disabled.

In addition, a sps flag (sps_tpm_blending_ctrl_flag) is introduced to indicate whether the mentioned slice-level flag (triangle_blending_off_flag) is present in the bitstream. If no, this slice-level flag is inferred to be zero.

# Experimental Results

The experiments were conducted on top of the VTM-5.0 software with the common test conditions [1] and CE8 testing conditions [2] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-5.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

At the encoder, the introduced sps flag (sps_tpm_blending_ctrl_flag) is always set enabled for each test. The introduced slice-level flag (triangle_blending_off_flag) is configured by using the same hash-ratio based function as MMVD uses to swap between fractional and integer distance offset tables and the threshold value is set to be 40 for triangle_blending_off_flag.

- Performance results of CE8 sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.60% | -0.49% | -0.50% | 101% | 100% | -0.91% | -0.77% | -0.81% | 101% | 99% |
| TGM720 | -0.07% | -0.01% | -0.02% | 101% | 100% | -0.15% | -0.13% | -0.31% | 99% | 97% |
| Animation | 0.01% | 0.01% | 0.01% | 101% | 101% | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Mixed | -0.13% | -0.14% | -0.14% | 101% | 101% | -0.29% | -0.28% | -0.27% | 102% | 101% |
| Camera | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | -0.20% | -0.16% | -0.16% | 101% | 100% | -0.34% | -0.29% | -0.35% | 100% | 99% |

- Performance results of CTC sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | x | x | x | x | x |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | x | x | x | x | x |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.01% | 0.01% | 0.01% | 103% | 101% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | x | x | x | x | x | 0.02% | 0.02% | 0.02% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.01% | 0.01% | 0.01% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.03% | 103% | 101% | 0.01% | 0.01% | 0.01% | 104% | 101% |
| Class F | -0.01% | 0.08% | 0.08% | 100% | 101% | 0.01% | -0.04% | -0.45% | 100% | 98% |
| Class SCC | -0.54% | -0.39% | -0.42% | 100% | 100% | -0.59% | -0.49% | -0.41% | 99% | 101% |

# Proposed Draft Text

The proposed texts are drafted based on JVET-N1001 (v9) [3]. The following section highlights the change in VVC working draft of each respective test.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag ) { |  |
| sps_tpm_blending_ctrl_flag | u(1) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| … |  |

sps_tpm_blending_ctrl_flag equal to 1 specifies that triangle partition merge mode disables the blending operation applied to the boundary between two triangle partitions inside a coding unit. sps_tpm_blending_ctrl_flag equal to 0 specifies that the blending operation is always enabled. When not present, the value of sps_tpm_blending_ctrl_flag is inferred to be 0.

#### 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ) { |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if( sps_tpm_blending_ctrl_flag ) |  |
| triangle_blending_off_flag | u(1) |
| } |  |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| slice_qp_delta | se(v) |
| … |  |

triangle_blending_off_flag equal to 1 specifies that triangle partition merge mode disables the blending operation applied to the boundary between two triangle partitions inside each coding unit of the current slice. triangle_blending_off_flag equal to 0 specifies that the blending operation is always enabled in the current slice. When not present, triangle_blending_off_flag is inferred to be 0.

#### 8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-821)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-822)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-823)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-824)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-825)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-826)

–	If triangle_blending_off_flag is equal to 1, the following applies:

wValue = ( wValue != 4 ) ? ( wValue < 4 ? 0 : 8 ) : ( triangleDir == 1 ? 0 : 8 )	(8-827)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-827)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

# References

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1010, Mar. 2019.

- X. Xu, Y.-H. Chao and et. al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, Mar. 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1001, Mar. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).