| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0651-v2 |
| --- | --- |

| Title: | Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, aramasub, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to use the horizontal prediction instead of DC for a DM chroma block when its corresponding luma block is encoded using IBC or PCM mode. In another test, the horizontal and Planar prediction modes are used for a DM chroma block when its corresponding luma block is IBS and PCM blocks, respectively. Since PCM is disabled in VTM5.0, the performance of the two tests is identical and they achieve minor average (Y, U, V) BD rate gain for class F and Class SCC as follows:

AI: (-0.02%, 0.00%, 0.06%) and (-0.01%, 0.01%, 0.00%)

RA: (-0.01%, -0.07%, 0.15%) and (-0.02%, -0.01%, -0.09%)

LDB: (-0.03%, -0.08%, -0.06%) and (0.04%, 0.03%, -0.27%).

# Introduction

Current VTM5.0 supports several 8 intra prediction modes for chroma blocks including Planar, DC, horizontal, vertical, 3 CCLM modes, and DM mode. When a chroma block is encoded using the DM mode, the actual intra prediction mode applied to this block is derived from the intra prediction mode of the corresponding luma block. When the corresponding luma block is encoded using IBC or PCM mode, the DC mode is applied for the chroma block.

It should be noted that, the vertical and horizontal modes are the simplest modes among the candidates of the intra chroma since it simply copies the left column or above line. On the other hand, the other modes need to access both the left column and above line. For the DC mode, the DC value needs to be obtained for prediction while for Planar mode, an interpolation needs to be performed to get the prediction values.

# Proposed Method

In this proposal, the horizontal prediction is used for a DM chroma block when its corresponding luma block is encoded using IBC or PCM mode. This will make the DM simple by just copying the left column for prediction.

In another test, In another test, the horizontal and Planar prediction modes are used for a DM chroma block when its corresponding luma block is IBS and PCM blocks, respectively.

# Experimental Results

The proposed simplifications have been implemented on top of VTM5.0. The experimental results under the AI, RA and LDB with RDPCM, IBC and HASH ME enabled for Class F and Class SCC are summarized in Table 1. Since this simplification works only when IBC enabled, the performance of the other class with IBC off is not affected.

It can be seen from these tables that, the experimental results demonstrated that the proposed approaches achieve minor average (Y, U, V) BD rate gain for class F and Class SCC as follows:

AI: (-0.02%, 0.00%, 0.06%) and (-0.01%, 0.01%, 0.00%)

RA: (-0.01%, -0.07%, 0.15%) and (-0.02%, -0.01%, -0.09%)

LDB: (-0.03%, -0.08%, -0.06%) and (0.04%, 0.03%, -0.27%).

Table 1. Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -0.02% | 0.00% | 0.06% | 101% | 102% |
| Class TGM | -0.01% | 0.01% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -0.01% | -0.07% | 0.15% | 102% | 101% |
| Class TGM | -0.02% | -0.01% | -0.09% | 102% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.04% | 0.03% | -0.27% | 100% | 98% |
| Class TGM | -0.03% | -0.08% | -0.06% | 99% | 95% |

# Conclusions

This contribution proposes to use the horizontal prediction instead of DC for a DM chroma block when its corresponding luma block is encoded using IBC or PCM mode. It is recommended to adopt this simplification into the next version of VVC.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).