| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0652-v3 |
| --- | --- |

| Title: | CE4-related: Line buffer size reduction method for switchable interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Sergey Ikonin, Elena Alshina | Tel: Email: | +79250089652 solovyev.timofey@huawei.com sergey.ikonin@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes modification of switchable interpolation filter index propagation process for method proposed in CE4-1. In order to reduce the line buffer size, borrowing of interpolation filter index is restricted to be within the current CTU line. The simulation results on top of CE4-1.2 reportedly show 0.02% and 0.03% luma BD-rate loss for RA and LDB configurations respectively. The simulations results on top of CE4-1.3 reportedly show 0.02% luma BD-rate loss for RA configuration and 0.02% luma BD-rate gain for LDB configuration.

# Introduction

Switchable interpolation filter technique proposed in N0309 is studied in CE4-1. The switching of the half-pel luma interpolation filter is performed depending on the motion vector accuracy. In addition to the existing IMV modes, a new half-pel accuracy IMV mode is introduced. When half-pel motion vector accuracy is selected, one (CE4-1.2) or two (CE4-1.3) alternative half-pel luma interpolation filters can be used depending on signalled index. In case of merge mode index of interpolation filter applied for the half-pel position is inherited from the corresponding merge candidate.

# Problem Statement

In current design interpolation filter indices of coding units located on CTU bottom boundary should be stored in the line buffer, because borrowing this indices across the top CTU boundary is allowed for merge mode. If picture width is equal to w, then (w/4) or (w/4)*2 bits should be additionally stored in the line buffer for tests CE4-1.2 and CE4-1.3 respectively. For example, for the 4K resolution ( 3840 / 4 ) = 960 or ( 3840 / 4 ) * 2 = 1920 bits should be stored in line buffer for tests CE4-1.2 and CE4-1.3 respectively.

# Proposed Solutions

It’s proposed to disable borrowing interpolation filter indices across top CTU boundary in merge mode:

  - prior to merge list construction determine whether current block border on top CTU boundary

  - for blocks bordered on top CTU boundary during the derivation of spatial candidates B0, B1 and B2 set interpolation filter index equal to the default value – value corresponding to regular half-pel interpolation filter.

# Simulation Results

The proposed modification is evaluated for random-access (RA) and Low-delay-B (LDB) configurations using VTM-5.0 software [1] according to the common test conditions as described in JVET-N1010 [2]. The detailed experimental results on top of CE4-1.2 are provided in Table 1 and Table 2. The detailed experimental results on top of CE4-1.3 are provided Table 3 in and Table 4.

During crosschecking of CE4-1, minor software problem was noticed. The detailed experimental results on top of CE4-1.2 with the bugfix are provided in Table 5 and Table 6.

Table 1. Coding performance of proposed method over CE4-1.2 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | -0.03% | #VALUE! | #VALUE! |
| Class A2 | 0.03% | 0.06% | -0.03% | #VALUE! | #VALUE! |
| Class B | 0.01% | 0.06% | 0.08% | 100% | 100% |
| Class C | 0.03% | -0.01% | 0.02% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.02% | #VALUE! | #VALUE! |
| Class D | 0.01% | 0.02% | -0.02% | 100% | 99% |
| Class F | 0.00% | 0.02% | 0.04% | 100% | 100% |

Table 2. Coding performance of proposed method over CE4-1.2 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | 0.34% | #VALUE! | #VALUE! |
| Class C | 0.06% | 0.16% | 0.17% | 99% | 97% |
| Class E | 0.00% | 0.16% | 0.35% | 98% | 95% |
| Overall | 0.03% | -0.01% | 0.29% | #VALUE! | #VALUE! |
| Class D | 0.03% | 0.51% | -0.24% | 99% | 97% |
| Class F | 0.02% | 0.71% | 0.26% | #VALUE! | #VALUE! |

Table 3. Coding performance of proposed method over CE4-1.3 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.18% | 0.03% | #VALUE! | #VALUE! |
| Class A2 | 0.01% | -0.13% | -0.01% | #VALUE! | #VALUE! |
| Class B | 0.01% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.02% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.02% | -0.01% | #VALUE! | #VALUE! |
| Class D | 0.04% | -0.04% | 0.01% | 100% | 100% |
| Class F | 0.02% | -0.04% | -0.04% | 100% | 99% |

Table 4. Coding performance of proposed method over CE4-1.3 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.08% | #VALUE! | #VALUE! |
| Class C | 0.02% | 0.11% | -0.06% | 100% | 99% |
| Class E | -0.10% | 0.01% | 0.37% | 99% | 99% |
| Overall | -0.02% | 0.04% | 0.04% | #VALUE! | #VALUE! |
| Class D | 0.02% | -0.16% | -0.33% | 100% | 98% |
| Class F | 0.08% | -0.29% | 0.27% | #VALUE! | #VALUE! |

Table 5. Coding performance of proposed method over CE4-1.2 with the bugfix (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.2 fixed |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0,05% | 0,16% | 0,15% | 104% | 125% |
| Class C | 0,03% | 0,03% | 0,04% | 107% | 124% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,00% | 0,03% | -0,06% | 107% | 126% |
| Class F | 0,01% | 0,01% | -0,01% | 105% | 139% |

Table 6. Coding performance of proposed method over CE4-1.2 with the bugfix (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-1.2 fixed |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class C |  | 0,03% |  | -0,25% |  | -0,10% |  | #VALUE! |  | #VALUE! |  |
| Class E |  | 0,01% |  | -0,45% |  | 0,12% |  | #DIV/0! |  | #DIV/0! |  |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class D |  | 0,06% |  | -0,07% |  | -0,04% |  | 104% |  | 118% |  |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).