| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0655 |
| --- | --- |

| Title: | Non-CE3: On chroma DM derivation table for 4:2:2 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports a defect on chroma intra angle mapping table for 4:2:2 chroma format implemented in the current VTM and proposes a modification of the table values. In HEVC, due to the limitation of the prediction angle, luma intra prediction modes ranging from 2 to 5 are mapped to 2 for chroma intra mode derivation in 4:2:2 chroma format. VTM adopts the similar mapping table ported from HM and extended from 35 to 67 entries, by which luma intra prediction modes ranging from 2 to 8 are also mapped to 2 even though wide angle intra prediction (WAIP) was adopted in VVC. The proposal replaces some values of the entries of the mapping table to convert prediction angle more precisely for chroma blocks. The experimental results show slight but consistent coding gain for all intra configuration without encoding and decoding time change.

Problem Statement

In the current VTM, 4:2:2 and 4:4:4 chroma formats are supported as well as 4:2:0. Chroma derived mode (DM) derivation table for 4:2:2 chroma format was ported from that defined in HM and extended from 35 to 67 entries to align with the extension of intra prediction modes as follows:

const uint8_t g_chroma422IntraAngleMappingTable[NUM_INTRA_MODE] = {

0, 1, 2, 2, 2, 2, 2, 2, 2, 3, 4, 6, 8, 10, 12, 13, 14, 16, 18, 20, 22, 23, 24, 26, 28, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 44, 44, 45, 46, 46, 46, 47, 48, 48, 48, 49, 50, 51, 52, 52, 52, 53, 54, 54, 54, 55, 56, 56, 56, 57, 58, 59, 60, DM_CHROMA_IDX

};

Since HEVC specification does not support prediction angle below -135 degree and above 45 degree, luma intra prediction modes ranging from 2 to 5 are mapped to 2. In spite of the fact that VVC newly introduces wide angle intra prediction (WAIP) which allows to use wider angle prediction for rectangular blocks, luma intra prediction modes ranging from 2 to 8 are always mapped to 2 for the corresponding chroma block as shown in Figure 1. This defect may cause potential coding loss and this should be fixed.

Figure 1. Chroma intra angle mapping in VTM

Proposed modification

Simple solution for this defect is to replace the mapping table as follows:

const uint8_t g_chroma422IntraAngleMappingTable[NUM_INTRA_MODE] = {

0, 1, 61, 62, 63, 64, 65, 66, 2, 3, 4, 6, 8, 10, 12, 13, 14, 16, 18, 20, 22, 23, 24, 26, 28, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 44, 44, 45, 46, 46, 46, 47, 48, 48, 48, 49, 50, 51, 52, 52, 52, 53, 54, 54, 54, 55, 56, 56, 56, 57, 58, 59, 60, DM_CHROMA_IDX

};

Experimental results

The proposed modification is implemented on top of VTM-5.0 and coding experiments using 4:2:2 test sequences are conducted. Table 1 and Table 2 show BD-rate performance of all intra configuration. It is remarked that slight but consistent BD-rate gain was observed for most of the 4:2:2 sequences without encoding and decoding time increase.

Table 1 BD-rate performance of the proposed modification in AI configuration over VTM-5.0

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| EBUGraphics | -0.04% | -0.07% | -0.01% | 101% | 101% |
| EBUHorse | -0.01% | -0.01% | -0.02% | 101% | 99% |
| EBUKidsSoccer | 0.00% | -0.02% | -0.08% | 100% | 100% |
| EBUWaterRocksClose | -0.01% | -0.03% | -0.03% | 100% | 100% |
| Kimono | -0.02% | -0.04% | -0.10% | 101% | 100% |
| Seeking | -0.01% | 0.02% | -0.01% | 100% | 100% |
| Traffic | -0.09% | -0.08% | -0.03% | 100% | 100% |
| All | -0.02% | -0.03% | -0.04% | 100% | 100% |

Proposed specification change

- 8.4.4 Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is derived using IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] and sizeId set equal to MipSizeId[ xCb ][ yCb ] as specified in Table 8-4.

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The variavble modeIdx is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6.

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived as follows:

- If ChromaArrayType is equal to 2, IntraPredModeC[ xCb ][ yCb ] is set using modeIdx as specified in Table 8-X.

- Otherwise, IntraPredModeC[ xCb ][ yCb ] is set equal to modeIdx.

Table 8-5 – Specification of modeIdx depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-6 – Specification of modeIdx depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Table 8-X Specification of IntraPredModeC when ChromaArrayType is equal to 2

| modeIdx | X <= 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | X |  | 61 | 62 | 63 | 64 | 65 | 66 | 2 | 3 | 4 | 6 | 8 | 10 | 12 | 13 | 14 |
| modeIdx | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| IntraPredModeC | 16 | 18 | 20 | 22 | 23 | 24 | 26 | 28 | 30 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| modeIdx | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| IntraPredModeC | 40 | 41 | 42 | 43 | 44 | 44 | 44 | 45 | 46 | 46 | 46 | 47 | 48 | 48 | 48 | 49 | 50 |
| modeIdx | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 |  |
| IntraPredModeC | 51 | 52 | 52 | 52 | 53 | 54 | 54 | 54 | 55 | 55 | 56 | 56 | 57 | 58 | 59 | 60 |  |

Conclusion

This contribution reports a defect on chroma intra angle mapping table for 4:2:2 chroma format implemented in the current VTM and proposes a modification of the table values. This proposal replaces some values of the entries of the mapping table to convert prediction angle more precisely for chroma blocks. The experimental results show slight but consistent coding gain for all intra configuration without encoding and decoding time change. It is recommended that the proposed modification is adopted to the next version of draft specification.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).